3GPP TSG-SA2 Meeting #160	S2-2312202

# November 13 – 17, 2023, Chicago, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0391 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 5G ProSe UE-to-UE Relay Discovery with Index |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In step 3 of procedure for 5G ProSe UE-to-UE Relay Discovery with Model B defined in clause 6.3.2.4.3, the discoveree 5G ProSe End UE responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message which contains the User Info ID of the discoverer 5G ProSe End UE and the User Info ID of discoveree 5G ProSe End UE. The problem is that in step 4, how does the 5G ProSe UE-to-UE Relay send the 5G ProSe UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay and 2) the 5G ProSe UE-to-UE Relay may serve more than one discoverer 5G ProSe End UE simultaneously? In this paper, the 5G ProSe UE-to-UE Relay sends the 5G ProSe UE-to-UE Relay Discovery Solicitation message to the discoveree 5G ProSe End UE with the Index associated with the RSC and the Source Layer-2 ID of the discoverer 5G ProSe End UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Including the Index associated with the RSC and the Source Layer-2 ID of the discoverer 5G ProSe End UE in the 5G ProSe UE-to-UE Relay Discovery Solicitation message toward the discoveree 5G ProSe End UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe UE-to-UE Relay Discovery with Model B doesn’t work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.3.2.4.3	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

Depicted in Figure 6.3.2.4.3-1 is the procedure for 5G ProSe UE-to-UE Relay Discovery with Model B.

Figure 6.3.2.4.3-1: 5G ProSe UE-to-UE Relay Discovery with Model B

1.	The discoverer 5G ProSe End UE (UE-1) sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of itself, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

2.	A 5G ProSe UE-to-UE Relay that matches the RSC sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of the discoverer 5G ProSe End UE (UE-1), User Info ID of UE-to-UE Relay, RSC, User Info ID of the discoveree 5G ProSe End UE (UE-2), and the Index associated with the RSC and the Source Layer-2 ID of the discoverer 5G ProSe End UE. and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe End UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

3.	The discoveree 5G ProSe End UE (UE-2) that matches the RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. It also includes the Index in the received 5G ProSe UE-to-UE Relay Discovery Solicitation message. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays with the same RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2), it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

4.	The 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. It can identify the received 5G ProSe UE-to-UE Relay Discovery Response message based on the Index for deciding which discoverer 5G ProSe End UE to send. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

* * * End of Changes * * *