SA WG2 Meeting #160	S2-2312226

Chicago, USA, November 13 – 17, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5094 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of URSP rule provsion in EPS according to LS from CT1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the LS (C1-237892) and CRs (C1-237770, C1-237771, C1-237772, C1-237775 and C1-237913) from CT1, some of the statement in URSP rule provision to EPS should be corrected in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of URSP rule provision in EPS according to the conclusion from CT1 CRs. The changes are summarized below: Firstly, change the “Indication of URSP Provisioning Support in EPS” to “URSP provisioning in EPS support indicator” Secondly, distinguish between cases where the PDN connection establishment request is in attach and cases where it is not in attach At last, according to the CT1 conclusion, the following consensus exists: - the UE and network can interact the capability of URSP provisioning in EPS support indicator each other. During the attach request or first PDN connectivity request, the UE includes the URSP provisioning in EPS support indicator in PCO. And if the MME supports ePCO and the SMF+PGW-C supports URSP provisioning in EPS, it the SMF+PGW-C provides the URSP provisioning in EPS support indicatorIndication of URSP Provisioning Support in EPS ePCO to UE in the PDN Connectivity Accept message. - And if the interaction is successfully achieved, it means the MME shall support ePCO. And in current specification, the SMF+PGW-C shall ePCO feature. - Add the situation that network can update the URSP rule to UE in some situation. - After interaction of capability, only if needed, the UE can include the UE policy container to network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and stage 3 statements are not consistant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.17.8	URSP Provisioning in EPS

When the UE first registers in 5GS, the UE includes the URSP provisioning in EPS support indicator in the UE Policy Container carried in Registration Request. Only when receiving this indication in the UE Policy Container, the PCF can provision the URSP to UE in EPS.

During Initial Attach procedure and the PDN connectivity request message is sent together with initial attach request in EPS, the UE provides the URSP provisioning in EPS support indicator in PCO or ePCO in the PDN connectivity request to SMF+PGW-C. If the PDN connectivity request message was not sent together with an Attach request message, the UE requested PDN connectivity procedure is performed for the first PDN connection and the URSP provisioning in EPS support indicator is provided in PCO or ePCO in the first PDN connectivity request message. If the MME supports ePCO and the SMF+PGW-C supports URSP provisioning in EPS, the SMF+PGW-C provides the URSP provisioning in EPS support indicator in ePCO to UE in the PDN Connectivity Accept message.

NOTE 1:	The UE provides the URSP provisioning in EPS support indicator in ePCO only when the network has indicated support of the Extended protocol configuration options IE as defined in clause 6.6.1.1 of TS 24.301 [13].

When the UE receives the URSP provisioning in EPS support indicator in ePCO from the network, the UE can initiate the UE requested bearer resource modification procedure and includes the UE Policy Container in ePCO to request UE policy when the UE requires the URSP rule from network, and the UE Policy Container in ePCO will be further forwarded by MME to SMF+PGW-C. When the UE Policy Container in ePCO is received by SMF+PGW-C, it forwards transparently the UE Policy Container to PCF for the PDU Session, then the PCF for the PDU Session establishes the UE Policy Association with PCF for the UE. The PCF for the UE generates the corresponding URSP rules in a similar way as it is done in 5GS and sends the URSP rules to UE in the UE Policy Container as described in clause 4.11.0a.5 of TS 23.502 [3].

After successful exchange of the URSP provisioning in EPS support indicators, the SMF+PGW-C can initiate the EPS bearer context modification procedure or the dedicated EPS bearer context activation procedure to transport UE policy container (including URSP rule) to the UE if the PCF for UE needs to send or update the URSP rule to UE in EPS.

If the UE does not receive the URSP provisioning in EPS support indicator in ePCO from EPC, then the UE is not allowed to initiate the UE requested bearer resource modification procedure to send the UE Policy Container to EPC.

The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode. When the UE is attached to EPS, the PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribes to the applicable PCRTs to SMF+PGW-C. The PCF for the PDU Session sends the UE Policy Container in the same PDN connection/PDU session in which the UE Policy Container was received.

During EPS to 5GS mobility with N26, the UE Policy Association is terminated by PCF for PDU Session when it receives the indication of RAT type change from the SMF+PGW-C.

During 5GS to EPS mobility with N26, the PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS mobility").

NOTE 2:	At 5GS to EPS mobility with N26, the guard timer in the AMF (as specified in clauses 4.11.1.2.1 and 4.11.1.3.2 of TS 23.502 [3]) ensures that the UE Policy Association remains until the PCF for the UE detects that a UE Policy Association establishment is received from a PCF for the PDU Session indicating 5GS to EPS mobility.

NOTE 3:	In the case that the UE is still registered and reachable over non-3GPP access, the PCF for UE can have two UE policy associations for one UE, and based on local configuration the PCF can decide to use one of the UE policy associations to update URSP rule to the UE. The UE can receive URSP Rules over any of these two accesses.

* * * End of Change * * *