3GPP TSG-SA WG2 Meeting #160	S2-2312248

# Chicago, USA, November 13 – 17, 2023								      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0394 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Convey Layer-2 ID of target Layer-2 End UE to source Layer-2 End UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 sent SA2 the LS on L2ID and User Info for L2 based U2U (S2-2311980/R2-2311566) to inform SA2 of the following RAN2 working assumption and ask SA2 if the working assumption is feasible from SA2 perspective. RAN2 further discussed how to link the User Info ID with the Local ID at the remote UE, based on two options that are both feasible from RAN2 perspective, and then RAN2 made a working assumption that the RRCReconfigurationSidelink message carries both the L2 ID and the Local ID of the peer Remote UE and assumes that the association between the User Info ID and the L2 ID of the peer Remote UE is done at the Remote UE ProSe layer. Therefore, RAN2 would ask SA2 if the above working assumption is feasible from SA2 perspective. RAN2 will appreciate timely response from SA2 before TSG-RAN#102. RAN2 intend to implement this working assumption if SA2 indicate it is feasible. As discussed in S2-2312246, in order to make RAN2 working assumption workable, it is propsed to enhance the Direct Communication Accept message over the first hop to include the Layer-2 ID of target Layer-2 End UE to the source Layer-2 End UE, so that the souce Layer-2 End UE keeps the association between User Info ID and Layer-2 ID of the target Layer-2 End UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Direct Communication Accept message over the first hop to include the Layer-2 ID of target Layer-2 End UE to the source Layer-2 End UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the association between User Info ID and Layer-2 ID of the target Layer-2 End UE makes RAN2 working assumption not feasible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.4.3.7.2	Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay as described in clause 6.7.2, the description in clause 6.4.3.7.1 applies with the following differences and clarifications:

-	The Direct Communication Accept message over the first hop PC5 reference point additionally includes Layer-2 ID of target 5G ProSe Layer-2 End UE which is obtained by 5G ProSe Layer-2 UE-to-UE Relay during the Security Procedure of the second hop PC5 reference point as described in step 6 of clause 6.7.1.1.

The message contents over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.1 to clause 6.4.3.5 are same for the end-to-end connection between peer 5G ProSe End UEs.

Editor's note:	Whether the LIU between peer Layer-2 End UEs has same message contents as direct PC5 LIU messages is FFS.

* * * * End of Changes * * * *