3GPP TSG-SA WG2 Meeting #160	S2-2312254

# Chicago, USA, November 13 – 17, 2023								      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0397 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NG-RAN Location reporting for multi-path communication via direct Uu path and via Layer-2 UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 sent SA2 the LS on handling of location information in multi-path operation (S2-2311965/R3-235761) to ask SA2 about the following question. For the Multi-path operation in SL Relay Enhancement WI, RAN3 discussed the aspect on whether the NG-RAN node reports both the cell ID of the direct path and the cell ID of the indirect path to the AMF, in case the direct path and the indirect path connect with different cells of same gNB. RAN3 observed that the NG-RAN node can also report to the AMF the cell identity of PSCell in the dual connectivity. Because it is the 5GC to use the User Location Information, RAN3 would like to request SA2 answer the following question: Question: Whether it is required for NG-RAN node to report both cell IDs of direct path and indirect path to AMF, in case the direct path and the indirect path connect with different cells of same gNB? For LI purpose, it is proposed to support NG-RAN reporting both cell IDs of direct path and indirect path to AMF, in case the direct path and the indirect path connect with different cells of same gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding requirement for Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path reporting when supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delivery of Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path would not be available in the CN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.9.3	Multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

Editor's note:	Alignment with RAN WGs is to be done.

When a 5G ProSe Layer-2 Remote UE is accessing the network via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay, the NG-RAN Location reporting procedures as specified in clause 4.10 of TS 23.502 [5] is applied with the following differences:

-	When multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay is activated, Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path is only reported if requested by the AMF.

-	For step 1, if the Reporting Type indicates to report whenever the UE changes cell and if Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path reporting is requested and multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay is in use, the RAN node shall also report to the AMF whenever the Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path changes.

-	For step 2, if Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path reporting is requested and multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay is activated, then the NG-RAN node shall also include the Cell ID information of 5G ProSe Layer-2 UE-to-Network Relay path.

* * * * End of Changes * * * *