3GPP TSG-WG SA2 Meeting #160	S2-2312265

Chicago, US, Nov 13 – 17, 2023

Title:	[DRAFT] Reply LS on Clarification related to Network Slicing enhancement

Response to:	LS (S2-2311932/C4-233776) on Clarification related to Network Slicing enhancement

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	[Huawei to be] SA2

To:	CT4

Cc:	CT1, CT3

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 5098, TS 23.502 CR 4576

1. Overall Description:

SA2 would like to thank CT4 for their LS on Clarification related to Network Slicing enhancement.

Specific to the CT4 questions in C4-233776, SA2 would like to provide the following feedback:

Clause 5.15.6 states:

If the serving AMF supports the Network Slice Replacement feature and is configured to use the NSSF for Network Slice Replacement triggering, the AMF subscribes with the NSSF of the VPLMN for notifications when an HPLMN S-NSSAI needs to be replaced with an Alternative S-NSSAI, in addition to notifications for the Serving PLMN S-NSSAIs. The NSSF of the VPLMN shall subscribe with the NSSF of the HPLMN for notifications when an HPLMN S-NSSAI needs to be replaced with an Alternative S-NSSAI.

Question 1: Does the above requirement apply only to Home Routed roaming use case? Or also to LBO roaming use case?

Answer: When the AMF subscribes notification of HPLMN S-NSSAI replacement, it needs to determine whether the HPLMN S-NSSAI replacement is needed or not. For one S-NSSAI if all associated DNN is only for LBO roaming, the AMF is not required to subscribe the notification of the HPLMN S-NSSAI replacement. The reasons are listed in the following:

- The PDU session associated with such DNN/S-NSSAI is LBO roaming in which the HPLMN S-NSSAI is not used for user plane establishment in the VPLMN.

- The V-SMF shall use the original HPLMN S-NSSAI which is provided by the UE to retrieve subscription data from UDM.

- SA2 has not defined the charging aspect for slice replacement, especially for HPLMN S-NSSAI replacement in LBO roaming case. CT4 can further check the charging aspects related to HPLMN S-NSSAI replacement, if necessary, with SA5.

Clause 5.15.19 states:

, the AMF provides the Alternative S-NSSAI for this S-NSSAI in the Allowed NSSAI and in the Configured NSSAI, if not included yet, and the mapping between S-NSSAI(s) to Alternative S-NSSAI(s) to the UE in UE Configuration Update message as follows:

-    for non-roaming UEs, the AMF provides the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE.

-    for roaming UEs when the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the AMF provides the mapping of the VPLMN S-NSSAI to the Alternative VPLMN S-NSSAI to the UE.

-    for roaming UEs when the HPLMN S-NSSAI has to be replaced by an Alternative HPLMN S-NSSAI, the AMF provides the mapping of the HPLMN S-NSSAI to the Alternative HPLMN S-NSSAI to the UE.

Question 2: In the above requirement, HPLMN S-NSSAI replaced with HPLMN Alternative S-NSSAI is applicable only in case of Home routed roaming use case? Or also to LBO roaming use case?

Answer: The above notification from AMF to SMF only applies to Home Routed roaming case.

In the current TS 23.502,

Clause 4.3.2.2.1 states:

For roaming scenario in local breakout (LBO), the AMF also sends the corresponding S-NSSAI of the HPLMN from the Mapping Of Allowed NSSAI or Mapping Of Partially Allowed NSSAI to the SMF. If the the AMF determines to replace the HPLMN S-NSSAI received from the UE with the HPLMN Alternative S-NSSAI or the AMF receives the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI provided by the UE, the AMF sends both HPLMN S-NSSAI and HPLMN Alternative S-NSSAI to the SMF.

Question 3: In the above requirement, why would AMF replace HPLMN Alternative S-NSSAI in case of LBO roaming? (for example, why would congestion or unavailability of a slice in HPLMN impact an equivalent slice being used in VPLMN by an LBO roaming UE?)

Answer: In case of LBO roaming, SA2 agrees that the HPLMN alternative S-NSSAI is not required to be notified to SMF.

2. Actions:

To CT4 group.

ACTION: 	SA2 asks CT4 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160-Ad Hoc-e	22nd – 29th January, 2024			Online

TSG-SA2 Meeting #161		26th February – 1st March, 2024		Athens, GR