3GPP TSG-WG SA2 Meeting #160	S2-2312274

# Chicago, US, Nov 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5102 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Replacement in case the replaced S-NSSAI is available again |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the current descriptions in clause 5.15.19, if an S-NSSAI is to be replaced with an Alternative S-NSSAI, the handover procedure including any PDU session associated with the S-NSSAI to be replaced shall continue unaffected by the Network Slice Replacement. However, when the the replaced S-NSSAI is available again and and the AMF determines for the UE to use the replaced S-NSSAI, the handover procedure including any PDU session associated with the Alternative S-NSSAI is not described. It is proposed that in such scenario the handover procedure shall also continue unaffected by the Network Slice Replacement. In addition, in case of Network Slice Replacement when an S-NSSAI becomes unavailable or congested or this S-NSSAI is available again, it is expected that the AMF ensures the S-NSSAI to be used is supported at UE current location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Network Slice Replacement in case the replaced S-NSSAI is available again |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network Slice Replacement in case the replaced S-NSSAI is available again is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.19	Support of Network Slice Replacement

The Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested. The Network Slice Replacement may be triggered in the following cases:

-	If the NSSF detects that an S-NSSAI becomes unavailable or congested (e.g. based on OAM or NWDAF analytics output), it sends network slice availability notification for the S-NSSAI to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The NSSF notifies the AMF when the S-NSSAI is available again.

-	If the PCF detects that an S-NSSAI becomes unavailable or congested for a UE (e.g. based on OAM or NWDAF analytics output), it sends access and mobility related policy notification to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The PCF notifies the AMF when the S-NSSAI is available again for the UE.

-	The OAM sends notification to AMF when an S-NSSAI becomes unavailable or congested (and also when this S-NSSAI becomes available again) and provides the Alternative S-NSSAI to AMF.

The network slice associated with the Alternative S-NSSAI is assumed in this specification to have NS-AoS to be covering at least the NS-AoS of the replaced network slice.

NOTE 1:	There are no means for the PLMN to prevent the UE from obtaining service in the Alternative network slice in cells outside the NS-AoS of the replaced network slice but within the NS-AoS of the Alternative network slice if the Alternative network slice NS-AoS exceeds the NS-AoS of the replaced network slice.

Based on the notification above from NSSF or PCF or OAM, the AMF may determine that an S-NSSAI is to be replaced with Alternative S-NSSAI. For roaming case, the AMF may receive network slice availability notification of the HPLMN S-NSSAI from NSSF in the HPLMN via NSSF in VPLMN, to trigger the Network Slice Replacement of the HPLMN S-NSSAI as described in clause 5.15.6.

NOTE 2:	It is recommended that, the operator configures to use only one mechanism when triggering the Network Slice Replacement for S-NSSAI.

The AMF determines the Alternative S-NSSAI for a UE registered with the S-NSSAI based on the notification from NSSF or PCF, or based on local configuration if the NSSF or PCF do not provide an alternative S-NSSAI. The Alternative S-NSSAI shall be supported at UE current location. If AMF cannot determine the Alternative S-NSSAI for the S-NSSAI, e.g. PCF or NSSF doesn't provide Alternative S-NSSAI, the AMF may further interact with the PCF to determine the Alternative S-NSSAI. The event trigger in AMF for interacting with PCF is described in clause 6.1.2.5 of TS 23.503 [45].

The UE indicates the support of Network Slice Replacement feature during the UE Registration procedure. For supporting UE in CM-CONNECTED state and if there is a PDU Sessions in the UE context associated with the S-NSSAI that needs to be replaced, the AMF provides the Alternative S-NSSAI for this S-NSSAI in the Allowed NSSAI and in the Configured NSSAI, if not included yet, and the mapping between S-NSSAI(s) to Alternative S-NSSAI(s) to the UE in UE Configuration Update message as follows:

-	for non-roaming UEs, the AMF provides the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE.

-	for roaming UEs when the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the AMF provides the mapping of the VPLMN S-NSSAI to the Alternative VPLMN S-NSSAI to the UE.

-	for roaming UEs when the HPLMN S-NSSAI has to be replaced by an Alternative HPLMN S-NSSAI, the AMF provides the mapping of the HPLMN S-NSSAI to the Alternative HPLMN S-NSSAI to the UE.

NOTE 3:	The Alternative S-NSSAI or the Alternative HPLMN S-NSSAI do not have to be one of the Subscribed S-NSSAIs as long as they can be mapped to a HPLMN S-NSSAI that is part of the Subscribed S-NSSAIs.

For the supporting UE when the UE has a NAS signalling connection, i.e. it is CM-CONNECTED or it has become CM-CONNECTED, e.g. through a Service Request procedure or through a UE registration procedure, if the AMF determines that the S-NSSAI is to be replaced and there is a PDU Session associated with the S-NSSAI in the UE context (see also NOTE 3), the AMF sends the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE in the UE Configuration Update message or in the Registration Accept message.

NOTE 3:	It is left to AMF local policy whether to send the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE when there is no PDU session associated with the S-NSSAI or wait and send the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE when the UE establishes a PDU Session associated with the S-NSSAI.

During a new PDU Session establishment procedure for a S-NSSAI,

-	if the UE has received together with the Allowed NSSAI a mapping of the S-NSSAI to an Alternative S-NSSAI, the UE shall provide both the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message. When the AMF receives the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message, the AMF includes both the Alternative S-NSSAI and the S-NSSAI to the SMF in Nsmf_PDUSession_CreateSMContext service operation.

-	if the UE has not yet received with the Allowed NSSAI a mapping of the S-NSSAI to the Alternative S-NSSAI, the UE provides only the S-NSSAI in the PDU Session Establishment message. If the AMF determines that the requested S-NSSAI is to be replaced with the Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the Alternative S-NSSAI and provides both the Alternative S-NSSAI and the S-NSSAI to the SMF in Nsmf_PDUSession_CreateSMContext service operation.

The SMF proceeds with the PDU Session establishment using the Alternative S-NSSAI. The SMF sends the Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message.

For existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, after the AMF sends mapping of the S-NSSAI to the Alternative S-NSSAI to the supporting UE in UE Configuration Update message, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to Alternative S-NSSAI and includes the Alternative S-NSSAI as follows (see details in clause 4.3.3 of TS 23.502 [3]):

-	If the SMF determines that the PDU Session is to be retained (e.g. if the anchor UPF can be reused with the alternative S-NSSAI and SSC mode 1), the SMF sends the Alternative S-NSSAI to the UPF in the N4 message, to the NG-RAN in N2 message and to the supporting UE in PDU Session Modification Command message. The S-NSSAI provided to the (R)AN and to the UPF is the Alternative S-NSSAI.

-	If the SMF determines that the PDU Session is to be re-established, the SMF sends the Alternative S-NSSAI to the supporting UE either in PDU Session Modification Command if the PDU Session is of SSC mode 3, or in PDU Session Release if the PDU Session is of SSC mode 2 or SSC mode 1, to trigger the re-establishment of the PDU Session. The UE includes both, the S-NSSAI and the Alternative S-NSSAI in the PDU Session Establishment message.

When the AMF is notified that the S-NSSAI is available again (e.g. the congestion of the S-NSSAI has been mitigated), if the AMF has configured the supporting UE with the Alternative S-NSSAI, and the AMF determines for the UE to use the replaced S-NSSAI again, the AMF reconfigures the supporting UE (e.g. by using UE Configuration Update procedure or in the next registration procedure) to use the replaced S-NSSAI again by removing the mapping of the replaced S-NSSAI to Alternative S-NSSAI. The replaced S-NSSAI shall be supported at UE current location.

If there is an existing PDU Session associated with the Alternative S-NSSAI, the AMF updates the SMF(s) of the PDU Session(s), by Nsmf_PDUSession_UpdateSMContext service operation, causing the PDU Session to be transferred to the S-NSSAI.

During a handover procedure, the handover procedure (including any PDU session associated with the S-NSSAI to be replaced or with the S-NSSAI to be recovered) shall continue unaffected by the Network Slice Replacement. Any Network Slice Replacement for the S-NSSAI shall not take place during the handover.

# * * * * End of changes * * * *