3GPP TSG-SA2 Meeting #160	S2-2312324

# Chicago, USA, 13 – 17 November, 2023						      (revision of S2-2311877)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0435 | rev | 5 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution update for ranging procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei], [HiSilicon], [Sony], CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures of SL-MT-LR involving LMF in clause 6.20 has the following issues: - The LCS Client or the AF may have the information of Apps applying Ranging/Sidelink Positioning location results, the UE location, UE capability, UE roles (e.g. target UE, reference UE), etc, so it may know which UE is more suitable to be the initiator UE that initiates the Ranging/SL Positioning. - When (H)GMLC selects the initiator UE that initiates the Ranging/SL Positioning, if the selected UE is not reachable, the following steps cannot be performed successfully. - For the the relative locations or distances and/or directions related to the UEs, it can obtained through the Ranging/SL Positioning provedure or through their absolute locations. But now the creteria is not clear. In rev5: 1) Based on the pre-meeting call discussion, it is proposed that Application Layer ID will be included in supplementary services message. The two related ENs can be removed. 2) Per RAN agreement, all UEs involved in Ranging should be in coverage and registered to the same PLMN. This assumption should be clearly stated. 3) The SLPP is defined in RAN spec, which should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rev2: merging texts from 10493. merging texts from 11006. other editorial changes received during online session. Clarifications, e.g., in the very last paragraph of 6.20.3 Undo 1st change of Rev1 Rev1 and original: - Propose target UE ID can be either either a SUPI or a LCS Correlation identifier - An initiator UE indication may be included to help (H)GMLC select the corresponding UE as the initiator UE. - If the UE is not reachable, the (H)GMLC does not select it as the initiator UE. - The LMF determins to obtain the relative locations or distances and/or directions related to the UEs through the Ranging/SL Positioning provedure or through their absolute locations, based on the UE capabilities of supported positioning method (SL, or Uu, or both), supported algrithms, accuracy, delay, etc. In rev5 the following changes (highlighted in green) are made: 1) Remove the two ENs about Application Layer ID transfer. 2) Clarify all UEs involved in Ranging should be in coverage and registered to the same PLMN. 3) Clarify SLPP is specified in RAN spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures of SL-MT-LR involving LMF in clause 6.20 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20.3, 6.20.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.20.5	5GC-MT-LR Procedure using SL positioning

The procedure is used to estimate the location of a UE by using the location of one or more Located UEs and the distance and/or direction between the UE and the Located UE(s).

NOTE: 	The procedure can be triggered by GMLC, e.g. if the location result of Target UE determined by previous MT-LR procedure for the same request cannot fulfil the required QoS.

Procedure of Figure 6.1.2-1 can be reused, with step 12 of Figure 6.1.2-1 replaced by the step 10-16 of Figure 6.20.3-1 with the following adaptations:

-	UE1 is the Target UE, and UEs 2 to n are Located UEs.

-	In step 10, the types of required location results is absolute solution, and the other UEs 2 to n are the candidate Located UE(s) if included. SL-MT-LR request also includes the indication of Target UE/LMF selecting Located UE.

-	In step 14, if UE1 receives the indication of LMF selecting Located UE in step 11, step 14 includes the obtained information of all the discovered Located UEs. If UE1 receives the indication of Target UE selecting Located UE in SL-MT-LR request, UE1 performs the Located UE selection, and SL-MT-LR response includes the selected Located UEs ID.

# * * * * Second change * * * *

6.20.3	Procedures of SL-MT-LR involving LMF

The SL-MT-LR procedure is used to estimate the relative locations or distances and/or directions between the UEs.

Figure 6.20.3-1 illustrates a procedure to enable an LCS Client or AF to obtain Ranging/Sidelink Positioning location results for a group of n UEs (n≥2), i.e. UE1, UE2, ..., UEn. In the procedure, the GMLC determines a UE among the n UEs to be designated UE1 (i.e. Target UE in TS 23.586 [40]) and one or more other UEs designated UE2, UE3, ..., UEn (n≥2) (i.e. Reference/Located UEs in TS 23.586 [40]). The Ranging/Sidelink Positioning location results may include absolute locations, relative locations or distances and directions related to the UEs, velocities and relative velocities based on the service request. The Ranging/Sidelink Positioning location results may be represented by a geographical coordinate or a local coordinate or both.

Procedure for periodic and triggered SL-MT-LR is defined in clause 6.20.4.

Figure 6.20.3-1: SL-MT-LR Procedure

Precondition:	It is assumed all UEs are in coverage and registered with the same PLMN that supports Ranging/Sidelink Positioning.

1.	The LCS Client or the AF (via NEF) sends an LCS service request to the (H)GMLC for Ranging/Sidelink Positioning location results for the n UEs which may each be identified by a GPSI or a SUPI. The request may include the required QoS, the required location results (e.g. absolute locations, relative locations or distances and/or directions related to the UEs, velocities and relative velocities), the SL reference UE(s) in case of relative locations, located UE, distance, or direction, and Coordinate ID defined in TS 38.455 [15] representing a local coordinate in case of absolute locations. The information of UE initiating the Ranging/SL Positioning may be included to help (H)GMLC select the corresponding UE in step 3. The (H)GMLC or NEF authorizes the LCS Client or the AF for the usage of the LCS service. If the authorization fails, the remaining steps are skipped and the (H)GMLC or NEF responds to the LCS Client or the AF with the failure of the service authorization.

In addition, an Application Layer ID shall be included for each of the n UEs to enable discovery of the UEs at step 12.

2.	The (H)GMLC invokes a Nudm_SDM_Get service operation towards the UDM of each of the n UEs to get the privacy settings of the UE identified by its GPSI or SUPI. The UDM returns the UE Privacy setting of the UE. The (H)GMLC checks the UE LCS privacy profile.

3.	The (H)GMLC invokes a Nudm_UECM_Get service operation towards the UDM of each of the n UEs (for which GPSI or SUPI is available), one at a time, using the GPSI or SUPI of each UE. The (H)GMLC selects the UE (e.g. which is treated as UE1 in following steps) that initiates the Ranging/SL Positioning and selects the corresponding serving AMF, based on UE subscription and UE reachability. If the UE is not reachable, the (H)GMLC does not select such UE to initiate the Ranging/SL Positioning.

NOTE:	The UDM is aware of the serving AMF address at UE registration on an AMF as defined in clause 4.2.2.2.2 of TS 23.502 [19]. The UDM is aware of a serving (V)GMLC address at UE registration on an AMF as defined in clause 4.2.2.2.2 of TS 23.502 [19].

4.	For a non-roaming case, this step is skipped. In the case of roaming, the (H)GMLC may receive an address of a (V)GMLC (together with the network address of the current serving AMF) from the UDM in step 3, otherwise, the (H)GMLC may use the NRF service in the (H)PLMN to select an available (V)GMLC in the (V)PLMN, based on the (V)PLMN identification contained in the AMF address received in step 3. The (H)GMLC then sends the location request to the (V)GMLC by invoking the Ngmlc_Location_ProvideLocation service operation towards the (V)GMLC. In the cases when the (H)GMLC did not receive the address of the (V)GMLC, or when the (V)GMLC address is the same as the (H)GMLC address, or when both PLMN operators agree, the (H)GMLC sends the location service request message to the serving AMF. In this case, step 4 is skipped. The (H)-GMLC also provides the LCS client type of AF, if received in step 1, or LCS client type of LCS client and other attributes to be sent to AMF in step 5.

5.	In the case of roaming, the (V)GMLC first authorizes that the location request is allowed from this (H)GMLC, PLMN or from this country. If not, an error response is returned. The (H)GMLC or (V)GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF serving UE1 to request Ranging /Sidelink positioning location results of the n UEs. The service operation includes the SUPI of UE1, Application layer IDs of the UEs, the client type and may include the required LCS QoS, the required location results (e.g. relative locations i.e., distances and/ directions betweenpairs of UEs, velocities and relative velocities) and other attributes as received or determined in step 1.

6.	If UE1 is in CM-IDLE state, the AMF initiates a network triggered Service Request procedure to establish a signalling connection with UE1.

If signalling connection establishment fails, steps 7-17 are skipped.

7-8.	If the indicator of privacy check indicates an action is needed, then same operation as that of step 7-8 of clause 6.1.2 is carried out.

9.	The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Ranging/Sidelink Positioning) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information received at step 5 e.g. required location results (e.g. relative locations i.e., distances and/directions between pairs of UEs velocities and relative velocities), SL reference UE(s) in case of relative locations, Application layer IDs of the UEs if received in step 5. The service operation includes a LCS Correlation identifier.

10.	The LMF sends an SL-MT-LR request to the serving AMF as a supplementary services message, using the Namf_Communication_N1N2MessageTransfer service operation, and the session ID parameter is set to the LCS Correlation identifier.

The SL-MT-LR request may include the application layer IDs of the other UEs 2 to n, the types of required location results (e.g. relative locations or distances and/or directions) and SL reference UE(s) in case of relative locations.

11.	The serving AMF forwards the SL-MT-LR request and a Routing identifier equal to the LCS Correlation identifier to UE1 using a DL NAS TRANSPORT message.

12.	UE1 attempts to discover the other UE 2 to n using their Application Layer IDs if not already discovered using procedure defined in clause 6.4 of TS 23.586 [40].

13.	UE1 obtains the sidelink positioning capabilities of the discovered UEs via the SLPP as defined in TS 38.355 [aa] if not already obtained.

14.	UE1 returns a supplementary services SL-MT-LR response to the serving AMF in an UL NAS TRANSPORT message and includes the Routing identifier received in step 11.

The SL-MT-LR response indicates which of UEs 2 to n have been discovered and the sidelink positioning capabilities of the discovered UEs.

15.	The serving AMF forwards the SL-MT-LR response to the LMF indicated by the Routing identifier received at step 14 and includes a LCS Correlation identifier equal to the Routing identifier.

16.	Ranging/Sidelink Positioning of UE1 and the other discovered UEs occurs as for an SL-MO-LR as described for steps 10-19 of clause 6.20.1 with the difference that Ranging/Sidelink Positioning location measurement data or results are always returned to the LMF and the LMF indicates to UE1 at step 13 or step 14 of clause 6.20.1 whether the Ranging/Sidelink Positioning location results will be calculated by the LMF (at step 19) or by UE1 (at step 17).

For the undiscovered UEs or UEs for which the Ranging/Sidelink Positioning location measurements cannot be obtained based on their sidelink positioning capabilities among the other UEs 2 to n, the LMF may interact with GMLC to initiate the 5GC-MT-LR procedure for UE2 to n to get their absolute locations. In order to calculate the relative locations or distances and/or directions between UE1 and these UEs, if the abolute location of UE1 is not known yet, the LMF will trigger the 5GC-MT-LR procedure to derive the location of UE1. The LMF uses the  absolute locations of all the UEs to calculate the relative locations or distances and/or directions between the pairs of these UEs. LMF may determine the Ranging/Sidelink Positioning location results in local coordinate if Coordinate ID is received in step 9.

To fulfill the required QoS, the LMF may determine to use both Ranging/SL Positioning and Uu absolute Positioning to obtain the location results.

17-20.	The LMF returns the Ranging/Sidelink positioning location results via AMF and GMLC to the LCS Client or AF as in steps 13-15 and step 24 of clause 6.1.2. The results also include failure information of the UE(s) that was not discovered or the requested information was not possible to derive.

# * * * * End of changes * * * *