3GPP TSG-SA WG2 Meeting #160	S2-2312327

# Chicago, USA, November 13 - 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4588 |  | rev |  | - |  |  | Current version: |  |  | 18.3.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  | UICC apps |  |  |  | ME |  |  | Radio Access Network |  |  | Core Network | x |  |

| Title: | Correction of Application-Specific external parameter provisioning services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, a new set of parameters, named “Application-Specific Expected UE Behaviour parameters”, are introduced in clause 4.15.6.3f to be provisioned by external parties. Additionally, confidence and/or accuracy levels can also be associated with these parameters. But this new set of parameters and the confidence and/or accuracy levels are not included in the specification of the corresponding service operations of NEF and UDM. Moreover, in some service operations, the “Expected UE Behaviour parameters” is also missing. This CR adds the missing input parameters to those service operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating external parameter provisioning services of NEF and UDM to include “Application-Specific Expected UE Behaviour parameters” and also the confidence and/or accuracy levels |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.6.2, 5.2.3.6.3, 5.2.6.4.2, 5.2.6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

##### 5.2.3.6.2	Nudm_ParameterProvision_Update service operation

Service operation name: Nudm_ParameterProvision_Update.

Description: The consumer updates the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, Enhanced Coverage Restriction Information, ECS Address Configuration Information), 5G VN group related information (5G VN group data, 5G VN membership management), Multicast MBS group related information, or Application-Specific UE Behaviour.

Inputs, Required: AF Identifier, Transaction Reference ID(s).

Inputs, Optional: GPSI or SUPI, External Group ID, DNN, S-NSSAI, at least one of the Expected UE Behaviour parameters with associated confidence and/or accuracy levels or at least one of the Application-Specific Expected UE Behaviour parameters with associated confidence and/or accuracy levels or at least one of the Network Configuration parameters or 5G VN group related information, MTC Provider Information, Validity Time or Location Privacy Indication parameters or Enhanced Coverage Restriction Information or ECS Address Configuration Information, Multicast MBS related information, or Application-Specific UE Behaviour.

Outputs, Required: Transaction Reference ID(s), Operation execution result indication, or DNN and S-NSSAI specific Group Parameters.

Outputs, Optional: Transaction specific parameters, if available.

For Multicast MBS related information, refer to TS 23.247 [78].

********** Next Changes **********

##### 5.2.3.6.3	Nudm_ParameterProvision_Create service operation

Service operation name: Nudm_ParameterProvision_Create

Description: The consumer creates a Network Configuration with one or more parameters, a 5G VN group related information (e.g. 5G VN group data, 5G VN membership management), or Multicast MBS related information.

Inputs, Required: AF Identifier, Transaction Reference ID(s).

Inputs, Optional: GPSI, External Group ID, DNN, S-NSSAI, one or multiple Network Configuration parameters, one or multiple Expected UE Behaviour parameters with associated confidence and/or accuracy levels or one or multiple Application-Specific Expected UE Behaviour parameters with associated confidence and/or accuracy levels or ECS Address Configuration Information, or for 5G VN group creation, External Group ID and 5G VN group related information, MTC Provider Information, for Multicast MBS related information, or DNN and S-NSSAI specific Group Parameters.

Outputs, Required: Transaction Reference ID(s), Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available; Internal Group ID if the inputs include a new 5G VN configuration.

For Multicast MBS related information, refer to TS 23.247 [78].

********** Next Changes **********

##### 5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, ECS Address Configuration Information) or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management), or for Multicast MBS related information or DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, External Group ID at least one of the Expected UE Behaviour parameters with associated confidence and/or accuracy levels or at least one of the Application-Specific Expected UE Behaviour parameters with associated confidence and/or accuracy levels or at least one of the Network Configuration parameters or 5G VN related information or ECS Address Configuration Information, Validity Time or Location Privacy Indication parameters, MTC Provider Information, or Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

********** Next Changes **********

##### 5.2.6.4.3	Nnef_ParameterProvision_Create service operation

Service operation name: Nnef_ParameterProvision_Create

Description: The consumer creates a 5G VN group, or Multicast MBS related information group.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, one or multiple Expected UE Behaviour parameters with associated confidence and/or accuracy levels or one or multiple Application-Specific Expected UE Behaviour parameters with associated confidence and/or accuracy levels, External Group ID for 5G VN group creation or for multicast MBS group creation, External Group ID, 5G VN group related information (e.g. 5G VN group data, 5G VN membership management), MTC Provider Information, Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

********** End of Changes **********