SA WG2 Meeting #160	S2-2312343

# October 13th – 17th, 2023; Chicago, USA

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: OAM-Assisted Energy-related information exposure

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#1 where the energy-related information are obtained from OAM.

# 1	Discussion

Considering i) OAM id already capable to collect significant amount of energy-related information, and ii) NOTE 1 of KI#1, this paper proposes a solution for exposure of energy-related information where the OAM is instructed to collect the required information by a new NF and then the information is exposed to the consumer via NEF.

# 2 Proposal

It is proposed to update TR 23.700-66 according to the following text.

********** First Change **********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 22.882: "Study on Energy Efficiency as service criteria".

[6]	3GPP TS 28.554 "5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 28.310 "Management and orchestration; Energy efficiency of 5G".

[8]	3GPP TS 22.261 "Service requirements for the 5G system".

[x]	3GPP TS 28.532 "Management and orchestration; Generic management services"

[y]	3GPP TS 28.550 "Management and orchestration; Performance assurance"

********** Next Change (All new text) **********

## 6.x	Solution #X: OAM-Assisted Energy-Related Information Exposure

### 6.x.1	Key Issue mapping

This proposed end-to-end solution addresses the aspect of the KI#1 as follows:

- Whether and what network energy related information can be exposed.

- This solution only considers exposing the energy-related information which OAM is capable of collecting.

- At what granularity (e.g. per network slice, UE, NF, PDU Session, QoS flow, etc) the network energy related information can be exposed.

- This solution enables exposure of energy-related information per network slice, per NF and 5G core segment of network slice.

- How the network energy related information is exposed.

- This solution proposes extensions of NEF services to expose energy-related information to the authorized consumer.

- How and what network energy related information from the Network entities (i.e. RAN nodes, 5GC NFs) can be obtained in order to support network energy related information exposure.

- This solution is assisted by OAM to collect the required energy-related information to be exposed.

### 6.x.2	Functional Description

The following consolidated potential requirements are summarized in clauses 6.3 and 6.4 of TR 22.882 [5]:

- CPR 6.3-1: Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity.

- CPR 6.3-2: Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party, independently from NG-RAN deployment scenarios.

- CPR 6.3-3: Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, through same update rate e.g. hourly or daily,

- CPR 6.4-1: Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party.

- CPR 6.4-3: Subject to operator policy, the 5G system shall provide means for the trusted 3rd party, to configure which network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party, needs to be exposed along with the information on energy consumption for serving this 3rd party.

- CPR 6.4-5: Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time.

According to these requirements, the following issues need to be addressed:

- The exposed energy-related information should be conformed according to the operator’s policy and the agreement with the 3rd party.

- In addition to energy-related information, the performance-related information can also be exposed to the consumer.

- The consumer can request the information for a specific time period.

- Exposing energy-related information at slice level should be supported; moreover, the consumer can request energy-related information of the serving functions of the slice (independent of RAN deployment); i.e. the energy-related information of the core segment of the slice.

OAM can assist to address these issues in two aspects:

- The operator’s policy and also the agreements with consumers are configured/stored in OAM; therefore, the compliance of the exposed information w.r.t. the operator’s policy and the agreements can be checked via assistance from OAM.

- OAM provides the KPIs (and corresponding measurements) defined in TS 28.554 [8] which can be used as the energy-related and also performance data to be exposed to the consumer.

Based on these observations, and assuming that the required information to be exposed can be provided by OAM, in this paper, an OAM-assisted solution is proposed with the following characteristics:

- A new network function, named EMF (Energy Manager Function), is introduced that provides exposure services for the AF (either directly to the trusted AF or via NEF to the untrusted AF).

NOTE 1: The functionality of EMF is not limited to the exposure of energy-related information; the functionalities and services by EMF may be extended based on further requirements in other KIs as well.

- The exposure services, provided by EMF, allows the AF to request energy-related and performance-related information for a network slice, 5G core segment of network slice, or NF for a period of time.

NOTE 2: It is assumed that the energy-related and performance-related information at the specified granularity can be provided by OAM.

- The EMF is responsible to check the requested information w.r.t. to the operator policy and 3rd party agreements, to initiate data collection procedure in OAM and providing the required information to AF.

### 6.x.3	Procedures

#### 6.x.3.1	OAM-Assisted energy-related information exposure procedure

Figure 6.x.3-1 depicts the procedure for the proposed solution where EMF provides the exposure services to the AF via assistance by the OAM.

3b. createMeasurementJob(Performance Measure Config)

Trusted AF

9. Nemf_EnergyInfoExposure_notify(Exposure Data)

9b. Nnef_EnergyInfoExposure_notify (Exposure Data)

9a. Nemf_EnergyInfoExposure_notify (Exposure Data)

7. Download data file from OAM

1. Nemf_EnergyInfoExposure_subscribe(Exposure Config)

1c. Nemf_EnergyInfoExposure_subscribe (Exposure Config)

1b. Authorization

1a. Nnef_EnergyInfoExposure_subscribe (Exposure Config)

AF

NEF

EMF

OAM

2. Operator’s policy & 3rd party agreement check

3a. createMeasurementJob(Energy Measure Config)

4a. Subscribe(Input)

4b. Subscribe(Output)

5. Performance and Energy Data/KPI collection/computing

6. Notification (notifyFileReady)

8. stopMeasurementJob(jobIDs)

Untrusted AF

Trusted AF

Untrusted AF

Figure 6.x.3-1: OAM-assisted Energy and Performance related information exposure

1.	The EMF service consumer (i.e. AF) subscribes to the energy related information either by invoking the Nemf_EnergyInfoExposure_subscribe if it is a trusted AF or by invoking Nnef_EnergyInfoExposure_subscribe if it is an untrusted AF. In the latter case, NEF invokes Nemf_EnergyInfoExposure_subscribe on behalf of the AF after successful authorization of the request. The parameters that can be provided by the EMF service consumer (i.e. “Exposure Config”) are described in clause 6.x.3.2.

2.	The EMF by consulting to OAM, checks whether the AF is authorized the get the requested information in Step 1 w.r.t. the operator’s policy and the 3rd party agreement as well.

3.	According to the requested information specified in step 1, the EMF initiates two measurement jobs in OAM, one for the energy-related information and the second one for the performance-related information by invoking createMeasurementJob as defined in clause 6.1.1 of TS 28.550 [y]. The parameters that can be provided by the EMF (i.e. “Energy Measure Config” and “Performance Measure Config”) are described in clause 6.x.3.2.

4a.	By invoking Subscribe (Input), as explained in clause 11.6.1.3.2 of TS 28.532 [x], the EMF subscribes to the notification(s) related to the measurement job initiated in step 3 from the OAM.

4b.	The EMF gets response Subscribe (Output) form OAM, as defined in Clause 11.6.1.3.3 of TS 28.532 [x], indicating whether the subscription is successful or not.

5.	The OAM executes the measurements jobs, i.e., collects the required energy and performance related information according to the inputs specified in step 3.

6.	The OAM notifies the EMF by Notification (notifyFileReady), as defined in Clause 11.6.1.1 of TS 28.532 [x], when the measurement data file(s) are ready.

7.	The EMF fetches the collected measurement data by using the file transfer protocols as defined in clause 11.6.2 of TS 28.532 [x].

8.	[Optional] The EMF stops the measurement job initiated in step 3 if they are no longer needed, by invoking stopMeasurementJob as defined in clause 6.1.2 of TS 28.550 [y].

9.	The EMF sends back the requested information by invoking Nemf_EnergyInfoExposure_notify either to the AF (in the case of trusted AF) or to the NEF (in the case of untrusted AF) where in the latter case, NEF notifies the AF via Nnef_EnergyInfoExposure_notify.

#### 6.x.3.2	Service operations inputs

The proposed procedure in clause 6.x.3.1 can be used to expose various energy-related and performance-related information at three different levels of granularities including network slice level, 5G core segment of the slice level, and NF level. The required exposure information is determined by the input of Nemf_EnergyInfoExposure_subscribe, i.e. “Exposure Config”, which is then used by the EMF to specify the input of createMeasurementJob and Subscribe (Input).

The content of “Exposure Config” provided by the consumer of the Nemf_EnergyInfoExposure_subscribe or Nnef_EnergyInfoExposure_subscribe include the following parameters:

- Granularity level: NS level (end-to-end slice or only the core part), NF level

- ID: NSI ID, NF Instance ID

- Energy-related information: Any combination of parameters defined clause 6.7 of TS 28.554 [6] subject to the granularity level.

- [OPTIONAL] Performance-related information: Any combination of parameters defined clauses 6.3 and 6.8 of TS 23.554 [6] subject to granularity level.

- [OPTIONAL] Time period information: The information (e.g., start, stop, period, etc.) of the period of time that the AF is interested in the exposed data.

Based the Exposure Config, the EMF provides the appropriate input for the createMeasurementJob where the input parameters, defined in clause 6.1.1.2 of TS 28.550 [y], can be customized as follows:

- iOCName, iOCInstanceList, measurementCategoryList are determined according to the Granularity level, ID, Energy-related information, Performance-related information

- granularityPeriod, reportingPeriod, startTime, stopTime are determined according to the Time period information.

### 6.x.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

MEF:

-	New network functionalities to provide the energy-related information exposure.

-	New network functionalities to gather energy and performance related information from OAM based on the requirements from the AF.

NEF:

-	Extending service to expose the services provided by the EMF.

********** End of Changes **********