3GPP TSG-WG SA2 Meeting #160	S2-2312387

# Chicago, USA, Nov 13 – 17, 2023		(revision of S2-2310296)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0029 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on GMLC services for supporting ranging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.6.3 of TS 23.586, the ranging AF may contact GMLC directly or via NEF to provide a ranging or sidelink positioning operation request to the 5GS. In clause 5.6.2.3 of TS 23.586, in the control plane-based procedure, the Client UE sends the 5GC-MO-LR request to AMF and this triggers the AMF to send the Ranging/SL positioning request to the GMLC for the Ranging/SL Positioning service. In both case, GMLC needs provide service for supporting ranging to AMF or NEF. This paper proposes to update GMLC service description accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update GMLC service for supporting ranging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GMLC service do not support ranging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.5	Service-based interfaces

Nlmf:	In addition to the relevant services defined in TS 23.273 [8], if the LMF support Ranging/SL Positioning service, it may be used to provide service to other NFs related to it.

Ngmlc:	In addition to the relevant services defined in TS 23.273 [8], if the GMLC support Ranging/SL Positioning service, it may be used to get the location of Located UE/Reference UE from GMLC using the Application Layer ID in case of Sidelink Positioning and Ranging operation, and services provided by GMLC are consumed by NEF to request relative locations, distance, or direction between UEs.

Nudm:	In addition to the relevant services defined in TS 23.501 [2] for Nudm, in the case of Ranging/SL Positioning service, services provided by UDM are used to get the related subscription information to AMF during Initial registration procedure or UE Configuration Update (UCU) procedure to inform AMF subscription information has changed.

Npcf:	In addition to the relevant services defined in TS 23.501 [2] for Npcf, in the case of Ranging/SL Positioning service, services provided by H-PCF are used to provide SL Positioning and Ranging Service related parameters to V-PCF for UE and NG-RAN in the roaming case.

Nudr:	In addition to the relevant services defined in TS 23.501 [2] for Nudr, in the case of Ranging/SL Positioning service, services provided by UDR are used to notify the PCF and the UDM of the update of the SL Positioning and Ranging-based service related information.

Namf:	In addition to the relevant services defined in TS 23.501 [2] for Namf, in the case of Ranging/SL Positioning service, services provided by AMF are consumed by PCF to provide the Ranging/SL Positioning service related parameters for the UE and the NG-RAN to AMF, and to enable the AMF create or update UE context related to Ranging/SL Positioning service.

Nnef:	In addition to the relevant services defined in TS 23.501 [2] for Nnef, in the case of Ranging/SL Positioning service, services provided by NEF are used by the Application Server to update Ranging/SL Positioning service related information of 5GC.

Nnrf:	In addition to the relevant services defined in TS 23.501 [2] for Nnrf, in the case of Ranging/SL Positioning service, services provided by NRF are used to discover the PCF that supports Ranging/SL Positioning.

# * * * * End of changes * * * *