3GPP TSG-WG2 Meeting #160	S2-2312397

# Chicago, US, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4536 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Broadcasting the Event ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN WG2 has agreed (S2-2310120) that the gNB always broadcasts Event ID in SIB9 if it supports and uses the feature. The change of the Event ID is used to inform the UE to reconnect to the network to retrieve the latest clock quality metrics from gNB. Based on NOTE 1 in clause 5.27.1.12 in TS 23.501, the UE cannot learn the clock quality metrics that meet the pre-configured thresholds: NOTE 1: NG-RAN is assumed not to provide clock quality metrics better than the pre-configured threshold, i.e. if a clock quality metric is better than the corresponding threshold, the NG-RAN reports the threshold value to the UE in an RRC message instead. The consequence of this is that the UE should not be mandated to reconnect to the network if the pre-configured thresholds are not exceeded. This means that the Event ID broadcasted by the gNB should indicate if any of the thresholds are exceeded. This ensures that the UE does not reconnect to the gNB unnecessarily when the UE is camping under a new gNB and thresholds are not exceeded in the new gNB. The current description does not state what the gNB should do if the TSS attribute change while exceeding the pre-configured threshold. In this case the gNB should generate and broadcast a new value of Event ID, so that the UEs reconnect again to the network, and learn the new clock quality metrics. The current description is not clear on how the TSCTSF learns whether the clock status report from gNB indicates clock failure or improvement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 4.15.9.5.2, add a reference to clause 5.27.1.12 in TS 23.501 on the details when and how the NG-RAN broadcasts the reference report ID, and how the UE reacts upon it. Remove the duplicate description from clause 4.15.9.5.2. Clarify how the TSCTSF can determine that the clock status report from a RAN node is for clock failure, i.e. when the status report includes one or more of the TSS attributes listed in Table 5.28.3.1-2 of TS 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE reconnects to the gNB every time the UE changes the gNB, or when any of the clock attributes change within the gNB (due to a change of Event ID). This causes significant amount of signalling to the network, and impacts negatively to the UE battery consumption. Unclarity in the specification, how the TSCTSF knows the network clock status in gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.5.1, 4.15.9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR5049 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

##### 4.15.9.5.1	Network timing synchronization status

Figure 4.15.9.5.1-1: Procedure for TSCTSF subscription to RAN and/or UPF/NW-TT timing synchronization status

0.	The AF requests creation or modification of ASTI or PTP based time synchronization service as described in clauses 4.15.9.4 and 4.15.9.3 including clock quality detail level and clock quality acceptance criteria (if applicable) in the request.

If the request is received at the NEF, it checks whether the AF is authorized to send the request and forwards the request to the TSCTSF.

If network timing synchronization status reports are provisioned using node-level signalling via control plane, the TSCTSF determines the serving AMF(s) and the UPF/NW-TT nodes (if applicable) for the UE(s) that needs to initiate network timing synchronization status monitoring.

Otherwise, if network timing synchronizations status reports are provisioned via OAM, steps 1-3 and 5-7 are skipped.

1-2.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution or by PTP instance activation, modification):

Upon the reception of the clock quality detail level and clock quality acceptance criteria (if applicable) in the AF request in step 0, the TSCTSF needs to be subscribed to NG-RAN timing synchronization status updates at the NG-RAN nodes that may provision access stratum time distribution information to the target UE.

Namf_Communication_NonUeN2InfoSubscribe in step 1 initiates the subscription for the NG-RAN timing synchronization status updates in the AMF. In step 2, the AMF configures the NG-RAN node(s) for the timing synchronization status reporting. Based on local configuration, AMF may configure some or all NG-RAN nodes for the timing synchronization status reporting.

3.	(When the procedure is triggered by the AF request for PTP instance activation, modification and if the UPF/NW-TT is involved in providing time information to DS-TT):

Upon the reception of the clock quality acceptance criteria in the AF request in step 0, the TSCTSF needs to be subscribed to UPF/NW-TT timing synchronization status updates at the UPF/NW-TT that may provision time information via PTP to the target UE. UPF/NW-TT timing synchronization status updates provisioning may be configured via OAM or via UMIC.

4.	The RAN node is pre-configured for the thresholds for each timing synchronization status attribute as described in clause 5.27.1.12 of TS 23.501 [2]. When the network timing synchronization status exceeds or meets again any of the pre-configured thresholds, the NG-RAN node detects a change on its timing synchronization status.

5-6.	If the NG-RAN node detects a change on its timing synchronization status as described in clause 5.27.1.12 in TS 23.501 [2] and timing synchronization status reporting is configured via the AMF in step 2, the NG-RAN node notifies the AMF providing a NG-RAN timing synchronization status update. The update can contain the information elements listed in Table 5.27.1.12-1 of TS 23.501 [2], and the scope of the timing synchronization status (as described in clause 5.27.1.12 of TS 23.501 [2]). The AMF forwards the update to the subscribed TSCTSF.

7.	If the UPF/NW-TT detects a change on its timing synchronization status and timing synchronization status reporting is configured via UMIC in step 3, the UPF/NW-TT notifies the TSCTSF providing a UPF/NW-TT timing synchronization status update via UMIC. The update can contain the information elements listed in Table 5.28.3.1-2 of TS 23.501 [2].

8.	Upon the reception of a change in the NG-RAN and/or NW-TT timing synchronization status update, the TSCTSF shall determine if the UE is impacted and whether the clock quality acceptance criteria can still be met. For each scope of the timing synchronization status received from the RAN in step 6, if the status indicates degradation or failure (i.e. the status report includes one or more of the TSS attributes listed in Table 5.28.3.1-2 of TS 23.501 [2]), TSCTSF may set the Area of Interest to gNB node ID(s) or Cell IDs in the scope of the timing synchronization status. The TSCTSF uses the resulted Area of Interest to subscribe for the UE presence in Area of Interest from the AMF that is serving the gNB node ID as described in clause 5.27.1.12 of TS 23.501 [2]. If the status indicates recovery (i.e. the status report does not include any of the TSS attributes), the TSCTSF may remove the corresponding scope from the subscription.

Upon reception of notification for the UE presence in the Area of Interest, if the TSCTSF determines that the UE is impacted for ASTI service, the TSCTSF performs steps 15-16 in clause 4.15.9.4 to notify the AF the acceptance criteria result.

Upon reception of notification for the UE presence in the Area of Interest, if the TSCTSF determines that the UE is impacted for PTP service, the TSCTSF performs steps 9-11 in clause 4.15.9.3.2 to notify the AF the acceptance criteria result.

*************************** Next Change ****************************

##### 4.15.9.5.2	5G access stratum time distribution status reporting to the UE

Figure 4.15.9.5.2-1: Procedure for reporting RAN timing synchronization status to subscribed UE

1.	Creation or update of ASTI or PTP based time synchronization service (AF requested ASTI or PTP based time synchronization service is described in clauses 4.15.9.4 and 4.15.9.3; subscription based ASTI is described in clause 4.28.2.1) including in the request clock quality detail level indication and optionally the clock quality acceptance criteria. The clock quality level indication and optionally the clock quality acceptance criteria are sent to the NG-RAN and the UE reconnect indication is sent to the UE as described in clause 4.15.9.4.

2.	The RAN node is pre-configured for the thresholds for each timing synchronization status attribute as described in clause 5.27.1.12 of TS 23.501 [2]. If there is a change on its primary source so that the thresholds are exceeded or met again, the NG-RAN node indicates the status change to the UEs via SIB information as described in clause 5.27.1.12 in TS 23.501 [2].

3.	(When the UE is in RRC_INACTIVE or RRC_IDLE state):

If supported by the UE and if the UE determines, based on the reading of SIB9 information as described in clause 5.27.1.12 in TS 23.501 [2], a clock quality information update is available, and the AMF has provided the UE reconnection indication in step 10 of clause 4.15.9.4, the UE reconnects to the network as described in TS 24.501 [25].

4.	(When the UE is in RRC_CONNECTED state):

The NG-RAN node notifies the UE providing clock quality information as configured in step 10 of clause 4.15.9.4 (i.e. sending clock quality metrics or acceptable/not acceptable indication) using unicast RRC signalling whenever the UE enters RRC_CONNECTED state and while the UE remains in RRC_CONNECTED, when any of the clock quality metrics or the acceptable/not acceptable indication for the UE changes.

*************************** End of Changes ****************************