3GPP TSG-WG2 Meeting #160	S2-2312426

# Chicago, US, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5044 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set Handling for unidentified PDUs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LS reply from SA4 in S2-2310131 raises concerns on the current procedure in clause 5.37.5.1 in TS 23.501 where the UPF marks the unidentified PDUs as part pf PDU Sets. This issue applies e.g. when RTP and RTCP are multiplexed on a single IP address and port, as commonly used e.g. in WebRTC. The outcome is that the PDU Set Sequence number in GTP-U header and in RTP header do not match after the first unidentified PDU. Another issue is that the RAN would follow the given PDU Set Delay Budget (PSDB) and PDU Set Error Rate (PSER) for the unidentified PDU (e.g. RTCP PDU), rather than the PDB and PER that are given for the QoS Flow and would be followed otherwise. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.37.5, the PSA UPF leaves the unidentified PDUs unmarked for PDU Set information. In clause 5.7.7.2, PSDP is not applied for unidentified PDUs. In clause 5.7.7.3, PSER is not applied for unidentified PDUs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN follows the given PDU Set Delay Budget (PSDB) and PDU Set Error Rate (PSER) for the unidentified PDU (e.g. RTCP PDU), rather than the PDB and PER that are given for the QoS Flow and would be followed otherwise. Unclear how the SMF determines to set the End of data burst marking indicator to the UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.2, 5.7.7.3, 5.37.5.1, 5.37.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 5.7.7.2	PDU Set Delay Budget

The PDU Set Delay Budget (PSDB) defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). PSDB applies to the DL PDU Set received by the PSA UPF over the N6 interface, and to the UL PDU Set sent by the UE.

NOTE:	To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. This maximum inter arrival time does not exceed PSDB. NG-RAN behaviour when the SLA is not fulfilled is out of scope of this specification.

A QoS Flow is associated with only one PDU Set Delay Budget. The value of the PDU Set Delay Budget is the same in UL and DL. PSDB is an optional parameter that may be provided by the PCF. The provided PSDB can be used by the NG-RAN to support the configuration of scheduling and link layer functions.

When the PSDB is available, the PSDB supersedes the PDB for the given QoS Flow for PDUs that belong to a PDU Set.

The AN PSDB is derived at NG-RAN by subtracting CN PDB (as described in clause 5.7.3.4) from the PSDB.

*************************** Next Change ****************************

#### 5.7.7.3	PDU Set Error Rate

The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related PDU Set losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access).

NOTE 1:	In this Release, a PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

NOTE 2:	How RAN enforces PSER is up to RAN implementation.

A QoS Flow is associated with only one PDU Set Error Rate. PSER is an optional parameter. If the PSER is available, the PSER supersedes the PER for PDUs that belong to a PDU Set. The value of the PDU Set Error Rate is the same in UL and DL.

*************************** Next Change ****************************

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based QoS Handling can be applied for GBR and non-GBR QoS Flows.

In addition to the PDU related service information, the AF may provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates transport protocol (e.g. RTP, SRTP), transport protocol header extensions (e.g. RTP Header Extension for PDU Set Marking as defined in TS 26.522 [179]), payload type and format (e.g. H.264, H.265), and format parameters (e.g. H.264 profile level and packetization mode) used by the service data flow.

AF provided PDU Set QoS Parameters and Protocol Description may be used in determining PCC Rules by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the Protocol Description may be used for identifying the PDU Set information by the PSA UPF.

When the SMF receives a PCC rule containing one or more PDU Set QoS Parameters (PSER, PSDB and PSIHI), the SMF adds these PDU Set QoS parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based QoS Handling without receiving PCC rules from a PCF.

For the downlink direction, if the PSA UPF identifies that a PDU belongs to a PDU Set, the PSA UPF marks the PDU for PDU Set Information, otherwise the PSA UPF leaves the PDU unmarked. The identification and marking of PDUs for PDU Set Information is described in clause 5.37.5.2.

*************************** Next Change ****************************

#### 5.37.5.2	PDU Set Information and Identification

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE 1:	In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.

NOTE 2:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

The SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the Protocol Description used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

The PSA UPF identifies PDUs that belong to PDU Sets and marks them for PDU Set Information. The PSA UPF can identify the PDU Set Information using the Protocol Description and the received transport protocol headers and payload or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [179].

For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

*************************** End of Changes ****************************