3GPP TSG-SA WG2 Meeting #160	S2-2312488

# 13-17 November 2023, Chicago, Illinois, USA							 (revision of S2-2311874)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0031 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the UE-only operation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [vivo], [Interdigital], MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the UE only operations to the Ranging/Sidelink Positioning control, it is understood that the server UE is used as the role of LMF for offering method determination, assistant data distribution and/or location calculation functionalities for Sidelink Positioning and Ranging based service. Then following the same logic of SL-MO-LR in TS23.273, the triggers to invoke the SL positioning server UE function is needed. UE1 needs to send request to the SL positioning server UE as the “LMF” for Ranging/Sidelink Positioning control invocation. The UE1 needs to indicate the other UEs 2 to n using the Application layer ID, indicates any assistance data needed, indicates the Ranging/SL positioning result types needed (e.g. absolute locations, relative locations or distances and directions between pairs of UEs). The required QoS for Ranging/SL positioning is also indicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When desciding to use the SL positioning server UE, the UE1 needs explictily to request the SL positioning server UE to perform the Ranging/Sidelink Positioning control with necessery indication, e.g., the UE2 to n info, assistant data, QoS requirement. The SL positioning server UE following the LMF in the SL-MO-LR procedure in TS23.273 performs the capabilities request, assistant provide and ranging result provide. - Specify the case to use the SL sever UE where the Network-based Operation is not supported by the 5GC network which is based on the AMF indication in the registration accept message. - Clarified in clause 6.8 which steps are handled in SLPP and which in PC5, based on RAN2 agreements and joint conference call outcomes. - Collapsed steps 9-11 of Figure 6.8.1-1 into a single step of SLPP procedures under RAN2 responsibility. - Removed assistance data request from step 6 of Figure 6.8.1-1 (at this stage UE1 does not know what method will be selected and what assistance data will be needed). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the SL positioning server UE is triggered in the Ranging/Sidelink Positioning control and how the SL positioning server UE performs the Ranging/Sidelink Positioning control for the UE-only operations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 6.8	Procedures of Ranging/Sidelink Positioning control

Either UE-only Operation or Network-based Operation is applied in the Ranging/Sidelink Positioning control procedures.

UE-only Operation as specified in this clause is applied for the following cases:

-	Neither Target UE nor SL Reference UE is served by NG-RAN.

-	Network-based Operation is not supported by the 5GC network as described in clause 5.2.3.

.

-	SL-MO-LR request is rejected by the network.

For any other cases, Network-based Operation as specified in clauses 6.20 of TS 23.273 [8] is applied.

Figure 6.8.1-1 Procedures for Ranging/Sidelink Positioning control (UE-only operation)

1.	UE1 (i.e. Target UE or a SL reference UE) may receive a Ranging/SL Positioning Service request from:

1a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1.

For absolute location, the service request includes the SL Positioning Client UE's user info and Target UE's user info, and required positioning QoS, and may also include the user info for a list of candidate Located UE(s).

For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, SL Reference UE's user info(UE2/.../UEn), and Ranging/SL Positioning QoS information.

1b.	RSPP application layer.

The service request includes type of the result (i.e. absolute location, relative location or ranging information) and the required QoS.

2.	UE1 discovers UE2/.../UEn (i.e. SL Reference UEs/Located UEs) as defined in clause 6.4, if needed.

NOTE 1:	Details of security related procedures during UE discovery are developed by SA WG3.

3.	If none of UE1/.../UEn are served by NG-RAN or the serving network does not support Ranging/SL Positioning, UE-only Operation is applied.

4.	UE1 and UE2/.../UEn may perform capability exchange via SLPP procedures. Step 4 may be performed during step 5 and/or between steps 7 and 8 triggered by the request for capabilities from the SL Positioning Server UE.

NOTE 2:	Details of SLPP signalling and procedures are developed by RAN WG2.

5.	If UE1 does not support SL Positioning Server functionalities or UE1 opts to select a SL Positioning Server UE different from UE1, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a separate UE) is discovered (if not yet discovered in step 2) and selected. If a SL Positioning Server UE is co-located with a SL Reference UE/Located UE or operated by a separate UE, UE1 discovers and selects the SL Positioning Server UE as described in clauses 5.2.3 and 6.4 and requests SL Positioning Server UE to participate in the Ranging/Sidelink positioning as described in step 6.

NOTE 3:	Details of security and privacy related procedures during SL Positioning Server UE discovery and operation are developed by SA WG3.

6.	If a SL Positioning server UE is selected, UE 1 sends a Ranging/SL positioning request to the selected SL Positioning Server UE. This request indicates the other UEs 2 to n using the Application layer IDand the Ranging/SL positioning result types needed (e.g. absolute locations, relative locations or distances and directions between pairs of UEs). The required QoS for Ranging/SL positioning is also indicated.

7.	The SL Positioning Server UE sends a request over PC5 to UE1 for the capabilities of UEs 1 to n. If step 4 was not performed, UE1 retrieves capabilities of UE2/…/UEn via SLPP procedures.

8.	UE1 sends a response over PC5 to the SL Positioning Server UE with the related capabilities.

9.	The SL Positioning Server UE downselects from UE1/.../UEn to determine which SL Positioning Reference UEs will be used in the subsequent SL positioning/ranging operation. The selection of SL Positioning Reference UEs is left to SL Positioning Server UE implementation.

10.	Sidelink Positioning assistance data is transferred among UE1/ .../UEn and the SL Positioning Server UE via SLPP procedures.

11.	The SL Positioning Server UE retrieves the needed location information from UE1/ …/UEn via SLPP procedures if the SL Positioning Server UE performs the result calculation. This step comprises any SL-PRS measurement among UE1/ …/UEn that is needed to carry out the positioning method selected by the SL Positioning Server UE.

12.	 Based on the type of the result received in step 6 and the location information received in step 11, absolute location, relative location or ranging information is calculated at the SL Positioning Server UE.

13.	The SL Positioning Server UE sends a Ranging/SL positioning response over PC5 to UE1 including the result that is required in step 6.

14.	Ranging/SL Positioning result is transferred to:

14a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1;

14b.	Application layer.

* * * * End of Change * * * *