3GPP TSG-WG SA2 Meeting #160	S2-2312492

# Chicago, United States, November 13th – 17th, 2023	(was S2-2310760)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0952 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Nnwdaf_MLModelTraining Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect input/output parameters for Nnwdaf_MLModelTraining_Unsubscribe service operation: - Move the parameters “Cause code (e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.). Final aggregated ML model information (if FL has finished) or updated aggregated ML model information (if FL is suspended).” from optional outputs to optional inputs in clause 7.10.3. Correct the other errors in clause 7.10.4 for the Nnwdaf_MLModelTraining service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes proposed in this CR include: - Moved the parameters “Cause code” and “Final aggregated ML model information or updated aggregated ML model information” from optional outputs to optional inputs for the Nnwdaf_MLModelTraining_Unsubscribe service operation in calsue 7.10.3. - Moved “NOTE: The detail reason in the cause code is up to stage 3.” to the position after the parameter “Termination Request” in calsue 7.10.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect input/output parameters for Nnwdaf_MLModelTraining Service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.3, 7.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 7.10.3	Nnwdaf_MLModelTraining_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Unsubscribe

Description: Terminate NWDAF ML model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: Cause code (e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.). Final aggregated ML model information (if FL has finished) or updated aggregated ML model information (if FL is suspended).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.Cause code (e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.). Final aggregated ML model information (if FL has finished) or updated aggregated ML model information (if FL is suspended).

* * * Next of Changes * * *

### 7.10.4	Nnwdaf_MLModelTraining_Notify service operation

Service operation name: Nnwdaf_MLModelTraining_Notify

Description: NWDAF notifies the consumer instance of the trained ML model that has subscribed to the specific NWDAF service. The NWDAF can also use this service to indicate to consumer it will terminate the ML model training.

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML model training.

Inputs, Optional:

-	Set of the tuple (Analytics ID, ML model Information as defined in clause 6.2F.2);

-	ML Correlation ID, when for Federated Learning;

-	Corresponding Use case context;

-	Termination Request: this parameter indicates that NWDAF requests to terminate the ML model training, i.e. NWDAF will not provide further notifications related to this request, with cause code (e.g. NWDAF overload, not available for the FL process anymore, etc.);

NOTE:	The detail reason in the cause code is up to stage 3.

-	ML Model ID: this parameter identifies the provisioned ML model;

-	ML Model Accuracy: The model accuracy of the global ML model, which is calculate by the FL Client NWDAF using the local training data as the testing dataset;

-	Status report of FL training: Accuracy of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the FL Client NWDAF during FL procedure;

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * *