3GPP TSG SA WG2 Meeting #160	S2-2312496

Chicago, United States, November 13 - 17, 2023	(was S2-2310371)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0936 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PFD Determination Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Input data IP flow including URL and domain which is more than the IP flow concept, needs to be updated as application traffic flow. The Application ID handling in UPF as described in the Input data table note is implementation related, not mandatory requirement. The procedure figure consumer NEF needs to be replaced as consumer NEF(PFDF), the Option1 and Option2 need to be show as optional lines. The application identifier is mandatory required for the know application, while in the procedure still describes as "may" for application identifier. Also other errors needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the IP Flow Information as Application Traffic Flow Information in the input data table and updates the table note description on Application ID handling in UPF as optional. Correct procedure figure with NEF(PFDF) as consumer, update optional step lines and remove "may" for application identifier in step 3 description and adding the missing descriptions in step 6. Also with other corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct IP flow input description and UPF handling, Not correct procedure figure and not aligned procedure descriptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.2, 6.16.3, 6.16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### 6.16.2	Input Data

The NWDAF collects information on user data traffic from NF(s) for a known Application ID optional with specific S-NSSAI and/or DNN and retrieves the existing PFDs from the NEF(PFDF). The detailed input data are described in Table 6.16.2-1.

Table 6.16.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| Application ID (NOTE 1) | UPF | Identifies the application |
| Application Traffic Flow Information (1..max) | UPF | Per Application Traffic flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF (PFDF), as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain Name matching criteria and information about applicable protocol(s). |
| NOTE 1: The Application ID may be mapped in the UPF to a detection algorithm that has a wider scope than the Application ID provided by the consumer. |  |  |

NOTE:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

# *** 2nd Change ***

### 6.16.3	Output Analytics

The output analytics of PFD Determination is defined in Table 6.16.3-1. The output analysis may be used to provision new or updated PFDs information for known applications. The NWDAF may assign a confidence to the PFD Determination statistics based on the analytics of input data provided by UPF and NEF(PFDF).

Table 6.16.3-1: PFD Determination statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| List of application traffic description |  |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing 3-tuple server side IP address, server port number and protocol of uplink/downlink application traffic as described in TS 23.503 [4]). |
| > URL list | the significant parts of the URL to be matched, e.g. host name. |
| > Domain name information | A Domain name matching criteria and information about applicable protocol(s). |
| Confidence | Confidence on the provided application traffic description for the Application ID. |

# *** 3rd Change ***

### 6.16.4	Procedures

Figure 6.16.4-1 shows the procedure that a consumer can use to request PFD Determination analytics.

Figure 6.16.4-1: A procedure for NWDAF-assisted PFD Determination

1.	The Consumer NF (NEF(PFDF)) requests or subscribes to the NWDAF to request PFD Determination analytics for a known application identifier. The Target of Analytics Reporting is set to Any UE.

The Analytics Filter Information may optionally include the S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF) for the Application ID provided in step 1, as listed in Table 6.16.2-1.

3.	The NWDAF collects information from the UPF (event "UserDataUsageMeasures") as listed in Table 6.16.2-1 either via SMF (Option 1) or directly from UPF (Option 2) as defined in clause 5.8.2.17 of TS 23.501 [2]. NWDAF discovers the SMF (Option 1) or UPF (Option 2) through NRF as defined in clause 4.15.4.5.3 of TS 23.502 [3]. The event "UserDataUsageMeasures" is defined in clause 5.2.26.2 of TS 23.502 [3], the subscription to the event includes an Application ID set to a value that allows the NWDAF to retrieve input information to derive PFD Determination Analytics for the Application ID that is provided by the consumer.

NOTE:	The Application ID provided by the consumer and the Application ID provided to the UPF to retrieve input data for PFD Determination Analytics can be different ones.

4.	UPF reports the data directly to NWDAF.

5.	The NWDAF derives PFD Determination analytics, e.g. new or updated PFD information for the existing Application ID. When the consumer provides the preferred level of accuracy, the NWDAF provides PFD Determination Analytics that reaches this level to the consumer.

6.	In the case that PFD information for the existing Application ID is new or to be updated, the NWDAF notifies or responds the PFD Determination analytics to the consumer NF (i.e. NEF(PFDF)) with PFD Information and the confidence of accuracy.

*** End of Changes ***