3GPP TSG-SA WG2 Meeting #160	 S2- 2312530

# Chicago, US, Nov 13 – 17, 2023																	(rev of-)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4604 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of the PDN connection to use for the delivery of URSP updates in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in DP S2-2312528: - if the negotiation of URSP Provisioning support fails for the first PDN connection established by the UE in EPS, the PCF is not able to deliver URSP rules to the while in EPS, even when other ongoing PDN connections for the UE could provide this support. - if the PDN connection in which the UE Policy Container (UE STATE INDICATION) was received from the UE is terminated or lost the PCF is not able to deliver URSP rules to the UE in EPS. - It is unclear/undefined how the PCF for the PDU session determines the PDN connection to use for the delivery of URSP in EPS after a 5GS to EPS handover for the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) If the negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C fails for the first PDN connection the UE retries the procedure for the next PDN connections until the negotiation succeeds for one of the PDN connections. Then, when the UE receives the Indication of URSP Provisioning Support in EPS PCO from SMF+PGW-C in the PDN Connectivity Accept message for such PDN connection, the UE initiates the UE requested bearer resource modification procedure over that PDN connection and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message. 2) The PCF for the PDU session determines that a PDN connection supports the delivery of URSP in EPS if: - During the establishment of the PDN connection the SMF+PGW-C indicates the PDN connection supports the delivery of URSP in EPS or - The PDN connection has been transferred from 5GS and the corresponding SMF+PGW-C indicated support for URSP delivery in EPS during the establishment of the SM Policy association in 5GS. Note that for the scenario of EPC interworking it is assumed the MME supports ePCO. 3)At SM Policy association establishment the SMF+PGW-C provides an indication about whether it supports ePCO and URSP delivery in EPS for the PDN connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NW cannot delivery URSP rules to the UE in EPS if the SMF+PGW-C for the first PDN connection the UE establishes in EPS doesn’t support the provisioning of URSP rules in EPS. if the PDN connection in which the UE Policy Container (UE STATE INDICATION) was received from the UE is terminated or lost the PCF is not able to deliver URSP rules to the UE in EPS. It is unclear/undefined how the PCF for the PDU session selects a PDN connection for URSP delivery after a 5GS to EPS handover for the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.2a.0, 4.11.0a.2a.1, 4.11.0a.2a.2.1, 4.11.0a.2a.2.2, 4.11.0a.2a.3, 4.11.0a.5, 4.11.0a.2a.5 and 5.2.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related with LS sent by CT3 during meeting CT3#130 (C3-234513) asking for feedback about PDN connection selection for URSP delivery after 5GS to EPS handover. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

##### 4.11.0a.2a.0	General

To Support URSP provisioning in EPS, the SMF+PGW-C receives the Indication of URSP Provisioning Support in EPS PCO and ePCO support indication from the UE, if the SMF+PGW-C also supports URSP Provisioning Support in EPS and ePCO, it selects a PCF which also supports URSP Provisioning in EPS to establish the SM Policy Association based on the received Indication of URSP Provisioning Support in EPS PCO and local configuration.

When the SMF+PGW-C receives the UE Policy Container ePCO in Bearer Resource Command during UE requested bearer resource modification procedure, it forwards transparently the UE Policy Container to PCF for the PDU Session in the Npcf_SMPolicyControl_Update Request including indication that "UE Policy Container received" PCRT was met. When the PCF for Session Management receives UE Policy Container from PCF for the UE, it forwards the UE Policy Container to SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify Request. The PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

When URSP Provisioning is supported in EPS, the PCF for the PDU Session may establish the UE Policy Association with PCF for the UE:

When the first UE Policy Container is received from the UE for an SM Policy association.

-	During 5GS to EPS mobility with N26 when the SMF+PGW-C notifies the PCF for the PDU Session about a RAT-Type change via Npcf_SMPolicyControl_Update.

The PCF for the UE generates the URSP and sends it to the PCF for the PDU Session in the UE Policy Container via Npcf_UEPolicyControl_UpdateNotify Request. The PCF for the UE triggers the re-evaluation of applicable URSPs and determines an update of URSP is needed for the UE when an event as described in clause 6.1.2.2.3 of TS 23.503 [20] occurs in the PCF for the PDU Session.

The procedures for the UE Policy association defined in clauses 4.16.11, 4.16.12 and 4.16.13 apply in general in EPS with the main differences below:

-	AMF is replaced by the PCF for the PDU Session.

-	Delivery of UE Policy Containers from the PCF for the UE to the PCF for the PDU Session is done via Npcf_UEPolicyControl service instead of via UE Configuration Update procedure.

URSP delivery via EPS is supported in roaming scenarios, both for a PDN connection established in Home Routed and for a PDN connection established in LBO.

This clause captures the enhancement to the interaction with PCC in clause 4.11.0a.2 to support URSP delivery via EPS.

* * * Next Changes * * *

##### 4.11.0a.2a.1	SM Policy Association Establishment

The following additions are applicable to clause 4.16.4 (SM Policy Association Establishment procedure):

In addition to step 4 the PCF determines whether the PDN connection supports the provisioning of URSP rules in EPS by checking URSP delivery in EPS support indication provided by the SMF+PGW-C. The SMF+PGW-C provides that indication based on local capabilities and configuration.

* * * Next Changes * * *

4.11.0a.2a.2.1	PDN connection establishment in EPS

The following additions are applicable to clause 4.16.5.1 (SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1 if the PCF receives both the indication that the PCRT "UE Policy Container received" was met and the UE Policy Container, steps 4a to 4c below are executed; otherwise step 5 in clause 4.16.5.1 is executed.

NOTE:	If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps 4b-4d are internal to PCF.

-	Step 4a: The PCF for the PDU Session discovers the PCF for the UE by querying the BSF and using the SUPI received in Npcf_SMPolicyControl_Create Request as described. If no result is obtained from the BSF the PCF for the PDU Session selects the PCF for the UE by querying NRF.

-	Step 4b: The PCF for the PDU Session establishes a UE Policy Association towards the PCF for the UE as described in clause 4.11.0a.2a.5.

-	Step 4c: If the PCF for the UE provides any Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response the PCF for the PDU Session takes them into account for the generation of Policy Control Request Triggers to the SMF+PGW-C.

* * * Next Changes * * *

4.11.0a.2a.2.2	5GS to EPS mobility

During 5GS to EPS mobility procedure, the PCF for the PDU Session determines whether to establish a UE Policy Association towards the PCF for the UE for the purpose of delivery of URSP in EPS. During EPS to 5GS mobility procedure the PCF for the PDU Session terminates the UE policy association if established during 5GS to EPS mobility.

The following additions are applicable to clause 4.16.5.1 (SMF initiated SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1, the PCF determines that 5GS to EPS or EPS to 5GS mobility applies by checking a change in RAT and Access-Type. If 5GS to EPS mobility applies steps 4a to 4d below are executed; if EPS to 5GS applies the PCF for the PDU Session checks if a UE Policy association was created during 5GS to EPS mobility and in that case, it requests the termination of the UE Policy association as described in clause 4.11.0a.2a.8; otherwise step 5 in clause 4.16.5.1 is executed.

-	Step 4a: For non-roaming and Home Routed roaming the (H-)PCF for the PDU Session determines whether the UE supports provisioning of URSP Rules in EPC by checking the indication that is stored within the UE context Policy Subscription data in UDR. If the UE supports provisioning of URSP Rules in EPC the flow continues in step 4b if a UE Policy Association towards the PCF for the UE is not already established.

For LBO roaming the V-PCF for the PDU Session has no access to UDR and it is not able to determine whether the UE supports provisioning of URSP rules in EPC. In this case it determines based on local configuration whether to establish a UE Policy association towards the V-PCF for the UE.

The PCF considers that every SM Policy association transferred from 5GS to EPS support the provisioning of URSP rules to the UE if the corresponding SMF+PGW-C indicated support for URSP delivery in EPS during the establishment of the SM Policy association in 5GS.

NOTE:	If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps 4b-4d are internals to PCF.

-	Step 4b: For the non-roaming and Home Routed roaming the (H-)PCF for the PDU Session discovers the PCF for the UE by querying the BSF using the SUPI of the UE as described in clause 6.3.7.4 of TS 23.501 [2].

For LBO roaming the V-PCF for the PDU Session discovers and selects an instance of the V-PCF for the UE based on the available PCF instances obtained from the NRF or locally configured information in the V-PCF, depending on operator's policies, as described in clause 6.3.7.1 of TS 23.501 [2].

-	Step 4c: The PCF for the PDU Session establishes a UE Policy Association towards the PCF for the UE as described in clause 4.11.0a.2a.5. This request includes an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS Mobility").

-	Step 4d: If the PCF for the UE provides any Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response the PCF for the PDU Session takes them into account for the generation of Policy Control Request Triggers to the SMF+PGW-C in Npcf_SMPolicyControl_Update answer.

* * * Next Changes * * *

##### 4.11.0a.2a.3	PCF initiated SM Policy Association Modification

For the purpose of URSP delivery in EPC a new condition to initiate the SM Policy Association Modification procedure is added triggered by an interaction from the PCF for the UE. This interaction applies when the PCF for the PDU Session has previously established a UE Policy Association towards the PCF for the UE for the delivery of URSP Rules in EPS as defined in clauses 4.11.0a.5 and 4.11.0a.2a.2.

The following additions are applicable to clause 4.16.5.2 (PCF initiated SM Policy Association Modification procedure):

NOTE:	If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps below are internals to PCF.

-	Step 1e: Alternatively, the PCF for the UE provides a UE Policy Container and/or an update in the Policy Control Request Triggers applicable to the UE by invoking Npcf_UEPolicyControl_UpdateNotify as described in clause 4.16.12.2, steps 3-4.

-	Step 4: If the PCF for the PDU Session receives a UE Policy Container from the PCF for the UE the PCF for the PDU Session first selects one of the ongoing PDU Sessions for the related UE in EPC that supports the provision of URSP rules to the UE according to clauses 4.11.0a.2a.1 and 4.11.0a.2a.2.2, and includes that UE Policy Container in Npcf_SMPolicyControl_update notify request for that PDN connection.

If the PCF for the PDU Session receives an update in the Policy Control Request Triggers applicable to the UE the PCF for the PDU Session determines whether an update on the current Policy Control Request Triggers needs to be sent to the SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify request.

* * * Next Changes * * *

#### 4.11.0a.5	PDN Connection Establishment

During establishment of non-emergency PDN connection in the EPC, the UE and the SMF+PGW-C exchange information via PCO as described in clause 5.15.7 of TS 23.501 [2]. For UE with 5GC NAS capability disabled (i.e. N1 mode is disabled), the UE may also allocate a PDU Session ID and send it to the SMF+PGW-C via PCO. If the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the SMF+PGW-C provides an S-NSSAI to the UE, the SMF+PGW-C should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs and that the S-NSSAI is not subject to Network Slice-Specific Authentication and Authorization, by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the SMF+PGW-C discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]). If the SMF+PGW-C is in a VPLMN, the SMF+PGW-C uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values. If the S-NSSAIs supported by the SMF+PGW-C are all subject to NSSAA, then the SMF+PGW-C should reject the PDN connection establishment. If the selected S-NSSAI is subject to NSAC and EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NSAC_NumberOfUEsUpdate services operation and/or the Nnsacf_NSAC_NumberOfPDUsUpdate services operation to check if the selected S-NSSAI is available as described in clause 4.11.5.9. The SMF+PGW-C uses the Nudm_SDM_Subscribe service operation to subscribe the change of the Session Management Subscription data. If the SMF+PGW-C is notified from UDM with subscription data change, the SMF+ PGW-C takes actions for the PDN connection as described in clause 5.17.2.1 of TS 23.501 [2].

As described in TS 23.548 [74], during establishment of a PDN connection, a UE that hosts EEC(s) may indicate to the SMF+PGW-C, in the PCO, that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s). If the UE indicated in the PCO that it supports the ability to receive ECS address(es) via NAS, the SMF+PGW-C may provide the ECS Address Configuration Information (as described in clause 6.5.2 of TS 23.548 [74]) to the UE in the PCO. The SMF+PGW-C may derive the Edge Configuration Server Information based on local configuration, the UE's location and/or UE subscription information.

The SMF+PGW-C may use the bearer modification procedure without bearer QoS update to send the UE a PCO with updated ECS Address Configuration Information as defined in clause 6.5.2 of TS 23.548 [74] to the UE.

During establishment of non-emergency PDN connection in the EPC, if PGW-C+SMF is selected for a UE that has 5GS subscription, the SMF may be configured to obtain the subscribed IP index from UDM as part of subscription data using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]).

During establishment of non-emergency PDN connection in EPC, if PGW-C+SMF is selected for a UE that has 5GS subscription, the SMF may be configured to obtain the subscribed User Plane Security Policy from UDM as part of subscription data using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]). The SMF uses the subscribed User Plane Security Policy as described in clause 5.10.3 of TS 23.501 [2].

During establishment of non-emergency PDN connection in the EPC, if SMF+PGW-C is selected for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN and if the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C+PGW-C may proceed as specified in first paragraph of this clause or select any S-NSSAI associated with the APN of the PDN connection. The SMF+PGW-C shall not provide any 5GS related parameters to the UE.

NOTE 1:	The SMF+PGW-C knows that the UE does not support 5GS NAS if the UE does not provide PDU Session ID in PCO (see clause 5.15.7 of TS 23.501 [2]).

During establishment of emergency PDN connection:

-	The SMF+PGW-C is to be derived from the emergency APN or to be statically configured in the Emergency Configuration Data in MME.

-	5GC interworking support with N26 or without N26 is determined based on UE's 5G NAS capability and local configuration (in the Emergency Configuration Data in MME).

-	The S-NSSAI configured for the emergency APN in SMF+PGW-C is not sent to the UE by the SMF+PGW-C. One S-NSSAI is configured for the emergency APN.

During establishment of non-emergency PDN connection and emergency PDN connection, if SMF+PGW-C is selected for a UE that does not support 5GC NAS, the SMF+PGW-C creates unique PDU Session ID for each PDN connection of the UE.

-	The unique PDU Session ID can be created based on the EPS Bearer IDs assigned by the MME for the PDN Connections associated with the UE and not be in the range of PDU Session ID values that can be created by a 5GC NAS capable UE.

-	If handover between EPS and EPC/ePDG (as specified in clause 8.6 of TS 23.402 [26]) is required, the SMF+PGW-C, based on operator configuration, may perform the following to ensure the uniqueness of PDU Session ID:

-	the SMF+PGW-C queries the UDM for any PDU Session ID(s) that are already registered in the UDM for the UE by invoking Nudm_UECM_Get service operation.

-	the SMF+PGW-C creates a PDU session ID that does not collide with the received PDU session ID(s).

-	When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C performs UDM registration using the Nudm_UECM_Registration service operation.

NOTE 2:	If the scenario that a UE handovers a PDN connection with APN1 established via MME to ePDG and then establishes a second PDN connection with APN2 via MME needs to be supported in a given deployment, then, based on operator configuration, the SMF+PGW-C can query the UDM to avoid that the same PDU Session ID value of first PDN connection would be assigned by the SMF+PGW-C for the second PDN connection.

When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C provides the ID of the PCF ID selected for the PDN connection in the UDM using the Nudm_UECM_Registration service operation.

A SMF+PGW-C may support L2TP as described in clause 4.3.2.4. In this case step 1 and step 7 of Figure 4.3.2.4-1 correspond to a PDN Connection establishment and a SMF+PGW-C replaces the SMF in that Figure.

To support User Plane Integrity Protection with EPS and policies that Require User Plane integrity protection to be used, at PDN connection establishment, the MME shall indicate to the SMF+PGW-C (via the Serving GW) whether the UE, the current eNB and the MME support User Plane Integrity Protection in EPS. If the MME and the UE support User Plane Integrity Protection, then the SMF+PGW-C informs the MME of the User Plane integrity protection policy (Required, Preferred, Not Needed) applicable to the PDN connection on a per-EPS bearer basis. In turn, the MME informs the eNB.

To support URSP Provisioning in EPS, during Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP Provisioning in EPS and ePCO capability, it provides the Indication of URSP Provisioning Support in EPS PCO in the Create Session Response. If the UE receives the Indication of URSP Provisioning Support in EPS PCO from SMF+PGW-C in the PDN Connectivity Accept message, the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message. If the default PDN connection is not established during Initial Attach procedure, the aforemetioned procedure happens during the first request for PDN connectivity. If the negotiation of the support of URSP provisioning in EPS fails for the first PDN connection the UE may retry the aforementioned procedure successively for the next PDN connectivity requests until it succeeds for one of them as specified in clause 5.17.8 of TS 23.501 [2]. When the PCF for the PDU Session receives the UE Policy Container ePCO from the PCF for the UE, it forwards the UE Policy Container ePCO to SMF/PGW-C in Npcf_SMPolicyControl_updateNotifyreq, then it is forwarded to UE in the Update Bearer Request by SMF/PGW-C.

* * * Next Changes * * *

##### 4.11.0a.2a.5	UE Policy Association establishment

To support the delivery of URSP in EPC as described in clause 6.1.2.2.3 of TS 23.503 [20], the PCF for the PDU Session may establish a UE Policy Association towards the PCF for the UE.

This procedure may be triggered by the following procedures:

1.	Initial Attach to EPC.

2.	PDN Connectivity Request during the first request for PDN connectivity.

3.	5GS to EPS handover or 5GS to EPS Idle Mode mobility.

Figure 4.11.0a.2a.5-1: UE Policy Association establishment procedure

In the non-roaming case, the V-PCF for the UE is not involved, and the PCF for the PDU Session interacts with the PCF for the UE. For the Home Routed roaming scenarios the V-PCF for the UE is not involved and the H-PCF for the PDU Session interacts with the H-PCF for the UE. For the LBO roaming scenarios, the V-PCF for the PDU Session interacts with the V-PCF for the UE and the V-PCF for the UE interacts with the H-PCF for the UE:

The procedure is based on the one defined in clause 4.16.11 for a UE in 5GC with the following differences:

The PCF for the PDU Session determines the need of establishment of a UE Policy Association when:

-	A UE Policy Container is received from the UE during SM Policy Association Update as described in clause 4.11.0a.2a.2.1; or

-	During 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2.

2.	Same as step 2 in clause 4.16.11, with the following exceptions:

The PCF for the PDU Session includes the UE Policy Container in case of SM Policy Association update or 5GS to EPS Mobility indication in case of SM Policy Association update at 5GS to EPS mobility.

For LBO roaming the V-PCF for the PDU Session discovers and selects an instance of the V-PCF for the UE based on the available PCF instances obtained from the NRF or locally configured information in the V-PCF, depending on operator's policies, as described in clause 6.3.7.1 of TS 23.501 [2]. The V-PCF for the PDU Session contacts the V-PCF for the UE and steps 3 and 4 are executed.

For non-roaming or Home Routed roaming the (H-)PCF for the PDU Session contacts the (H-)PCF for the UE and step5 follows.

3.	Same as step 3 in clause 4.16.11, with the following exceptions:

The UE Policy Container is received at SM Policy Association instead of initial registration. When the UE Policy Association is being established due to 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2, the V-PCF for a UE in the VPLMN needs to discover and select the same H-PCF for a UE that is serving the UE in the H-PLMN for 5GS by querying the BSF in the HPLMN. This is to be able to recover the information about the list of PSIs in the UE and the subscribed PCRTs from former UE Policy Association for the UE in 5GS as described in step 6. Therefore, the V-PCF for a UE in the VPLMN needs first to discover the BSF in the HPLMN.

NOTE 1:	For Home Routed roaming, the V-PCF for a UE is not involved, therefore in this roaming scenario it is not possible to retrieve the Application guidance on URSP Rule for inbound roamers of the PLMN and provide it to the H-PCF for the UE to be used as input for the generation of the URSP rules for the UE as described in clause 6.6.2.2.3 of TS 23.503 [20].

4.	Same as in clause 4.16.11.

5.	Same than in clause 4.16.11 step 5, with the following exceptions:

The subscription to notification of N1 message delivery of policy information to the UE using Namf_Communication_N1N2MessageSubscribe service is not applicable.

6.	The Same as step 6  in clause 4.16.11, with the following exceptions:

If "5GS to EPS Mobility indication was received in Npcf_UEPolicyControl_Create in step 2 instead, the (H-)PCF recovers the information about the list of PSIs in the UE,the subscribed PCRTs in 5GS from former UE Policy Association for the UE and the indication about the support of provisioning of URSP rules in EPC. If the UE doesn't support the provisioning of URSPs in EPS, the H-PCF rejects the establishment of the UE Policy Association.

NOTE 2:	In LBO roaming the V-PCF for a UE may trigger the establishment of the UE Policy Association for a UE that doesn't support the provisioning of URSP in EPS, as described in step 4a in clause 4.11.0a.2a.2.2.

The (H-)PCF provides the UE Policy Container in the Npcf_UEPolicyControl UpdateNotify Request.

7.	Same as in clause 4.16.11.

NOTE 3:	Steps 8 and 9 are executed only in LBO roaming.

8.	The V-PCF for the UE forwards the UE Policy Container to the V-PCF for the PDU Session by invoking Npcf_UEPolicyControl UpdateNotify Request. The V-PCF for the UE checks the size limit as described in clause 6.1.2.2.2 of TS 23.503 [20].

9.	The V-PCF for the PDU Session sends a response to the V-PCF for the UE.

10.	If the UE provides a UE Policy Container with the result of the URSP update to the PCF for the PDU Session as described in clause 4.11.0a.2a.2.3 the PCF for the PDU Session forwards the UE Policy Container to the (V-) PCF for the UE invoking Npcf_UEPolicyControl_Update Request.

11.	The (V-) PCF for the UE sends a response to the (V-) PCF for the PDU Session.

NOTE 4:	Steps 12 and 13 are executed only in LBO roaming.

12.	The V-PCF for the UE forwards the notification response to the H-PCF by invoking Npcf_UEPolicyControl_Update Request.

13.	The H-PCF sends a response to the V-PCF for the UE.

* * * Next Changes * * *

##### 5.2.5.4.2	Npcf_SMPolicyControl_Create service operation

Service operation name: Npcf_SMPolicyControl_Create.

Description: The NF Service Consumer can request the creation of a SM Policy Association and provides relevant parameters about the PDU Session to the PCF.

Inputs, Required: SUPI (or PEI in the case of emergency PDU Session without SUPI), PDU Session id, DNN, S-NSSAI and RAT Type.

Inputs, Optional: Information provided by the SMF, such as PDU Session Type, Request Type, Access Type, the IPv4 address and/or IPv6 prefix, PEI, GPSI, User Location Information, UE Time Zone, Serving Network identifier (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), Charging Characteristics information, Session-AMBR, subscribed default QoS information (5QI, 5QI Priority Level, ARP), UE support of reflective QoS (see TS 23.501 [2], clause 5.7.5.1), Number of supported packet filters for signalled QoS rules for the PDU Session (see TS 23.501 [2], clause 5.7.1.4), 3GPP PS Data Off status, Trace Requirements and Internal Group Identifier (see clause 5.9.7 of TS 23.501 [2]), DN Authorization Profile Index, DN authorized Session AMBR, Framed Route information (as defined in Table 5.2.3.3.1-1), MA PDU Request indication, MA PDU Network-Upgrade Allowed indication, ATSSS capabilities of the MA PDU Session, QoS constraints from the VPLMN (as defined in clause 5.7.1.11 of TS 23.501 [2]), Satellite Backhaul Category information, list of NWDAF instance Ids (used by AMF, SMF, UPF) and corresponding Analytics ID(s), PVS IP address(es) and/or PVS FQDN(s) and Onboarding Indication in the case of ON-SNPN (see clause 5.30.2.10.4.2 of TS 23.501 [2]), URSP rule enforcement that including Connection Capability, HR-SBO support indication (see clause 6.2.1.2 of TS 23.503 [20]) , URSP delivery in EPS support indication.

NOTE 1:	If SMF receives the DN authorized Session AMBR from the DN-AAA at PDU Session establishment, it includes the DN authorized Session AMBR within the Session-AMBR, instead of the subscribed Session-AMBR received from the UDM, in the request.

NOTE 2:  It is up to CT3 to determine whether the corresponding supportedFeature in Npcf_SMPolicyControl can be reused as URSP delivery in EPS support indication.

W-5GAN specific PDU Session information provided by the SMF is specified in TS 23.316 [53].

Outputs, Required: SM Policy Association ID defined in TS 29.512 [57]. Success or Failure.

Outputs, Optional: Policy information for the PDU Session as defined in TS 23.503 [20] and Policy Control Request Trigger(s) of SM Policy Association as defined in clause 6.1.3.5 of TS 23.503 [20].

See clause 5.8.2.2 of TS 23.501 [2] for allocation of IPv4 address and IPv6 prefix. The IPv6 prefix length is /64, or is shorter than /64 when Prefix Delegation applies.

See clause 4.16.4 for the detail usage of this service operation.

See clauses 4.22.2.1 and 4.22.3 for detailed usage of this service operation for ATSSS.

* * * End of Changes * * *