SA WG2 Meeting #160	S2-2312537

# Nov 13th – 17th, 2023; Chicago, US

Source: 	China Mobile

Title: 	Correction of Terminology Definition for TR 23.700-29

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: this paper proposes corrections of term. definition.

# 1	Discussion

The definition of S&F Satellite Operation in the clause 3.1 states

[quoted]

S&F Satellite operation: operation mode providing communication service (in storing and forwarding information) to a UE in periods of time and/or geographical areas in which the serving satellite is not simultaneously connected to the ground network via feeder link or ISL. For the case of UL, "store" refers to on-board storage of UL information from UE and "forward" refers to forwarding of stored UL information to the ground network. For the case of DL, "store" refers to on-board storage of DL information from the ground network and "forward" refers to forwarding of stored DL information to the UE.

[/quoted]

There are 2 points that are inaccurate:

- As shown above as “…the serving satellite is not simultaneously connected to the ground network via feeder link or ISL…” – here, using ISL is conflicting with what the architectural assumptions & principles in the section# 4 have specified.

In the section #4, it says:

- Store and Forward Satellite Operation assumes that UE-satellite-ground network connectivity can be intermittent as defined in clause 3.1.

- Store and Forward Satellite Operation shall work without ISL.

Evidently, the 1st sentence indicates the reference to the clause 3.1, while the 2nd sentence suggests S&F operation not involve ISL. However, the definition of S&F Satellite Operation in the clause 3.1 includes ‘or ISL’. This should be removed.

- For DL, the storage location of a message is on the ground when the feeder link is not available. Here, the definition uses ‘on-board storage’, which suggests the storage location be on a satellite in the case of feeder link being unavailable. This is not correct.

# 2 Proposal

It is proposed to update TR 23.700-29 for FS_5GSAT_ARCH_Ph3 as follows.

# * * * * First change * * * *

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

serving satellite: a satellite providing the satellite access to a UE (e.g. providing the serving cell(s)), either for GSO or NGSO. In the case of NGSO, the serving satellite is covering a given geographic area for a limited period of time due to the nature of the orbit.

S&F Satellite operation: operation mode providing communication service (in storing and forwarding information) to a UE in periods of time and/or geographical areas in which the serving satellite is not simultaneously connected to the ground network via feeder link. For the case of UL, "store" refers to on-board storage of UL information from UE and "forward" refers to forwarding of stored UL information to the ground network. For the case of DL, "store" refers to on-ground storage of DL information from the ground network and "forward" refers to forwarding of stored DL information to the UE.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NOTE:	These definitions are different from those in TS 22.261 [6].

# * * * * End of change * * * *