3GPP TSG-SA WG2 Meeting #160	S2-2312558

# Chicago, US, Nov 13th – Nov 17th , 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0992 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ML model accuracy information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. ML model accuracy information is defined as an accuracy value of the ML model or ML model metric to indicate accuracy of ML model, which is contained in Nnwdaf_MLModelProvision_Notify service operation as described in clause 6.2A.2. However, in Nnwdaf_MLModelMonitor_Notify service operation, ML model accuracy information is used to indicate number of inferences, network data and some other accuracy related information monitored by AnLF besides accuracy metrics, as described in clause 7.9.4. ML model information in these two service operation has different concept and is used in different procedure, respectively. To distinguish these two kinds of information, ML model accuracy information in Nnwdaf_MLModelMonitor_Notify service operation needs to be renamed. 2. There is no reporting threshold about deviation and no agreement on how to measure the deviations. Thus, the description about deviation needs to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Rename ML model accuracy information to the monitored ML model accuracy information in Nnwdaf_MLModelMonitor_Notify service operation in cluase 7.9.2. 2. Correct ML model accuracy information and avoid repeat description in clause 6.2E.3.3. 3. Romve description on deviation in clause 6.2E.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous concept on ML model accuracy information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.3.3, 7.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

#### 6.2E.3.3	Procedures for monitoring the analytics accuracy of an ML model

An NWDAF containing MTLF, due to the registration of monitoring of the analytics accuracy of an ML model received from NWDAF containing AnLF and local policies, subscribes to the NWDAF containing AnLF for receiving notifications of either the accuracy of the ML Model, or Analytics feedback information of the ML model. The NWDAF containing MTLF may get the Subscription endpoint address of the NWDAF containing AnLF from the information received in a previous registration or through a service discovery procedure at the NRF.

Figure 6.2E.3.3-1 illustrates the procedure either for monitoring the analytics accuracy of an ML model or for delivery of Analytics feedback information of an ML model. Nnwdaf_MLModelMonitor_Subscribe and Nnwdaf_MLModelMonitor_Notify service operations are used for the purposes. A service consumer, i.e. an NWDAF containing MTLF, subscribes at a service producer, i.e. an NWDAF containing AnLF, to be notified when either the analytics accuracy of the previously provisioned ML model is not sufficient, or Analytics feedback information is retrieved from analytics consumer NF.

Figure 6.2E.3.3-1: Procedure for monitoring the analytics accuracy of an ML model

0.	Upon the reception of an Nnwdaf_MLModelMonitor_Register request and based on local policy, the NWDAF containing MTLF determines to subscribe to the analytics accuracy monitoring for the ML model as defined in clause 5C.1.

1.	The NWDAF containing MTLF sends an Nnwdaf_MLModelMonitor_Subscribe request (Analytics ID(s), unique identifier(s) of the ML model(s) to be monitored, accuracy metrics to be monitored, optionally Reporting Threshold(s) or Reporting Period) to an NWDAF containing AnLF subscription endpoint.

When the NWDAF containing MTLF determines during the registration process described in clause 6.2E.3.2 that a subscription request for ML model accuracy monitoring to an NWDAF containing AnLF is related to a previous subscription for ML model accuracy information to a different NWDAF containing AnLF (due to changes in the provider of the ML accuracy monitoring for a given ML model, as an effect of analytics transfer among NWDAFs containing AnLF), the NWDAF containing MTLF may use as base for the new subscription request at the new NWDAF containing AnLF the parameters associated with the original subscription identification for the ML model accuracy information that was received in the registration request of the new NWDAF containing AnLF, as described in steps 1-2 of clause 6.2E.3.2.

2.	The NWDAF containing AnLF sends a response to the NWDAF containing MTLF.

3.	The analytics consumer NF may send Analytics feedback information to the NWDAF containing AnLF as described in clause 6.1.1.

4.	When step 1 is triggered, the NWDAF containing AnLF may start monitoring the analytics accuracy of the ML model(s), if it not started yet.

NOTE 1:	The NWDAF containing AnLF can monitor the analytics accuracy in many ways: e.g. comparing predictions of ML model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of analytics accuracy information etc.

5.	The NWDAF containing AnLF determines whether the analytics accuracy of the ML model is insufficient or the Reporting Period indicated in the Subscribe request is reached.

6.	Either the Analytics feedback information is retrieved at step 3 or the NWDAF containing AnLF detects the analytics accuracy of ML model is insufficient at step 5, the NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Notify request to the notification endpoint (e.g. the NWDAF containing MTLF). The Notify request includes either Analytics feedback information, or ML model accuracy monitored information as described in clause 7.9.4.

7.	The NWDAF containing MTLF sends a response.

8.	The NWDAF containing MTLF determines whether the ML model is degraded or not based on the notification at step 6. If the notification contains Analytics feedback information, the NWDAF containing MTLF may determine ML model degradation based on the procedures as described in clause 6.2E.2. Otherwise when the NWDAF containing MTLF has received the multiple analytics accuracy information, from one or more NWDAFs containing AnLF, it may consider that the ML model is degraded/to be updated (i.e. enough number analytics accuracy information received from one or more NWDAFs containing AnLF, indicating insufficient analytics accuracy).

NOTE 2:	The actual mechanism for the NWDAF containing MTLF for determining the degradation of the ML model degradation is an internal procedure of the NWDAF containing MTLF, e.g. the NWDAF containing MTLF calculate a global accuracy based on the analytics accuracy information and the number of inferences received from multiple NWDAFs containing AnLF.

9.	When an ML model is considered degraded / to be updated at step 8, the NWDAF containing MTLF re-trains the existing ML model or selects a new ML model. If the network data was not included in the Nnwdaf_MLModelMonitor_Notify request of step 5, the NWDAF containing MTLF may request data from the NWDAF containing AnLF, ADRF and/or other 5GS entities as specified in clause 6.2 and use the collected data for ML model retraining. The NWDAF containing MTLF notifies the NWDAF(s) containing AnLF with the updated trained ML Model Information by invoking Nnwdaf_MLModelProvision_Notify service operation, as described in clause 6.2A.

# * * * * Next changes * * * *

### 7.9.2	Nnwdaf_MLModelMonitor_Subscribe service operation

Service operation name: Nnwdaf_MLModelMonitor_Subscribe

Description: Subscribes to NWDAF for ML model accuracy (i.e. Analytics accuracy for an ML model as described in clause 6.2E.3.3) information monitored and Analytics feedback information for the analytics generated by the NWDAF with specific parameters.

Inputs, Required: (set of) Unique ML Model identifier(s), Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model monitor subscription), Accuracy metrics to indicate the metrics to calculate the accuracy information, reporting period to indicate the reporting periodicity in which the monitored ML Model accuracy information shall be reported, Accuracy reporting threshold to indicate the reporting condition above which the accuracy information shall be reported.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

# * * * * End of changes * * * *