3GPP TSG-SA2 Meeting #160	S2-2312597

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1000 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications related to FL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Text contains some ambiguities. For instance the same FL itteration is not repeated but a new one is performed. A Note reference 3GPP WG3, but should reference an appropriate 3GPP specification instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ambigupus text is improved. Reference to 3GPP SA3 is replaced by reference to TS 33.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous text and unclear references can confuse readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[33]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[34]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[35]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[36]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[37]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[38]	GSMA TS.06: "IMEI Allocation and Approval Process".

[39]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[40]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[41]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRMs). Integration Reference Point (IRP): Information Service (IS)".

[42]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and configuration management".

[43]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[44]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[45]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[46]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[47]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[x]	3GPP TS 33.501:	 “Security architecture and procedures for 5G system”.

2nd change

## 5.3	Federated Learning (FL) among multiple NWDAFs

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data sets. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data sets in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAFs for short), the main functionality includes:

FL server NWDAF:

-	discovers and selects FL client NWDAFs to participant in an FL procedure

-	requests FL client NWDAFs to do local model training and to report local model information.

-	generates global ML model by aggregating local model information from FL client NWDAFs.

-	sends the global ML model back to FL client NWDAFs to perform an additional training iteration if needed.

FL client NWDAF:

-	locally trains ML model as tasked by the FL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other FL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML model information to the FL server NWDAF.

-	receives the global ML model from FL server NWDAF and perform an additional training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML model either based on local configuration or when it receives an request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or when data cannot be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML model training, then FL procedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML model.

If the ML model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML model as described in clause 6.2C.2.2. The Subscription endpoint address of the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML model. The FL server NWDAF sends the ML model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML model training success.

NOTE 2:	How to authorize an MTLF to request ML models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is  specified in Clause X.10 of TS 33.501 [x].

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as described in clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using its local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

End of changes