3GPP TSG-WG SA2 Meeting #160	S2-2312627

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0476 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameters update for LMF user plane service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the LS C4-234522/S2-2311960, CT4 asked following question: SA2 has defined the Nlmf_Location_UPNotify service operation with the mandatory input parameter Notification Correlation Information. However, the Notification Correlation Information is not present as input in both Nlmf_Location_UPConfig and Nlmf_Location_UPSubscribe operations. To continue the stage 3 implementation, CT4 would like SA2 to clarify how does LMF know Notification Correlation Information and what’s the value of Notification Correlation Information. ‘Notification Correlation Information’ means ‘Notification Correlation ID’, and its value is assigned by AMF. The parameter of related service operation should be updated to make this clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update Nlmf_Location_UPConfig and Nlmf_Location_UPSubscribe service operation to cover Notification Correlation ID as an input paramater. - Change the ‘Notification Correlation Information’ into ‘Notification Correlation ID’ Nlmf_Location_UPNotify service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear clarify how does LMF know Notification Correlation Information and what’s the value of Notification Correlation Information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.7, 8.3.2.8, 8.3.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.2.7	Nlmf_Location_UPConfig service operation

Service operation name: Nlmf_Location_UPConfig

Description: The consumer NF requests to set up, modify or terminate a secure LCS-UP connection for a target UE.

Input, Required: Notification Target Address (+Notification Correlation ID), UE identifier (GPSI, SUPI).

Input, Optional: AMF reallocation indication, LCS-UP connection termination indication, LCS-UP connection set up request indication, Target LMF identifier.

Output, Required: Success/Failure indication

Output, Optional: Failure Cause (in the case of failure indication provided).

See clauses 6.18.2 and 6.18.3 for an example of usage of this service operation.

# * * * * Second change * * * *

8.3.2.8	Nlmf_Location_UPSubscribe service operation

Service operation name: Nlmf_Location_UPSubscribe

Description: Allow the consumer NF to subscribe about status of a secure LCS-UP connection for a target UE.

Input, Required: Notification Target Address (+Notification Correlation ID), UE identifier (SUPI and if available GPSI).

Input, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

# * * * * Third change * * * *

8.3.2.9	Nlmf_Location_UPNotify service operation

Service operation name: Nlmf_Location_UPNotify

Description: Allow the consumer NF get notified about status or modification of a secure LCS-UP connection for a target UE.

Input, Required: Notification Correlation ID, LCS-UP connection status (established, released or move indication).

Input, Optional: Target LMF identifier (only applicable in case of move of the connection).

Output, Required: None.

Output, Optional: None.

See clauses 6.18.1 and 6.18.3 for an example of usage of this service operation.

# * * * * End of changes * * * *