3GPP TSG-WG SA2 Meeting #160	S2-2312637

Chicago, United States, November 13 – 17, 2023	(revision of S2-231xxxx)

Source:	Samsung

Title:	New KI: NF selection for network energy saving.

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys/Rel-19

Abstract: This document proposes a new Key Issue description regarding the selection of Network Function with considering the energy saving and energy efficiency.

1. Introduction

This document proposes new Key Issues description regarding the following aspects:

- Selection of Networking Function with considering the energy saving and energy efficiency. WT#3 highlights the following parts:

- WT #3. Study 5GS enhancements (e.g., energy usage adjustment for NF from CN aspect, energy saving related decision making, NF selection leveraging NF energy states) for network energy saving including 5GC(NFs) and NG-RAN interactions, analytics, etc. Impacts on the UE are not ruled out e. g., for scenarios specified in TR 22.882 by SA1 EnergyServ.

Currently criteria of the Network Function selection mainly focus on the capability (e.g. support of certain feature), Service Area (e.g., Service area of the SMF), capacity related, UE location etc., however, lack of consideration on energy saving/energy efficiency which may cause sub-optimal result of energy saving related. Therefore, it is beneficial to add this new key issues in the study.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66

# * * * * First change * * * *

5.A	Key Issue #A: Selection of Network Function considering energy saving or energy efficiency

5.A.1	Description

Currently the criteria of the Network Function selection mainly focus on the capability (e.g. support of certain feature), Service Area (e.g., Service area of the SMF), capacity related, UE location etc., however, lack of consideration on energy consumption and/or energy efficiency which may cause sub-optimal result of network operation or service requirements fulfilment regarding energy saving and/or energy efficiency.

This key issue will study the following aspects:

-	Whether and how to enhance the NF selection considering energy saving and/or energy efficiency based on e.g. energy saving states, UE subscription, capability of NFs, energy related information provided by OAM or from other NFs.

-	Whether and how the UE assists NF selection including RAN selection for network energy saving.

-	How to handle UE registration and PDU session establishment request if no NF can be selected due to the network energy state.

# * * * * End of changes * * * *