3GPP TSG-SA2 Meeting #160	S2-2312649

# 13 - 17 November, 2023, Chicago, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1012 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinements for ML model storage to and retrieval from ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In last meeting, S2-2311618 was approved, which was to propose the following thing: when two NWDAFs interact ML model with each other, one NWDAF colud provide another NWDAF with the ML model information (i.e address of ML model) indirectly or ML model file directly. In that paper, a NOTE was also added like: NOTE X: It is up to NWDAF implementation to determine whether to include ML Model file in input parameters considering ML Model file size, etc. Based on this agreement, this paper is to clarify some similar thing for the scenario described in the clause 6.2B.7, where the MTLF may provide ML model file or ML model information to NWDAF service consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.2B.7, clarify the MTLF may provide ML model file or ML model information to NWDAF service consumer. - Add a NOTE: It is up to NWDAF containing MTLF implementation to determine whether to notify/ response to the NWDAF service consumer with ML Model file considering ML Model file size, etc. - Some other changes to service operation in the procedure according to clause 10.3 - Add ML Model identifier or Storage Transaction Identifier as an optional input for Nadrf_MLModelManagement_StorageRequest. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification not concistent and complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.7, 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 6.2B.7	ML model retrieval from ADRF

The procedure depicted in Figure 6.2B.7-1 is used by consumers (NWDAF containing MTLF and NWDAF containing AnLF) to retrieve ML models from an ADRF.

Figure 6.2B.7-1: Procedure for ML model(s) retrieval from ADRF

1.	The NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) subscribes/requests for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelInfo_Request service (if consumer is NWDAF containing AnLF) or by invoking the Nnwdaf_MLModelTraining_Subscribe service / Nnwdaf_MLModelTrainingInfo_Request (if consumer is NWDAF containing MTLF) operation.

2.	The NWDAF containing MTLF determines whether the set of ML Model(s) associated with a/an (set of) Analytics ID(s) should be retrieved from the ADRF.

When NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF directly, steps 3 and 4 is skipped.

If NWDAF containing MTLF determines that the set of ML Model(s) corresponding Analytics ID(s) requested in step 1 needs to be retrieved from ADRF and the NF consumer is agnostic to where the ML model(s) is stored, then Steps 3 and 4 is performed.

NOTE:	How NWDAF containing MTLF and ADRF authorizes the NF Consumer is in scope of SA WG3.

3.	The ADRF service consumer (NWDAF containing MTLF) requests for the ML model stored in ADRF by invoking the Nadrf_MLModelManagement_Retrieval request (Storage Transaction Identifier or one or more unique ML model Identifier(s)) service operation.

4.	The ADRF sends Nadrf_MLModelManagement_Retrieval response (address(es) e.g., URL or FQDN of Model file(s) stored in ADRF) service operation.

5.	The NWDAF containing MTLF may notify/ response to the NWDAF service consumer with the tuple Analytics ID, one or more tuples of unique ML Model identifier and ML Model Information. The ML model information may contain the ML model file address (e.g. URL or FQDN) or ADRF (Set) ID. When ADRF (Set) ID is provided, a Storage Transaction Identifier may be provided. The ADRF(Set) ID in included only when the NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF in step 2. Or the NWDAF containing MTLF may notify/ response to the NWDAF service consumer with the tuple Analytics ID, one or more tuples of unique ML Model identifier and ML Model file, in this case, the NWDAF containing MTLF shall first get the ML model file using the information indicated at step 4.

NOTE X: It is up to NWDAF containing MTLF implementation to determine whether to notify/ response to the NWDAF service consumer with ML Model file considering ML Model file size, etc.

6.	If in step 5, the NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) received ADRF (Set) ID (where the ML model(s) requested in step 1 is stored), then the NWDAF service consumer may invoke the Nadrf_MLModelManagement_RetrievalRequest (Storage Transaction Identifier or one or more unique ML Model identifier(s)) service operation to get the ML model stored in ADRF.

7.	[Conditional] The ADRF sends Nadrf_MLModelManagement_RetrievalResponse (ML Model identifier(s) and address(es) of Model file(s) stored in ADRF to the NWDAF service consumer.

* * * * Next Change * * * *

### 10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML Model(s). ML Model(s) file or ML Model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and set of:

-	ML model(s) file; or

-	the tuple (unique ML Model identifier and address (e.g. URL or FQDN) of Model file and Storage size required for each of the ML Model).

NOTE:	The ADRF can download ML Model files based on the ML Model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML Model and locally stores the ML Model is left for implementation.

Inputs, Optional:  ML Model identifier, Storage Transaction Identifier.

Outputs Required: Result Indication, one of the following:

-	[Conditional] one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

* * * * End of Change * * * *