3GPP TSG-SA WG2 Meeting #160	S2-2312662

# Chicago, US, 13 – 17 November, 2023

Title:	Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

Response to:	LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/C1-237897)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1, SA3

Cc:

Contact Person:

Name:	Fei Lu

Tel. Number:

E-mail Address:	lufei2 at OPPO.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thank CT1 for their LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/C1-237897). SA2 discussed the questions raised by CT1 and provided the following answers.

As specified in TS 33.503 clause 6.1.3.3.3.2, for 5G ProSe UE-to-UE Relay Discovery with Model B, the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set and sent between the discoverer 5G ProSe End UE and the discoveree 5G ProSe End UE via the 5G ProSe UE-to-UE Relay UE.

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

Answer: Although the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE, 5G ProSe UE-to-UE Relay UE can still identify different discoverer 5G ProSe End UEs and discoveree 5G ProSe End UEs. So SA2 does not see any issue for the 5G ProSe UE-to-UE Relay UE to send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE.

2. Actions:

To CT1 group.

ACTION: SA2 kindly asks CT1 to take the above answer into account.

To SA3 group.

ACTION: SA2 kindly asks SA3 to respond this LS if any concern is raised.

3. Date of Next TSG-SA2 Meetings:

SA2#160-AH-e			22 -29 January, 2024				Elbonia

SA2#161			26 February - 01 March, 2024			Athens, Greece