SA WG2 Meeting #SA2-160	S2-2312676

13 - 17 November, 2023, Chicago, USA

Title:	[Draft] Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

Response to:	LS to SA2 on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/ C1-237897)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:	SA3

Contact Person:

Name:	Wen Wang

E-mail Address:	wen.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	xxxx

- Overall description

SA2 thanks RAN3 for their LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/ C1-237897).

SA2 has discussed the question from CT1 and provides the following answer:

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

Answer: SA2 considers to use the Layer-2 ID of the discoverer 5G ProSe End UE to determine the discoverer 5G ProSe End UE as updated in the agreed CR attached, it requires that the Layer-2 ID of the discoverer 5G ProSe End UE is included in the UE-to-UE Relay Discovery Solicitation message to the discoveree 5G ProSe End UE and the Layer-2 ID of the discoverer 5G ProSe End UE is included in the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE, the UE-to-UE Relay takes the Layer-2 ID of the discoverer 5G ProSe End UE received from the discoveree 5G ProSe End UE as the destination Layer-2 ID for sending the UE-to-UE Relay Discovery Response message.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG-SA2 Meeting #160-Ad Hoc-e	emeeting	22-29 January 2024

3GPP TSG-SA2 Meeting #161		26 February- 01 March 2024	Athens, GR