3GPP TSG-SA WG2 Meeting #160	S2-2312677

# November 13 – 17, 2023, Chicago, USA							 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0399 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to U2U relay discovery with model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to: - 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and - 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously as pointed by CT1 LS (S2-2311951/ C1-237897), 5G ProSe UE-to-UE Relay UE seems to be unable to determine the UE-to-UE Relay Discovery Response which is to be sent to. To resolve above issue, it is proposed to use the Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) to distinguish the discoverer 5G ProSe End UE with following thinking: - In step2, the U2U relay UE addtionally includes the Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) (it can be detected in step1) in the discovery solicitiation message - In step3, the discoveree 5G ProSe End UE needs to add the recived Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) in the Discovery Response message to the U2U relay UE - In step4, the U2U relay UE sends the 5G ProSe UE-to-UE Relay Discovery Response message which destination Layer-2 ID is set to Layer-2 ID of the discoverer 5G ProSe End UE received in step3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In step2, the U2U relay UE addtionally includes the Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) (it can be detected in step1) in the discovery solicitiation message - In step3, the discoveree 5G ProSe End UE needs to add the recived Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) in the Discovery Response message to the U2U relay UE - In step4, the U2U relay UE sends the 5G ProSe UE-to-UE Relay Discovery Response message which destination Layer-2 ID is set to Layer-2 ID of the discoverer 5G ProSe End UE received in step3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The U2U relay UE can not figure out the desination of the UE-to-UE Relay Discovery Response message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

##### 6.3.2.4.3	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

Depicted in Figure 6.3.2.4.3-1 is the procedure for 5G ProSe UE-to-UE Relay Discovery with Model B.

Figure 6.3.2.4.3-1: 5G ProSe UE-to-UE Relay Discovery with Model B

1.	The discoverer 5G ProSe End UE (UE-1) sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of itself, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

2.	A 5G ProSe UE-to-UE Relay that matches the RSC sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of the discoverer 5G ProSe End UE (UE-1), User Info ID of UE-to-UE Relay, RSC, Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe End UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

3.	The discoveree 5G ProSe End UE (UE-2) that matches the RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1), Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) received from the 5G ProSe UE-to-UE Relay Discovery Solicitation message in step2 and User Info ID of discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays with the same RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2), it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

4.	The 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID as described in clause 5.8.4 and the Destination Layer-2 ID set to the Layer-2 ID of the discoverer 5G ProSe End UE (UE-1) received from the 5G ProSe UE-to-UE Relay Discovery Response message in step3.

* * * * End of Change * * * *