3GPP TSG-SA WG2 Meeting #160	S2-2312685

# Nov 13 – 17, 2023, Chicago, USA										 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0089 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Network-assisted SL Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.5.3.1, since the agreed paper S2-2311465 has adjusted the sequence of the SL-MO-LR procedure, it makes the reference of the steps not align with current SL-MO-LR prcedure for MO-LR Procedure for Network-assisted SL Positioning for UE with NAS connection. In addition, in clause 6.5.3.2, it is for MT-LR Procedure for Network-assisted SL Positioning, based on explanation for Network-assisted usage in clause 5.5.4, it can be seen that it is the UE to calculate the result, which needs to highlight the difference from the 5GC-MT-LR using SL positioning procedure as defined in clause 6.20.5 of TS 23.273 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the sequence of the step following SL-MO-LR procedure in TS23.273. - Add the difference of MT-LR Procedure for Network-assisted SL Positioning for UE with NAS connection campare to the 5GC-MT-LR using SL positioning procedure as defined in clause 6.20.5 of TS 23.273 - It is the UE performing the calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The steps refered to TS23.273 are not correct. Not clear what the difference for MT-LR Procedure for Network-assisted SL Positioning for UE with NAS connection campare to the 5GC-MT-LR using SL positioning procedure as defined in clause 6.20.5 of TS 23.273 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 6.5.3.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 6.5.3.1	MO-LR Procedure for Network-assisted SL Positioning for UE with NAS connection

The procedures defined in this clause apply to Network-assisted SL Positioning for UE with NAS connection, as defined in clause 5.5.4, when the service request is initiated by the Target UE.

If the Located UE(s) is known to the Target UE (e.g. based on Located UE discovery and selection), SL-MO-LR procedure defined in clause 6.20.1 of TS 23.273 [8] is performed with the following adaptations:

-	UE1 is the Target UE, and UE2/.../UEn is the Located UE. UE1 provides application layer ID of UE2/.../UEn as the Located UE ID in the SL-MO-LR request.

-	It is indicated in step 8 that absolute location of UE1 is required.

-	It is indicated in step 15 that Network-assisted SL Positioning is applied and optionally the selected SL Positioning Server UE, e.g. Target UE or Located UE, for result calculation.

-	In step 19, the response includes absolute location of UE1.

-	Step step 20 is skipped.

If the Located UE(s) is not known to the Target UE, 5GC-MO-LR procedure using SL positioning defined in clause 6.20.2 of TS 23.273 [8] is performed.

#### 6.5.3.2	MT-LR Procedure for Network-assisted SL Positioning for UE with NAS connection

The procedures defined in this clause apply to Network-assisted SL Positioning for UE with NAS connection, as defined in clause 5.5.4, when the service request is initiated by the AF or LCS Client.

5GC-MT-LR procedure using SL positioning defined in clause 6.20.5 of TS 23.273 [8] is performed with the following adaptations:

-	The UE performs the absolute location estimation.

* * * * End of Change * * * *