3GPP TSG-WG SA2 Meeting #160	S2-2312688

Chicago, IL, USA, Nov 13 – 17, 2023

Title:	Reply LS on Clarifications related to Network Slice Replacement feature

Response to:	LS (C3-234473) on Clarifications related to Network Slice Replacement feature from CT3

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	SA2

To:	CT3

Cc:

Contact Person:

Name:	Dongeun Suh

Tel. Number:

E-mail Address:	dongeun DOT suh AT samsung DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2312690

1. Overall Description:

SA2 thanks CT3 for their LS on clarifications related to Network Slice Replacement feature. SA2 would like to provide the following reply.

Question 1: When the original S-NSSAI is replaced by an Alternative S-NSSAI, should both the original S-NSSAI and the Alternative S-NSSAI be provided by the SMF to the PCF (and from PCF to BSF)?

SA2 reply: Yes. For the PDU Session whose S-NSSAI is replaced to the Alternative S-NSSAI, the both S-NSSAIs should be provided by the SMF (i.e., either the SMF for the existing PDU session when the PDU Session is retained or the SMF for the newly established PDU session if the PDU Session is not retained) to the PCF (and also by the PCF to the BSF), which is also clarified in S2-2312690.

Question 2: For the case where the original S-NSSAI is recovered and then the PDU session is transferred back to the original S-NSSAI, what information (if any) is expected that the SMF provides to the PCF (and from PCF to BSF)?

SA2 reply: The SMF (i.e., either the SMF for the existing PDU session when the PDU Session is retained or the SMF for the newly established PDU session if the PDU Session is not retained) provides the original S-NSSAI only to the PCF (and also by the PCF to the BSF), which is also clarified in S2-2312690.

Question 3:  In case of roaming LBO scenario, does the V-PCF need the mapping of V-PLMN S-NSSAI with H-PLMN S-NSSAI information for network slice replacement policy decision during roaming LBO scenario?

SA2 reply: SA2 understands that to support the case when the AMF querying the V-PCF to obtain the Alternative H-PLMN S-NSSAI, the V-PCF may need the mapping of V-PLMN S-NSSAI with H-PLMN S-NSSAI.

2. Actions:

To CT3 group.

ACTION: 	SA2 kindly asks CT3 group to take the above response into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2#160 Ad Hoc-e	January 22 – 29, 2024	e-meeting

TSG-SA2 Meeting#161	February 26 – March 1, 2023	Athens, Greece