3GPP TSG-SA2 Meeting #160	S2-2312760

Chicago, USA, November 13-17, 2023	(is revision of S2-2310820)

Source:	Lenovo

Title:	Key issue 1.3 on supporting UL PDU set based handling

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2/ Rel-19

Abstract of the contribution: Proposes a Key Issue on supporting UL PDU Set based handling

1	Background

According to the SP-231198[1], the study on architecture and functional enhancment for XRM Ph2 target at R19 so that XR and media service can be better supported. One of the objectives that this study attempts to address is:

WT#1.3 Enhancements to support PDU Set based QoS handling in uplink direction.

In this paper, UL PDU set based handling is discussed.

2	Discussion

For uplink PDU set based handling, RAN2 agrees that NG-RAN may configure UE to perform PDU set based discard, PDU Set Importance (PSI) based discard. Besides, NG-RAN may ask UE to perform the Delay Status Reporting (DSR), which includes the remaining delay of a PDU set and the associated data volume. In order to perform UL PDU set based handling (e.g., PDU set based discard, PSI based discard, DSR), UE’s AS layer needs to identify the PDU set information for UL packets. In Rel-18, if UL PDU Set based handling is configured by NG-RAN, the UE autonomously detects PDU sets, determines PDU Set information and performs PDU Set based handling in an implementation specific manner. In this release, it is expected that whether and how PDU Set based handling can be better supported in the uplink direction? E.g., network assistant information (e.g., Protocol Description, PDU Set QoS parameters etc.) can be considered.

An XR service may have uplink XR traffic only (e.g. camera sensing data), both UL and DL XR traffic (e.g. XR gaming), DL XR traffic only (e.g. live broadcasting). Those scenarios should be considered during the study.

3	Reference

[1] SP-231198, New SID on 5GS XRM Ph2, Nokia, Nokia Shanghai Bell (Moderator)

4 Proposal

It is proposed to update TR 23.700-70v010 for FS_XRM Ph2 as follows.

* * * * Start of Change * * * *

## 5.X	Key Issue X: < UL PDU Set based handling>

### 5.X.1	Description

NG-RAN may configure UE to perform UL PDU set based handling, e.g., PDU set based discard, PDU Set Importance (PSI) based discard, or provide Delay Status Reporting (DSR) which inlcudes the remaining delay of a PDU set and the associated data volume. In order to perform UL PDU set based handling, UE’s AS layer needs to identify the PDU set information for UL packets. In Rel-18, if UL PDU Set based handling is configured by NG-RAN, the UE autonomously detects PDU sets, determines PDU Set information and performs PDU Set based handling in an implementation specific manner. In this release, it is expected that whether and how PDU Set based handling can be better supported in the uplink direction? E.g., network assistant information (e.g., Protocol Description, PDU Set QoS parameters etc.) can be considered. Besides, an XR service may have uplink XR traffic only (e.g. camera sensing data), both UL and DL XR traffic (e.g. XR gaming), DL XR traffic only (e.g. live broadcasting). Those scenarios should be considered during the study.

The following aspects shall be studied to support UL PDU set based handling.

- Whether and which 5GC parameters need to be provided to UE in order to better support UL PDU set based handling?

- How to support the PDU Set handling with regard to a service with uplink XR traffic only, DL XR traffic only, or both UL and DL are XR traffic.

- Whether any further enhancement is needed considering RAN WG progress in uplink PDU set based QoS handling.

* * * * End of Change * * * *