3GPP TSG SA WG2 Meeting #160	S2-2312800

Chicago, US, 13-17 Nov 2023	(revision of xx-yyxxxx)

# Source:	China Telecom

# Title:	New WID: TEI19_NetFailTol; Enhancing Network Failure Tolerance

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhancing Network Failure Tolerance

Acronym: TEI19_NetFailTol

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | x | x |  |
| No | x | x |  |  | x |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MINT_Ph2 | SA1 | 970041 | Minimization of Service Interruption During Core Network Failure Phase 2 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 920062 | Stage 2 of MINT | R17 MINT enables a UE to perform Disaster Roaming to a 5G PLMN in the case of RAN failure. |

3	Justification

R17 MINT enables a UE to obtain service from a PLMN offering Disaster Roaming service when a Disaster Condition applies to the UE's determined PLMN. However, Disaster Condition only applies to NG-RAN nodes, which means the rest of the network functions except one or more NG-RAN nodes of the PLMN with Disaster Condition can be assumed to be operational. In addition, EPS does not support Disaster Roaming.

Based on SA1 requirements introduced in R19 MINT_Ph2, the application of MINT function should not be limited to RAN failure, but it should exclude any core network failure that renders the network subject to a disaster unable to authenticate its subscribers. Therefore, UDM/UDR/AUSF failure is not considered in MINT function and its enhancement.

To allow 5G-national roaming UEs to register for Disaster Roaming service on 4G of the same VPLMN, the 4G system shall be able to provide Disaster Roaming service. It covers both RAN and core network failures. In this release, the core network failure is limited to VPLMN, so that the core network of HPLMN is functional to perform user authentication.

In addition, current Disaster Condition notification and determination procedures assume RAN failure. In the case of core network failure, the procedures can be improved to minimize the interval between a UE losing service and attempting Disaster Roaming.

MINT function does not work under UDM/UDR/AUSF failure. Since the number of UEs affected by UDM/UDR/AUSF failure is huge, UE authentication and subscription data retrieval for registered UE may be skipped based on operator policy. Registered UE includes: a. UE in RM-REGISTERED state, and b. UE in RM-DEREGISTERED state with a valid 5G-GUTI and UE context. By bypassing UE authentication and subscription data retrieval, operators can provide communication services in the case of UDM/UDR/AUSF failure.

AMF and SMF, as consumers in direct communication, can be notified by NRF when UDM/UDR/AUSF fails and determine to bypass the above procedures if all UDM/UDR/AUSF in a set fail. In model D, they do not know NF status and cannot activate bypassing. A dedicated SCP error message is required in this case.

4	Objective

This new WID aims to enable the following to enhance network failure tolerance. It contains two Work Tasks: WT1 is an enhancement of R17 MINT based on SA1 requirements, and WT2 is not related to R17 MINT.

WT1.1: Support of providing Disaster Roaming service in EPS by

- Adding EPC as an allowed core network for Disaster Inbound Roamers

- Procedural updates similar to those for MINT in R17 5GC

- Introducing the MINT support in Attach/TAU procedure

- Enhancing MME to support MINT

WT1.2: Enhancing Disaster Condition notification and determination procedures in the case of core network failure

WT2: Bypassing UE authentication and subscription data retrieval in the case of UDM/UDR/AUSF failure

From the perspective of 5GS, the existing R17 MINT is unchanged. That is, for a UE registered with Disaster Roaming, interworking with EPC is forbidden. To minimize the impact to EPC, UE configuration and provisioning for Disaster Roaming is performed by its 5G HPLMN or 5G VPLMN while roaming.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.401 | Adding the support of MINT and updating the corresponding procedures. | - |  |
| TS 23.501 | - Updating clause 5.40 to indicate the support of MINT under core network failure. - Updating clause 5.40 to indicate MINT is supported in EPS. | - |  |
| TS 23.502 | - Updating clause 4.2.2 to support bypassing UE authentication in the case of UDM/UDR/AUSF failure. - Updating clause 4.2.2 and 4.3.2 to support bypassing UE subscription data retrieval in the case of UDM/UDR/AUSF failure | - |  |

6	Work item Rapporteur(s)

7	Work item leadership

SA2

8	Aspects that involve other WGs

CT1, CT4

9	Supporting Individual Members

Supporting IM name

China Telecom