3GPP TSG-SA WG2 Meeting #160	S2-2312814

# Chicago, IL, 13 - 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5166 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Number of PDU Session update for PDU Session on Alternative S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the S-NSSAI of the PDU session is replaced without re-establishment, the number of PDU session on the Alternative S-NSSAI needs to be updated (i.e., increased), if the Alternative S-NSSAI are subject to NSAC. If the maximum number of PDU session is reached on the Alternative S-SNSSAI, the AMF stops the slice replacement towards the Alternative S-NSSAI and may retry with different S-NSSAI. If the S-NSSAI of the PDU session is replaced without re-establishment, the number of PDU session on the old S-NSSAI needs to be updated (i.e., increased), if the old S-NSSAI are subject to NSAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable the SMF to trigger the procedure to update the number of PDU Sessions during the network slice replacement. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of PDU session on the Alternative S-NSSAI and/or the S-NSSAI is not updated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.15.11.2	Network Slice Admission Control for maximum number of PDU sessions

5.15.11.2.1	Non- Hierarchical NSAC architecture

The NSACF keeps track of the current number of PDU Sessions per network slice so that it can ensure it does not exceed the maximum number of PDU session allowed to be served by the network slice. When an event related to a UE causes the current number of PDU sessions established within the network slice is to increase, the NSACF checks whether the maximum number of PDU sessions per network slice for that network slice has already been reached and if it has, the NSACF applies admission control policies.

The anchor SMF triggers a request to NSACF for maximum number of PDU sessions per network slice control during PDU session establishment/release procedures in clauses 4.3.2 and 4.3.4 of TS 23.502 [3] and during the PDU session modification procedures for the network slice replacement in clauses 4.3.3 of TS 23.502 [3].

The SMF provides the Access Type to the NSACF when triggering a request to increase or decrease the number of PDU Sessions. The NSACF takes Access Type into account for increasing and decreasing the current number of PDU Sessions depending on the applicability of the Access Type for the NSAC for maximum number of PDU Sessions for the S-NSSAI.

NOTE 1:	For MA PDU Session, the SMF provides the Access Type to NSACF when the user plane connection is about to be established or released in the corresponding access network. With this, the SMF provides one or two Access Types for the MA PDU Session in the same request message to the NSACF. The NSACF can reject a single or both Access Types depending on the applicability of the Access Type for the NSAC.

NOTE 2:	I-SMF does not interact with NSCAF.

5.15.11.2.2	Hierarchical NSAC architecture

The main differences between the NSACFs deployed in a non-hierarchal architecture and NSACFs deployed in a hierarchal architecture is as follows:

-	The NSCAF is enhanced to support PDU session admission quota-based control.

-	When the local maximum number of PDU sessions is reached, the NSACF interacts with the primary NSACF to handle the request. The Primary NSACF either return an increased local maximum number to the NSACF, or reject the local maximum number value update request if all the global maximum number are consumed based on the status of established PDU sessions to the network slice.

-	Based on the response from primary NSACF, the NSACF updates the local maximum number if updated value is received from the primary NSACF. The NSACF updates local maximum number value (if received) and determines whether to accept or reject the NSAC request for PDU session establishment based on the local maximum number value.

-	The update of local maximum number value by the Primary NSACF can also happen at any time through the request/response service XYZ operation.

-	The Primary NSACF subscribes to all the NACFs to obtain the number of established PDU sessions at all NSACFs.

5.15.11.2.3	Centralized NSAC architecture

The main differences between the NSACFs deployed in a non-Hierarchical architecture and NSACFs deployed in a centralized architecture is as follows

-	If multiple NSAC service areas are deployed in PLMN, the SMF provides the NSAC service area information to the centralized NSACF. The centralized NSACF also stores the NSAC service area of the SMF the PDU session is established on.

END of CHANGES