3GPP SA WG2 Meeting #160	S2-2312848

Chicago, USA, November 13-17, 2023

Source:	Nokia, Nokia Shanghai Bell, Charter Communications, CableLabs, InterDigital Inc., ZTE

Title:	KI description for WT#3.1

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This paper proposes KI description for WT#3.1

# Discussion

The new approved SID titled Study on Architecture enhancement for XRM Ph2 (SP-231198) includes the below non-3GPP aspects to be studied as part of Rel-19:

WT#3 Further enhancement to support XR based on non-3GPP access.

WT#3.1 Study how to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.  ​

- Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF).

- Support L4S in wireline access (e.g. W-AGF).

NOTE 1: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

A Rel. 19 KI is needed for this WT. The following is proposed.

# Proposal

It is proposed to incorporate the following KI for Rel. 19 XRM_Ph2 WT#3.1 in the XRM Ph2 TR23.700-70 v0.1.0.

*** Start of changes ***

X.1	Key Issue #X: L4S for non-3GPP access networks and intermediate 5GS nodes

X.1.1	Description

As the use-cases and applications for XRM are not limited to 3GPP access, XRM devices and applications may use non-3GPP access as a means of communication. In such a scenario where the XRM device(s) use non-3GPP access path with 5GC backhaul, the intermediate nodes such as N3IWF, TNGF, W-AGF, & RGs may experience UP traffic congestion (for example, an N3IWF/TNGF/W-AGF/RG deployment supports multiple access connections and as a result may experience congestion).

L4S is a concept that allows for congestion reporting throughout the network (i.e., E2E), where any node managing traffic queues may support L4S-specific ECN marking (i.e., access points and intermediate routers such as N3IWF, TNGF, W-AGF, RGs).

The objective of this Key Issue is to extend the L4S mechanism for non-3GPP access and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes such as N3IWF, TNGF, W-AGF, RGs. The KI addresses several areas related to L4S with respect to non-3GPP access.

The following aspects should be studied:

- How L4S congestion in UL and DL directions with ECN marking is handled in non-3GPP access-specific 5GS intermediate nodes such as N3IWF, TNGF, W-AGF, RGs, etc.

- How congestion information exchange happens in UL and DL direction between the non-3GPP access intermediate nodes and 5GC, and vice versa​.

*** End of changes ***