3GPP TSG-WG SA2 Meeting #160	S2-2312856

Chicago, USA, November 13 – 17, 2023	(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5177 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provision of PDU Set QoS Parameters with Flow Direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As highlighted by LS from RAN3 in S2-2311967/R3-235890, the majority companies in RAN3 see benefits, and RAN3 kindly asks SA2 to provide the feedback on whether separating all DL and UL PDU Set QoS parameters or some of them from 5GC is necessary. Based on the discussion in R18, the QoS requirements of DL and UL SDFs of XRM service could be different with eachother. For example, the RT latency could be split into UL and DL with different values. In the related description e.g. in clauses 5.7 and 5.37 should be clarified to support the split of DL and UL PDU set QoS parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify the split of DL and UL PDU set QoS parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear that whether separating DL and UL PDU Set QoS parameters is support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.1, 5.7.7.x(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.7.7	PDU Set QoS Parameters

5.7.7.1	General

PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling.

The following PDU Set QoS Parameters are specified:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

4.	Flow Direction.

The QoS Profile may include the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile.

If the NG-RAN receives PDU Set QoS Parameters and supports them, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters as described in this clause.

Editor's note:	[XRM] The applicability and details of supporting PDU Set handling in uplink direction may be updated based on RAN WG's progress.

# * * * * the next change * * * *

5.7.7.x	Flow Direction

The Flow Direction indicates the applicability of the PDU Set QoS Parameters including PSDR, PSER and the PSIHI: UL, or DL, or both UL and DL for the associated QoS flow. Flow Direction is an optional parameter.

If different values of the PDU set QoS Parameter (e.g. PSDB) are provided for the SDF, then one is for uplink direction, while the other one is for downlink direction. The Flow Direction is provided together to identify the direction of the parameter.

A QoS Flow is associated with only one value of each PDU Set QoS Parameter.

# * * * * End of changes * * * *