# 3GPP TSG-SA2 Meeting #160		S2-2312910

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023	(revision of xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0091 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update about SL-PRS priority and SL-PRS delay budget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the LS S2-2311978(R2-2311389), RAN2 WG has confirmed that Delay Budget for SL-PRS should follow similar principles of PDB for radio resource allocation. Based on the LS S2-2311937(R2-2309343), RAN2 WG has agreed that “The SL-PRS priority can be provided by the UE’s own high layer when it triggers the SL-PRS transmission.” So it is proposed to let the Ranging/SL Positioning layer derive the Delay Budget for SL-PRS and the SL-PRS priority from the Ranging/SL Positioning QoS requirements and provides it to the AS layer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the related description about how to get the SL-PRS related parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not aligned with the RAN1 and RAN2 WGs’ agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.7.2	Handling of Ranging/SL Positioning QoS

Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request generated at the application layer, and is provided from the application layer to the Ranging/SL Positioning layer. Ranging/SL Positioning QoS requirement may be included in the Ranging/SL Positioning Service request from SL Positioning Client UE.

The Ranging/SL Positioning layer maps the Ranging/SL Positioning QoS requirement to the Ranging/SL Positioning QoS parameters and provides the Ranging/SL Positioning QoS parameters to the AS layer. Specially, the Ranging/SL Positioning layer will derive the SL-PRS priority and Delay Budget for SL-PRS based on the Ranging/SL Positioning QoS requirement (e.g. LCS QoS Class, Accuracy, Response Time) and provides them to the AS layer for radio resource allocation. If there is no received Ranging/SL Positioning QoS requirement from the application layer, the Ranging/SL Positioning layer determines the Ranging/SL Positioning QoS parameters based on the Ranging/SL positioning Policy/parameters as configured in the clause of 5.1.

Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions:

-	The accuracy attribute also includes

-	the relative horizontal accuracy, and the relative vertical accuracy for relative positioning;

-	the distance accuracy and direction accuracy for Ranging.

-	Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5.

Ranging/SL Positioning QoS information is used for determination of Ranging/SL Positioning method. The Ranging/SL Positioning methods are defined in TS 38.355 [12].

# * * * * End of changes * * * *