3GPP TSG-WG SA2 Meeting #160 	S2-2312942

13 - 17 November, 2023, Chicago, USA	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI#3, New sol: Solution of Data Channel Interworking

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract: This paper proposes to provide a solution for KI#3 of FS_NG_RTC_Ph2.

1. Introduction/Discussion

The Key Issuse#3 of SA2 R19 FS_NG_RTC_Ph2 addresses the following aspect:

- how to enhance IMS architecture and procedures to provide some form of IMS data channel applications service experience by data channel interworking with MTSI UE supporting only legacy media types where it is appropriate.

This document provides a solution to address above aspect of KI#3.

3. Text Proposal

It is proposed to capture the following changes into TR 23.700-77.

# * * * * First change * * * *

6.X Solution #X:Data Channel Interworking mechanism between DCMTSI UE and MTSI UE

6.X.1	Description

This solution addresses Key Issue#3 "Data channel interworking with MTSI UE" to support data channel interworking mechanism between DCMTSI UE and MTSI UE, which covers:

- how to enhance IMS architecture and procedures to provide some form of IMS data channel applications service experience by data channel interworking with MTSI UE supporting only legacy media types where it is appropriate.

Some data channel use cases do not require interaction from both ends. For example, in Real-time Sharing Screen use case specified in 3GPP TR 22.873, only the sharing party shares its mobile screen to other parties through data channel supported by its DCMTSI client in terminal, and the other parties only need to receive the sharing screen media stream, which could be transferred over video media streams supported by all the MTSI client in terminals. Hence, if these data channel services can be supported between a DCMTSI client in terminal and an MTSI client in terminal, it would speed up the commercial deployment process of IMS data channel.

This solution proposes a method to achieve the one-sided interactive communication between a DCMTSI client in terminal and an MTSI client in terminal.

6.X.2	Procedures

6.X.2.1	Interworking between a DCMTSI client in terminal and an MTSI client in terminal

The procedures described in this clause is applied to some data channel use cases in which it’s a video session and only unidirectional interaction is needed between both UEs.

For example, in Real-time Sharing Screen use case, only UE A supports IMS data channel and it establishes application data channels with the DCMF in its local network and transfers its mobile screen data, including RTP stream and RTCP stream, over the application data channels. The DCMF decapsulates RTP data and RTCP data from the data channels, encapsulates and sends these data through video media streams to UE B. Figure 6.X.2.1-1 depicts the workflow below.

Figure 6.X.2.1-1: Interworking between a DCMTSI client in terminal and an MTSI client in terminal

Figure 6.X.2.1-2 demonstrates the procedure of UE-initiated bootstrap data channel establishment between network A and UE A (a DCMTSI client in terminal), while an IMS video call is established between UE A and UE B (an MTSI client in terminal).

Figure 6.X.2.1-2: Bootstrap data channel establishment procedure for interworking between a DCMTSI client in terminal and an MTSI client in terminal

1-8. UE A completes media negotiation with local network through INVITE/18X/200 procedure and the dedicated bearer/QCI Flow of data channel is created between UE A and its local network. Based the updated DC media description information reported by the MMTel AS, in which the port of bootstrap data channel between the DCSF A and UE B is set zero, the DCSF A knows that UE B or network B does not support IMS data channel.

9-11. After the precondition procedure is completed, UE A initiates SCTP association and DTLS connection establishment procedures and establishes a bootstrap data channel with the DCMF A. Then UE A retrieves the bootstrap application and the data channel application list from the DCSF A, and download Real-time Screen Sharing application from the DCSF A. At the same time, the IMS video session is connected and video stream is transferred between UE A and UE B.

After UE A retrieves Real-time Screen Sharing application from the DCSF A and run the application, the following procedure shown in Figure 6.X.2.1-3 is performed to transfer UE A’s mobile screen data to UE B.

Figure 6.X.2.1-3: Application channel establishment procedure for interworking between a DCMTSI client in terminal and an MTSI client in terminal

1.  UE A initiates a re-INVITE request to negotiate application data channel media with the local network, in which three application data channels (for instance,stream 1000 and 1001) are contained. Stream 1000 is used to transfer video RTP stream of screen shots sequence and stream 1001 is used to transfer RTCP stream.

2. The MMTel AS A reports the media event of data channel the DCSF A.

3. Based on the initial SDP O/A media negotiation, the DCSF A knows that UE B or network B could not support IMS data channel. In addition to the App info carried in the report from the MMTel AS A, the DCSF A could decide that it need to start one-side data channel procedure in this IMS session. If the one-side data channel procedure is not applied to this App, the DCSF A should indicate the MMTel AS A to reject this re-INVITE request.

4. The DCSF A send a Data Channel Control Request to the MMTel AS A and indicates it to create two application data channels (stream 1000 and 1001).

5. The MMTel AS A send a Media Resource Management Request to the DCMF to allocate resource for two application data channels (stream 1000 and 1001).

6. The DCSF A sends a Call Control Request to the MMTel AS A and requires it to anchor the video media streams of UE B to the DCMF A.

7. The MMTel AS A sends a Media Resource Management Request to the DCMF A to allocate media resource for anchoring the video streams of UE B.

8-10. The MMTel AS A sends a re-INVITE request with the SDP offer for anchoring the video media of UE B to the DCMF A. UE B returns 200 OK with its SDP answer. Then the MMTel AS A sends ACK to UE B.

11-12. The MMTel AS A sends 200 OK for re-INVITE initiated by UE A and UE A returns ACK.

13. UE A initiates SCTP association and DTLS connection establishment procedures and establishes two application data channels (stream 1000 and 1001) with the DCMF A.

14. The MMTel AS A reports a Media Event to the DCSF A, and reports that anchoring video stream is successful.

15. The DCSF A sends a Call Control Request to the MMTel AS A indicating it to continue the call process.

16. The MMTel AS A sends a Media Resource Management Request to the DCMF A to associate application data channel stream 1001 and 1002 between UE A and the DCMF A with the allocated video streams between UE B and the DCMF A.

Then the application data channels (stream 1001 and 1002) between the DCMF A and UE A are associated with the RTP and RTCP video streams between the DCMF A and UE B. The RTP and RTCP stream of UE A’s screenshot sequence is received by UE B from video streams.

6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

# * * * * End of changes * * * *