3GPP TSG-WG SA2 Meeting #160	S2-2312961

Chicago, US, Nov 13 – 17, 2023	(revision of S2-231xxxx)

Source:	Huawei, HiSilicon

Title:	KI#X: New Sol: Support of dynamic change in traffic characteristics

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: This paper intends to propose a high-level solution to support the dynamic changes in traffic characteristics.

# 1. Introduction

In the approved FS_XRM_Ph2, one objective is to study whether and how to support the dynamic changes in traffic characteristics as following:

WT#2: QoS handling enhancement for XRM services

- WT#2.2: Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN.

This paper intends to propose a high-level solution to support the dynamic changes in traffic characteristics, including the pre-authorization of the QoS policies, detection of the changed traffic characteristics and the adaptive QoS adjustments to the dynamic changes in traffic characteristics.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change, all new texts * * * *

## 6.X	Solution #X: Support of dynamic change of traffic characteristics

### 6.X.1	Key Issue mapping

The solution addresses Key Issue #Y: Fast QoS Handling for support of dynamic changes on traffic characteristics.

### 6.X.2	Description

For XR or other interactive media services, the application layer’s network requirements could be quite dynamic. For example, the size of data burst in XRM services could vary in a wide range. To ensure the occasionally big bursts can be transferred within PDB/PSDB, currently the QoS parameters (e.g. GFBR or MDBV) need to be set according to the potential maximum burst value in current QoS mechanism. This overprovisioning leads to potential waste of network resource and lower user capacity.

In this solution, a fast QoS adaptation mechanism is proposed to support the dynamic changes in traffic characteristics:

-	The AF provisions the dynamic QoS Requirements for a target media flow, e.g. the MDBV range requirements for the data burst.

-	The PCF authorizes the QoS parameter range for the service data flow in the PCC rule based on the AF input and/or local operator configuration.

-	Based on the PCC rule from the PCF, the SMF generates and provides the QoS profiles with authorized QoS parameter range(s) (i.e. the maximum MDBV) to the NG-RAN, together with an indication to enable the fast QoS adaptation based on the in-band notification from UPF.

-	Similarly, the SMF instructs the UPF to detect the dynamic changes in the traffic characteristics for the target service data flow and to further notify NG-RAN on the changed traffic characteristics.

-	The UPF detects the dynamic changes in the traffic characteristics for the target service data flow. The UPF can identify the data burst size for the incoming data burst and add the data burst size for the DL packets of the data burst to notify NG-RAN on the size of the incoming data burst.

Editor’s Note: How UPF identifies the data burst size is FFS.

-	Receiving the DL packet within the notification from UPF, the NG-RAN adaptively adjusts the QoS parameters based on the notification and authorized QoS parameter range from SMF.

-	For example, once the data size of the incoming data burst is known, the NG-RAN selects a proper QoS parameter, i.e. a proper MDBV value within the authorized QoS parameter range. The MDBV may be increased to make sure the data burst can be efficiently transmitted in time or be decreased to leave more wireless network resources for other usages.

Figure 6.43.2-1: dynamic traffic characteristic change

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

# * * * * End of changes * * * *