3GPP TSG-WG SA2 Meeting #160	S2-2312982

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4647 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on multi-member AF session with required QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - During the procedure for revoking a Multi-member AF session with required QoS, if the session involves subscription to QoS monitoring events, the NEF should also invoke the Npcf_PolicyAuthorization_Unsubscribe towards the PCF. Clause 4.15.6.13.2 "Procedures for Creating a Multi-member AF session with required QoS" contains description for revoking the Multi-member AF session with required QoS, which is redundant and should be moved, i.e. (to be) covered in clause 4.15.6.13.4. - Regarding mapping of requests for Multi-member AF session with required QoS service to the corresponding Npcf_PolicyAuthorization request, the Npcf_PolicyAuthorization_Notify request is actually from the PCF and based on that the NEF sends the Nnef_AFsessionWithQoS_Notify request to the AF. - Current specification on the NEF optionally consolidating the outcome of the individual requests based on a configured timer is obscure and needs improvement. - Editorial modifications on description of QoS Monitoring, not involving the TSCTSF and etc. for the Multi-member AF session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add that NEF may invoke Npcf_PolicyAuthorization_Unsubscribe when revoking a Multi-member AF session with required QoS. - Clarify the mapping of Nnef_AFsessionWithQoS_Notify request with Npcf_PolicyAuthorization_Notify request. - Clarify the handling of NEF regarding optionally consolidating the outcome of the individual requests. - Editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear NEF behaviors for creating/revoking the Multi-member AF session with required QoS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.1, 4.15.6.13.2, 4.15.6.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

4.15.6.13.1	General Descriptions

This clause describes the procedure to request QoS and to perform QoS monitoring for the traffic flows for the communication between an AF and a set of UEs, identified by the list of UE address(es). For every UE in the set, this list contains the IP address and the port number that is used by the UE for the communication with the AF.

The NEF receives the request for a Multi-member AF session with required QoS for a set of UEs identified by their addresses. The NEF then maps the request for Multi-member AF session with required QoS to individual requests for AF session with required QoS (i.e., one request for individual AF session with required QoS per UE address) and interacts with each of the UE's serving PCFs on a per AF session basis. The interaction follows the AF session with required QoS procedure as described in clauses 4.15.6.6 and 4.15.6.6a, except that the involvement of the TSCTSF and the provisioning of TSCTSF related information are not supported.

When the NEF receives the outcome of the individual requests for AF session with required QoS corresponding to each UE's IP address, the NEF may consolidate them into a single response before forwarding it to the AF, e.g. based on alocally configured timer.

NOTE 1:	The consolidation of the outcome of the individual requests and the locally configured timer allow the optimization of the NEF to AF signalling according to the specific Multi-member AF session with required QoS. Multiple responses could be sent by an NEF. The details of the NEF behaviour (e.g. handling of UE addresses for which no response has been received within the locally configured timer) are to be defined by stage 3.

The AF can subscribe to QoS Monitoring (as described in clause 5.45 of TS 23.501 [2]) for the Multi-member AF session with required QoS. If so, QoS monitoring will be activated by the NEF for the whole set of UEs by interacting with each of the UE's serving PCFs on a per AF session basis. If the AF request contains QoS monitoring information without an indication of direct event notification, the NEF shall include that indication in the request to ensure that QoS Monitoring reports shall be sent by the UPF directly to the NEF. The NEF forwards the QoS Monitoring reports to the AF together with the respective UE address individually or, optionally, in an aggregated manner e.g. based on a locally configured timer.

When the AF subscribes to QoS Monitoring of UL and/or DL data rate (as described in clause 5.45 of TS 23.501 [2]) for the set of UEs, the AF may provide a Consolidated Data Rate threshold that is to be stored in the NEF. The Consolidated Data Rate threshold defines the upper bound of the aggregated data rate across all traffic flows corresponding to the list of UE addresses of the Multi-member AF session with required QoS. The AF may provide in addition a specific list of UE addresses subject to Consolidated Data Rate monitoring (which has to be the subset of the list of UE addresses for the Multi-member AF session), if only a part of the UEs participate in the current communication with the AF and the NEF maintains this list as well. The NEF aggregates the QoS Monitoring reports for data rate  for those UEs identified by the specific list of UE addresses subject to Consolidated Data Rate monitoring (if available) or otherwise by the list of UE addresses for the Multi-member AF session. The QoS Monitoring reports for data rate will then be sent to the AF by NEF, only if the aggregated data rate exceeds the Consolidated Data Rate threshold.

The following Table 4.15.6.13.1-1 describes the 4 types of requests for the Multi-member AF session with required QoS by the NEF and the corresponding requests to/from PCF using Npcf_PolicyAuthorization service. There may be different PCFs serving the PDU Sessions associated with the UE addresses in the Nnef_AFSessionWithQoS Request.

Table 4.15.6.13.1-1: Mapping of requests for Multi-member AF session with required QoS service to the corresponding Npcf_PolicyAuthorization service operations

| Nnef_AFSessionWithQoS Request | Npcf_PolicyAuthorization request |
| --- | --- |
| Create | Npcf_PolicyAuthorization_Create request for each UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization_Subscribe to subscribe to QoS Monitoring reports for each UE address targeted for QoS monitoring received in the Nnef_AFSessionWithQoS Request. |
| Update | Npcf_PolicyAuthorization_Update request to update the QoS and/or QoS monitoring according to the Nnef_AFSessionWithQoS_Update Request. Npcf_PolicyAuthorization_Create request for each added UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization_Delete request for each removed UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization_Subscribe to subscribe to QoS Monitoring reports for each new UE address targeted for QoS monitoring received in the Nnef_AFSessionWithQoS Request. More than one of the above operations can be requested at the same time. |
| Revoke | Npcf_PolicyAuthorization_Delete request for each UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization_Unsubscribe to unsubscribe to QoS Monitoring reports for each UE address targeted for QoS monitoring received in the Nnef_AFSessionWithQoS Request. |
| Notify | Npcf_PolicyAuthorization_Notify to report the events that NEF subscribed to. |

NOTE 2:	It is expected that the AF requests QoS and QoS monitoring for a specific traffic flow (used for the communication between a UE address and the AF) with either the procedure for Multi-member AF session with required QoS (described in this clause) or the procedure for AF session with required QoS as described in clause 4.15.6.6 and 4.15.6.6a.

* * * Next Change * * * *

4.15.6.13.2	Procedures for Creating a Multi-member AF session with required QoS

Figure 4.15.6.13.2-1: Procedures for creating a Multi-member AF session with required QoS

1.	The AF sends a request to reserve resources for the traffic flows for the communication between a set of UEs and an AF, using Nnef_AFsessionWithQoS_Create request message (a list of UE addresses, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20].

The AF may also provide the Consolidated Data Rate threshold and optionally, a list of UE addresses subject to Consolidated Data Rate monitoring. If so, the AF shall also subscribe to QoS Monitoring of UL and/or DL data rate described in clause 5.45 of TS 23.501 [2] .

NOTE 1:	When the Consolidated Data Rate threshold is provided, it, by default, applies to the list of UE addresses associated with the Multi-member AF session with required QoS. However, if the specific list of UE addresses subject to Consolidated Data Rate monitoring is also provided together with the Consolidate Data Rate threshold, then such list has to be the subset of the list of UE addresses.

2.	The NEF authorizes the AF request that contains a list of UE addresses and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 8) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF assigns a Transaction Reference ID to the Nnef_AFsessionWithQoS_Create request. The NEF activates Consolidated Data Rate monitoring only when both the Consolidated Data Rate threshold and a request to do QoS Monitoring of data rate are provided by the AF.

2a.	If the NEF recognizes, based on configuration, that the IP address(es) received in the list of UE addresses are different from the IP address(es) assigned by 5GC (i.e. the UE(s) are behind a NAT in UPFs), the NEF performs steps 3 to 6 of the AF specific UE ID retrieval procedure defined in clause 4.15.10 for each UE IP address with port number in order to identify the corresponding IP address (and IP domain, if necessary) that has been assigned by the 5GC. The NEF then uses the respective corresponding IP address (and IP domain, if necessary) in the following steps instead of the UE IP address provided by the AF.

3-4.	The NEF finds BSF serving the UE IP address(es) using NRF and then for each UE IP address, the NEF uses Nbsf_Management_Discovery service operation, providing the UE IP address, to discover the responsible PCF for each of the PDU Sessions.

Steps 5-7 apply for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request. If the request contains QoS monitoring information without an indication of direct event notification, the NEF shall include that indication in the request to ensure that QoS Monitoring reports shall be sent by the UPF directly to the NEF.

6.	Step 4 in Figure 4.15.6.6-1 applies. The PCF generates authorized QoS Monitoring policy according to the QoS Monitoring information if received from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.8.2.18 of TS 23.501 [2].

7.	The PCF sends the Npcf_PolicyAuthorization_Create response message to the NEF.

8.	The NEF aggregates the authorization responses from the PCFs and sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result for list of UE addresses) with the aggregated authorization responses to the AF. Result for list of UE addresses includes whether the authorization was successful or has failed for every UE address in the list. The NEF stores the list of UE addresses for which the authorization was successful together with the Transaction Reference ID, the QoS and the QoS monitoring information.

Steps 9-10 apply for each UE address in the list of UE addresses.

9.	Step 6 in Figure 4.15.6.6-1 applies.

10.	Step 7 in Figure 4.15.6.6-1 applies.

11.	The NEF aggregates the notifications from the PCFs and sends a Nnef_AFsessionWithQoS_Notify message (Transaction Reference ID, Result for list of UE addresses) with the aggregated resource allocation status events to the AF. Result for list of UE addresses includes, for every UE address in the list, the information whether resources are allocated, resources are not allocated or resources are allocated while the currently fulfilled QoS matches an Alternative Service Requirement. The NEF updates the locally stored list of UE addresses by removing any UEs for which resources could not be allocated.

NOTE 2:	For those UE address(es) that did not get any resources, the AF may request resource reservation again, by adding them to the list of UE address(es) as described in clause 4.15.6.13.3.

12.	As direct event notification is requested based on the parameters received in step 5, the QoS Monitoring events are reported by the UPF(s) to the NEF using Nupf_EventExposure service as described in clause 5.2.26.2.

13.	When the NEF receives QoS Monitoring events, the NEF sends Nnef_AFsessionWithQoS_Notify message with the individual or aggregated QoS Monitoring events to the AF as described for the QoS Monitoring and the Consolidated Data Rate monitoring in clause 4.15.6.13.1.

* * * Next Change * * * *

4.15.6.13.4	Procedure for Revoking a Multi-member AF session with required QoS

Figure 4.15.6.13.4-1: Procedure for revoking a Multi-member AF session with required QoS

1.	The AF sends a request to revoke the allocated resources for the traffic flows for the communication between a set of UEs and an AF, using Nnef_AFsessionWithQoS_Revoke request message (Transaction Reference ID).

2.	The NEF authorizes the AF request.

Steps 3-5 apply for each UE address in the list of UE addresses associated with the Transaction Reference ID:

3.	The NEF sends the Npcf_PolicyAuthorization_Delete request as described in clause 5.2.5.3.4 to the PCF. If the NEF has subscribed to notifications (e.g. on resource allocation status, QoS Monitoring of UL and/or DL data rate) for the UE address in the Multi-member AF session, the NEF also sends the Npcf_PolicyAuthorization_Unsubscribe request as described in clause 5.2.5.3.7 to the PCF.

4.	The PCF proceeds with the SM Policy Association Modifications procedure as defined in clause 4.16.5.2.

5.	The PCF sends the Npcf_PolicyAuthorization_Delete response message and Npcf_PolicyAuthorization_Unsubscribe response message (if applicable) to the NEF.

6.	The NEF sends the Nnef_AFsessionWithQoS_Revoke response message (Transaction Reference ID) to the AF.

* * * End of Changes * * * *