3GPP SA WG2 Meeting #160	S2-2313033

# 13-17 November 2023, Chicago, IL, USA	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0348 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the text in EPS interworking procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS, TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current text in inter-system mobility procedure to EPS without N26 says that the the SMF/PGW-C removes the UE from the Multicast MBS session context(s), if it exists, upon receiving a tracking area update from the UE. Nevertheless the SMF/PGW-C does not receive Tracking Area Update from the UE. There are also various options in this scenario that N26 is not supported: UE can support single or dual registration mode and target EPS can or cannot support IP address continuity. It is therefore better to refer to the whole procedure completion and not specific message. Also the text in clause 4.11.2.4.1 of TS 23.502 that is referred in previous paragraph is for the optional network functionality of dual registration mode but mobility without N26 can also apply in single registration mode therefore the right reference is 4.11.2.2. The UE is also not implicitly detach from MME, the SMF+PGW-C takes care of removing the UEs PDU sessions that are transferred to EPS (see step 14 in clause 4.11.2.2 of TS 23.502). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removes text implying that SMF/PGW-C receiving Tracking Area Update from the UE - clarifies text related to mobility without N26 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification may cause confusion to stage-3 and unecessary impacts in CN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 7.4.1	Inter-system mobility with interworking at service layer

For inter-system mobility with interworking at service layer, i.e. the same multicast service is provided via eMBMS in E-UTRAN and MBS, the UE is instructed to switch between MBS and eMBMS:

NOTE 1:	The source network can be configured with the frequencies where the same service is provided in the target network, i.e. the EPC network neighbouring the 5GC network support the same eMBMS service.

NOTE 2:	The UE can be configured, per TMGI, to know that the same service is provided over MBS and eMBMS.

-	Mobility from MBS to eMBMS.

When moving to E-UTRAN/EPC, the UE initiates procedures as defined in TS 23.246 [8] to receive MBMS service for the TMGI(s).

If the UE has one or more unicast PDU Sessions (including, but not limited to, the PDU Session used for MBS and for another service (e.g. Public Safety service) with the QoS Flow(s) for the other service) moving to EPS, and if the handover procedure from 5GS to EPS using N26 interface described in clause 4.11.1.2.1 of TS 23.502 [6] is used:

-	For the PDU Session used also for MBS, the SMF+PGW-C removes the UE from the Multicast MBS session context(s), if it exists, upon receiving a Modify Bearer Request of the PDU Session from the SGW (i.e. step 14a of clause 4.11.1.2.1 of TS 23.502 [6]).

-	The NG-RAN removes the UE from the Multicast MBS session context(s) if it exists, or removes the whole multicast session context if the UE is the last one for the Multicast MBS session (e.g. after receiving the UE Context Release Command message sent by the AMF).

For 5GS to EPS Idle mode mobility without N26, when the UE moves to the EPS and may perform E-UTRAN/EPS attach according to step 5 of clause 4.11.2.2 of TS 23.502 [6].  As part of the procedure in clause 4.11.2.2 of TS 23.502 [6], the SMF+PGW-C initiates release of the PDU Session(s) in 5GS transferred to EPS and removes the UE from the Multicast MBS session context(s), if they exist.

-	Mobility from eMBMS to MBS.

When the UE has moved to NR/5GC, it triggers the multicast context and multicast flow setup/modification via PDU Session Modification procedures as defined in clause 6.8 to receive MBS transport for the TMGI(s).

# >>>>END OF CHANGES<<<<