3GPP TSG-SA WG2 Meeting #160	S2-2313039

Chicago, USA, 13 – 17 November 2023

Title:	Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

Response to:	LS on 5G ProSe UE-to-UE relay discovery with security aspects
(S2-2311951)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	Ericsson (to be SA2)

To:	CT1

Cc:	SA3

Contact person:	Shabnam Sultana (shabnam.sultana@ericsson.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2313040

# 1	Overall description

SA2 would like to thank CT1 on the LS on 5G ProSe UE-to-UE relay discovery with security aspects (C1-237897).

SA2 has discussed the questions and would like to provide the following answer.

SA2 has agreed to add an additional parameter, Discoverer Layer-2 ID, to the parameters sent in the solicitation message from the 5G ProSe UE-to-UE Relay to the Discoveree 5G ProSe End UE and in the response message sent from the Discoveree 5G ProSe End UE to the 5G ProSe UE-to-UE Relay.

The Discoverer Layer-2 ID is the Source Layer-2 ID of the solicitation message received from the Discoverer 5G ProSe End UE. By adding the proposed parameter, the 5G ProSe UE-to-UE Relay will know which originating 5G ProSe End UE to forward the received response message to.

# 2	Actions

To CT1, SA3 groups.

ACTION: 	SA2 kindly asks CT1 and SA3 to take the above answers into account.

# 3	Dates of next SA2 Meetings

SA2#160-Ad Hoc-e	22nd – 29th January 2024 		Emeeting

SA2#161	26th February – 1st March 2024 		Athens