SA WG2 Meeting #S2-160	S2-2313048

13 - 17 November, 2023, Chicago, USA	revision of S2-23xxxxx

Source:	Nokia, Nokia Shanghai Bell

Title:	New Key Issue on Support of IMS Avatar Communication

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new KI for IMS based Avatar communication.

# Introduction

The NG_RTC_Ph2 SID (SP-231196) includes the following objective:

WT-5: Study whether and how to enhance IMS architecture, procedures, interfaces for supporting avatar call (including multi-party communication) and communication with accessibility. This includes service/capability negotiation, enabling transition and transcoding between video and avatar media and avatar representation in the UE and in the IMS network, considering UE capability, network condition, and user preference.

The corresponding KI is missing in TR 23.700-77.

# 2	Proposal

It is proposed to add the following KI to TR 23.700-77.

# * * * * Begin of changes* * * *

## 5.X	Key Issue #X: Support of IMS Avatar Communication

### 5.X.1	Description

This key issue aims to study enhancements of IMS architecture, interfaces, and procedures to support IMS based Avatar communication. This includes studying following aspects:

-	Define what Avatar communication in the context of IMS means.

-	Study which identifiers are used in the context of IMS Avatar communication, e.g., how to identify an -Avatar representation in IMS.

-	Study whether and how Avatar objects such as an Avatar representation are stored and accessed by the UE and/or IMS network nodes.

-	Study whether and to enable service/capability negotiation between UE and IMS network.

-	Study how to enable transition and transcoding between a MMTel session using audio/video codec and IMS Avatar based communication which may use a special Avatar codec.

-	Study how to enable UE based and network based rendering in case of IMS Avatar communication.

NOTE 1:	Transition, transcoding and rendering and is based on UE/network capabilities and user preferences.

NOTE 2:	Transition, transcoding, rendering and service/capability negotiation aspects require coordination with SA4.

NOTE 3:	Security aspects, e.g., selecting and using an Avatar representation in an IMS session, require coordination with SA3.

# * * * * End of changes * * * *