SA WG2 Meeting #S2-160	S2-2313052

13 - 17 November, 2023, Chicago, USA	revision of S2-23xxxxx

Source:	Nokia, Nokia Shanghai Bell

Title:	New KI on support of standalone data channels session

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new KI to support standalone data channel sessions.

# Introduction

The NG_RTC_Ph2 SID (SP-231196) includes the following objective:

-	WT-4: Enhancements to IMS data channel related services and operational aspects.

-	WT-4.1: how to support standalone IMS data channel without accompanying audio/video/messaging media in an IMS session.

-	WT-4.2: how to support 3GPP PS Data Off for IMS data channel and applications over IMS data channel.

-	WT-4.3: study enhancements of IMS DC architecture and procedure to support multiplexing a SCTP connection for multiple DC applications. Alignment with SA4 is required.

The corresponding KI to WT-4.1 is missing in TR 23.700-77.

# 2	Proposal

It is proposed to add a new KI in TR 23.700-77.

# * * * * First change* * * *

## 5.X	Key Issue #X: Support of Standalone IMS Data Channel Sessions

### 5.X.1	Description

This key issue aims to study enhancements of IMS architecture, interfaces, and procedures to support standalone IMS data channel session. This includes studying following aspects:

-	How to support standalone IMS data channel sessions without accompanying audio/video/messaging media in an IMS session. This includes studying the establishment and termination of standalone IMS data channel sessions.

-	How to add audio/video/messaging media to a standalone IMS data channel session.

-	How to remove audio/video/messaging media from an IMS session that also contains IMS data channel media.

-	Whether and how to define new service data in the HSS for support of standalone data channel or extend existing MMTel service data.

# * * * * End of changes * * * *