SA WG2 Meeting #S2-160	S2-2313054

13 - 17 November, 2023, Chicago, USA	revision of S2-23xxxxx

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2: New solution on IMS capability exposure via DC3/DC4

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new solution to KI#2.

# Introduction

This contribution proposes a solution to KI#2 on “Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session” including architectural aspects.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | 2 |  |  |
| 6.X |  | X |  |  |

# * * * * Next change* * * *

## 6.X	Solution #y: Solution for IMS capability exposure via DC3/DC4

### 6.X.1	Description

The architecture to enable IMS capability exposure in the context of IMS data channel (DC) sessions is based on Figure AC.2.1-1 in TS 23.228 and is depicted in Figure 6.X.1-1.

Figure 6.X.1-1: IMS capability exposure architecture

This solution defines the exposure of IMS capabilities related to DC session via DC3/DC4 which were out of scope of Rel-18.

With this solution capabilities can be exposed via DC3/DC4 interfaces (see Figure 6.X.1-1) from DCSF to DCAS via Ndcsf services as follows:

-	establishing, updating, terminating of P2A/A2P, P2A2P, P2P application data channels,

-	downloading of an application via an existing bootstrap DC, and

-	subscription and notification to DC related events.

Editor's note:	Need for additional capabilities is FFS.

One Ndcsf service can provide event subscription/notification capabilities (e.g., Ndcsf_dc_event_exposure) while another one exposes capabilities related to DC operation (e.g., Ndcsf_dc_operation) including establishing, termination, update of application data channels.

Editor's note:	Exact definition of the Ndcsf services is FFS.

DCAS provides following information to invoke a service on DC3/DC4:

-	There are two types of actions which can be provided:

1.	First type of action is related to DC establishment, DC update, and DC termination. Following information can be provided:

  - -	Type of DC, which can be P2A/A2P, P2A2P, and P2P,

  - -	DC application binding information,

  - -	User ID which includes Calling ID and/or Called ID, and

-	External DC identifier to determine a DC that is established, updated, or terminated; and

2.	Second type of action is related to DC event subscription and notification. Following information can be provided:

-	subscription to event for application DC establishment/termination,

-	subscription to event for downloaded DC application, and

-	subscription to IMS capability exposure.

-	Event can be application DC is established or terminated, specific DC application was downloaded via bootstrap DC.

-	Unique external DC identifier to determine a DC that is established, updated, or terminated.

Editor's note:	Further information provided from DCAS to DCSF is FFS.

Editor's note:	Whether DCAS can request establishing, update, termination of bootstrap data channel is FFS.

DCSF can return to DCAS the following information on DC3/DC4:

-	response code (success or failure),

-	other information, e.g., session identifier, IP address of the UE, location, MDC2 identifier, PVNI, and external DC identifier, Call ID which can be used to determine a DC uniquely, and

-	notifications to the reported events (DC establishment/termination, downloaded DC application, IMS capability exposure).

Editor's note:	Need for additional capabilities is FFS.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.3	Impacts on existing nodes and functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

Impacted nodes and functionality are listed as follows:

-	DC Signalling Function (DCSF): The DCSF interacts with the DCAS forexposes DC resource control relatedrelated capabilities to IMS capability exposure via DC3/DC4.

-	Network Exposure Function (NEF): It shall securely expose IMS capability and events provided by DCSF to DCASexposure and exposes new services to external AF.

-	DC Application Server (DCAS): An application server that interacts with the NEF or DCSF (or NEF) for IMS capability exposure using DC3/DC4 interfaces. DCAS acts as AF.

# * * * * End of changes * * * *