3GPP TSG-WG SA2 Meeting #160	S2-2313090

Chicago, US, Nov 13 – 17, 2023	(revision of S2-231xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on the evaluation of carbon emission of ICT network

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys/ Rel-19

Abstract: The papers discuss the factors of evaluating the carbon emissions and associates the proposal in the other documents for evaluating the carbon emissions.

# 1. Introduction

The KI#1 currently includes the issue on how and what energy related information is obtained, in case that this will be addressed in a different KI, the fact that this is an essential issue to be studied.

How and what network energy related information from the Network entities (i.e. RAN nodes, 5GC NFs) can be obtained in order to support network energy related information exposure.

# 2. Discussion

## 2.1	Carbon emission related information

### 2.1.1	Carbon emission

In ITU-T L-series [1] recommendation the Carbon emission for an organization is associate to the different scope, in ITU-T L.1450 they are defined as follow:

-	Scope 1 emission: Greenhouse gas (GHG) emission from sources owned or controlled by an organization.

-	Scope 2 emission: On site greenhouse gas (GHG) emission from the generation of electricity, heat or steam that has been purchased by the reporting organization.

-	Scope 3 emission: Any other indirect greenhouse gas (GHG) emissions from sources that are located along the reporting organization's value chain.

### 2.1.2	Carbon footprint

The carbon emission and the carbon footprint do not represent the same concept and scope even if they are clearly related:

-	Carbon emission is the emission of CO2 from the production of a product or for example from production of the electric power or from burning gasoline in case of transport.

-	The carbon footprint includes the emission from the whole life cycle, i.e. including the production phase from the mining, transport of minerals, all transformation and production phase, the usage phase where in case of ICT related mainly to the emission due to the electricity use for the usage and the end of life cycle for managing the waste process.

It should be noted that the carbon emission is a contribution to the Carbon footprint due to the usage operation of the ICT equipment.

### 2.1.3	Carbon intensity

The 3rd term is Carbon Intensity which represents the Carbon emission for unity of performance, for example in case of production of electric energy is Greenhouse gas emission intensity (g CO2e/kWh) calculated as the ratio of CO2e emissions from public electricity production (as a share of CO2 equivalent emissions from public electricity and heat production related to electricity production), and electricity production per unit of kWh. In contrast ITU-T L.1333 [2] network carbon intensity as the radio between the GHG emission due to electricity consumption of all equipment of all sites of a network during normal operation and the total amount of volume traffic handled in the period.

### 2.1.4	Carbon dioxide and carbon dioxide equivalent

A second set of terms are Carbon Dioxide (CO2) and carbon dioxide equivalent (CO2e). The carbon dioxide equivalent includes greenhouse gases besides CO2, i.e. methane, ozone, water vapor, nitrous oxides and fluorinated gases such as hydrofluorocarbons and chlorofluorocarbons, for example, carbon dioxide scores 1 (as a benchmark), methane would have a score of 25, nitrous oxide would have a score of 298 and some gases would score exponentially higher. For such reason, the CO2e which consider the contributions of other gases is used as a more accurate measurement.

## 2.2	Evaluation of carbon emission of ICT network

The Carbon emission of ICT network due to electricity generation (ECO2) is evaluated from the multiplication of EC per the Carbon intensity of electricity also called Emission Factor EF. According to ITU-T L.1333 [2], definition of the total emission can be evaluated as Energy consumption multiplied Emission Factor (also called Carbon Intensity) representing the mass of carbon emitted per kWh of electricity (kg CO2e/kWh), i.e.:

ECO2=EC*EF

In order to consider different contributors of energy sources, the carbon emission may be further calculated as the sum of contributors, if available, as

ECO2=EC1*EF1+EC2*EF2+ECj*EFj+ECn*EFn

Where:

EC1 is the consumption of electricity produced locally

EF1 is the Emission Factor for the electricity sourced used locally

EC2 is the consumption of local renewable energy

EF2 is the Emission Factor for the local renewable energy

ECj is the consumption of main power renewable energy for source j

EFj is the Emission Factor main power renewable energy for source j

ECn is the consumption of non-renewable energy

EFn is the Emission Factor of non-renewable energy

The Emission Factor (or carbon intensity) is a parameter that can be obtained as a value considering the overall electricity production in a given country considering the mix of energy sources that has been used over a period of 1 year. This information may be available from different sources, for example within the EU community the Carbon intensity (EF) [3] is shown in figure 2.2-1. The number are based on the mix of energy, therefore is the EF derived from that number, the ratio of renewable and non-renewable energy shall not be used since it is already implicitly included.

Figure 2.2-1: GHG emission intensity of electricity generation in EU country (source [3])

If the evaluation is based considering the different energy sources, since they are known more in detail, the EF factor per each source needs to be considered. The EF factor is not worldwide unique since it take into account the kind of electricity power plan which is installed in each country, an example is reported in table 2.2-1 from the French RTE (https://www.rte-france.com/en/eco2mix/co2-emissions)

Table 2.2-1 The contribution to CO2 emissions (source France RTE [x]).

| The contribution to C02 emissions: | source |
| --- | --- |
| 0.986 t CO2 eq/MWh | coal-fired plants |
| 0.777 t CO2 eq /MWh | oil-fired plants |
| 0.486 t CO2 eq /MWh | gas-turbine plants |
| 0.352 t CO2 eq /MWh | co-generation & combined-cycle plant |
| 0.583 t CO2 eq /MWh | other gas-fired plants |

The Contribution of CO2 emission needs also be carefully considered since it may be limited to the production or includes also the life cycle contribution of the power plan, therefore the Carbon emission is also for the renewable energy, an example is reported in figure 2.2-2 from IPCC. From SA2 perspective, the point is that a figure exists as average per energy source, and that actual value cam be different.

Figure 2.2-2 Average life-cycle CO2 equivalent emissions (source [4])

# 2. Conclusion and proposal

Considering the above, it can be concluded that:

- The carbon emission and carbon footprint are different concepts and we need to pay attention to which is used.

- The carbon emission of mobile network considers only the emission due to the energy consumption for the usage of the network. The Carbon emission contribution due to the electricity consumption can be related to the Carbon footprint of the electricity, which includes the contribution of the production of the electricity (i.e. only the contribution of the fuel used to produce the electricity) and the life cycle of the power plant.

- The carbon footprint of ICT as defined by ITU T series considers the whole life cycle of the network, e.g. the contribution of building the site (material, transport of material, cement, etc.) the equipment (production of equipment including the contribution of mine, material transportation and transformation, transport, production, etc.), the end of life cycle.

- The several methodologies are present in literature, normative (e.g. in ISO, ITU) and in regulation to evaluate the carbon emission and the carbon footprint of ICT, for example in ITU the carbon footprint of data center, site, transport network, mobile network, etc.

- The methodology may be associated to EC or to other elements which are outside the SA2 perimeters and all of them are in generally “evaluation” and not real-time data. Therefore, whether this information would be available in 3GPP requires the study by SA5.

In this release of the study, only focus on the Carbon emission aspect directly related to the Energy Consumption contribution and not consider the Carbon Footprint evaluation since it is related to the overal life cycle of equipment, site, etc.

# 3. References

[1]	GreenHouse Gas Protocol Initiative (2004). The greenhouse gas protocol – A corporate accounting and reporting standard, revised version. Geneva: World Business Council for Sustainable Development; Washington, DC: World Resources Institute. 116 pp. Available (viewed 2018-11-14) at: http://pdf.wri.org/ghg_protocol_2004.pdf

[2]	ITU L.1333 "Carbon data intensity for network energy performance monitoring"

[3]	National emissions reported to the UNFCCC and to the EU Greenhouse Gas Monitoring Mechanism, European Environment Agency (EEA) https://www.eea.europa.eu/en/analysis/indicators/greenhouse-gas-emission-intensity-of-1

[4]	IPCC Glossary https://www.ipcc.ch/report/ar5/wg1/