SA WG2 Meeting #160	S2-2313104

November 13 - 17, Chicago, US

Source:	Apple, Futurewei, Meta USA, Google

Title:	KI#2: Corrections to Key Issue wording and Encrypted Media Definition

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2 /Rel-19

Abstract of the contribution: This paper proposes corrections to definition of encrypted media and KI#2 in 23.700-70 v0.1.0

# 1	Discussion

In SA2#159, SA2 agreed on the following definitions for encrypted media traffic.

Fully Encrypted Media Flow: A media flow where both the media header and media payload are encrypted from end-to-end. Fully encrypted headers and payload are not visible in the network. Examples include RTP cryptex (RFC 9335 [7]), RTP over QUIC (RoQ) (draft-ietf-avtcore-rtp-over-quic [8]) and Media over QUIC (MoQ) (draft-ietf-moq-transport [9]). Partially Encrypted Media Flow: A media flow where some media headers (e.g., base header) are not encrypted. Other media headers (e.g., extension header) and media payload are encrypted from end-to-end. The payload and headers that are encrypted from end-to-end are not visible in the network. Examples include SRTP (RFC 3711 [5]) with partially encrypted header extensions (RFC 6904 [6], draft-ietf-avtext-framemarking [10]).

For RTP cryptex (RFC9335), certain header fields are left outside of encryption. Specifically, ‘M’ bit and ‘timestamp’ information are outside the encrypted header extensions. These are useful information for PDU Set identification as also described in TS 26.522 Annexe A.1.  It is proposed to list RTP cryptex as an example of partially encrypted media flow.

Additionally, this pCR proposes an editorial correction in KI#2 to add “traffic” to the key issue text.

# 2	Proposal

It is proposed to adopt the following changes into TR 23.700-70 v0.1.0.

## * * * * 1st Change (revision marked)

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Fully Encrypted Media Flow: A media flow where both the media header and media payload are encrypted from end-to-end. Fully encrypted headers and payload are not visible in the network. Examples include RTP over QUIC (RoQ) (draft-ietf-avtcore-rtp-over-quic [8]) and Media over QUIC (MoQ) (draft-ietf-moq-transport [9]).

Partially Encrypted Media Flow: A media flow where some media headers (e.g., base header) are not encrypted. Other media headers (e.g., extension header) and media payload are encrypted from end-to-end. The payload and headers that are encrypted from end-to-end are not visible in the network. Examples include SRTP (RFC 3711 [5]) with partially encrypted header extensions (RFC 6904 [6], RTP cryptex (RFC 9335 [7]), draft-ietf-avtext-framemarking [10]).

* * * * 2nd Change (revision marked) * * * *

## 5.2	Key Issue #2: Support PDU Set information identification for end-to-end encrypted XRM traffic

### 5.2.1	Description

The usage of end-to-end encryption is broadly deployed in current networks to provide security and the same is expected for XRM applications.

This key issue proposes to study the enhancement of PDU Set information Identification for encrypted XRM traffic in 5G networks.

The solutions should consider the following aspect:

-	If and how the 5GS performs PDU Set information Identification in an end-to-end encryption scenario.

NOTE 1:	Solutions that rely on breaking end-to-end encryption are out of the scope of this key issue.

NOTE 2:	The work on this key issue may need coordination with SA WG4 and SA WG3.

* * * * End of Changes * * * *