SA WG2 Meeting #160	S2-2313125

# November 13th – 17th, 2023, Chicago, USA	(revision of S2-)

Source: 	MediaTek Inc.

Title: 	pCR 23.700-29 Scope and Definition wrt constellations

Document for: 	Agreement

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This pCR proposes to update the Scope of the TR to cover all constellations covered since Rel-17 i.e. LEO, MEO, GEO, OTHERSAT.

# 1	Discussion

The Rel-17 and Rel-18 architecture support for Satellite access (for IoT and NR) is defined for any constellation type – namely referred to as LEO, MEO, GEO and OTHERSAT. The work of SA2 has considered necessary characteristics and associated functionality accounting for:

-	Moving satellites i.e. ground velocity > 0 m/s

-	Non-moving satellites i.e. ground velocity = 0 m/s

-	Large delays incurred by high orbits (ca. 36,000km)

-	Discontinuous coverage

- 	Fixed or steerable beam spots

Rel-19 work builds on existing specifications and therefore no restriction should be incurred wrt what is already supported in specifications.

Recent discussions on non-GEO GSO i.e. moving satellites at high orbits covering the same geographical location every 24 hours (sidereal) seem to go in the direction of removing GSO support – we see no reason for this given the above. It is therefore proposed to clarify TR23.700-29 accordingly.

The definition of Serving satellite is not correct for it refers to moving satellites with NGSO only. However, the upper time boundary that any satellite could serve a UE (whether the satellite is moving or not) is function of the satellite ground velocity, whether it uses fixed or steerable beams, and the position and mobility of the UE. It is proposed to clarify this.

Last, we acknowledge that the terminology at least in TS38.300 is off, however we do not expect this has any effect in SA2. We do not either see it is necessary for SA2 to contact RAN2 on this matter – but it could be resolved via company inputs into RAN2 directly. We see it important that 3GPP terminology be consistent with satellite industry terminology to avoid any confusion.

**** FIRST CHANGE ****

# 1	Scope

The scope of this Technical Report is to study the following aspects of 5GS/EPS integrating of satellite component in Release 19, including:

-	Regenerative payload generic architecture:

-	Impact to 5GS and EPS to support gNB/eNB onboard the satellite.

-	Store and Forward Satellite operation for delay tolerant services:

-	The control plane and user plane enhancements if applicable, including the minimum necessary set of Core Network elements to be embedded in the satellite, to support S&F Satellite operation, both for IoT NTN (EPS) and NR NTN (5GS);

-	The parameters needed to characterize and support S&F Satellite operation from a delay tolerant services perspective if applicable, both for IoT NTN (EPS) and NR NTN (5GS).

NOTE:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

-	UE-Satellite-UE communication enhancements for 5GS, supporting NR NTN NGSO constellation with and without ISL, with feeder link always available (at least for session establishment):

-	Following SA1 requirements, study how at least IMS enablers including mission critical can be supported;

-	Minimum necessary set of 5GS network functions onboard the satellite(s) to support such UE-Satellite-UE communication.

Unless explicitly stated otherwise, all constellation types LEO, MEO, GEO and OTHERSAT supported since Rel-17 are supported.

**** NEXT CHANGE ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[7]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Serving satellite: a satellite providing satellite access to a UE (e.g. providing the serving cell(s)).. A Serving satellite  may serve a UE for a limited period of time at a time, depending on the nature of its orbit (i.e. with or without ground velocity), the nature of its beams (i.e. fixed or steerable), and the position and mobility of the UE.

S&F Satellite operation: operation mode providing communication service (in storing and forwarding information) to a UE in periods of time and/or geographical areas in which the serving satellite is not simultaneously connected to the ground network via feeder link or ISL. For the case of UL, "store" refers to on-board storage of UL information from UE and "forward" refers to forwarding of stored UL information to the ground network. For the case of DL, "store" refers to on-board storage of DL information from the ground network and "forward" refers to forwarding of stored DL information to the UE.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NOTE:	These definitions are different from those in TS 22.261 [6].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ISL	Inter-Satellite Link

GEO	Geostationary Orbit

GSO	Geosynchronous Orbit

LEO	Low Earth Orbit

MEO	Medium Earth Orbit

NGSO	Non-Geosynchronous Orbit

NTN	Non-Terrestrial Network

S&F	Store and Forward

**** END OF CHANGES ****