SA WG2 Meeting #160 	S2-2313146

Chicago, Illinois, USA, 13-17 November 2023	(revision of)

Source:		Nokia, Nokia Shanghai Bell

Title:		New WID: NSSAA support in EPS

Document for:	Information

Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items  
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: NSSAA support in EPS

Acronym: TEI19_NSSAAEPS

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

# 3	Justification

It has become clear at this stage of5GS deployments that the situation where a UE is camping in EPS cannot be exclude. This is exactly the rationale that has led in rel-18 to allow a UE camping in EPS to perform Secondary authentication/authorization by a DN-AAA server (see  Annex H of TS 23.502) . For network slicing, despite EPS interworking is supported for network slicing, this makes the rollout of certain network slicing features difficult or impossible until a PLMN has fully upgraded to 5GS and refarmed the EPS bands, as the semantics of these features is broken when the UE is in EPS and needs to continue or initiate service in PDN connections associated to network slices affected by these features and so this creates hurdles to adoption of the same features also in 5GS. One of these features in .g. NSSAA. It is proposed to enhance the EPS so that it is possible to perform NSSAA in EPS.

# 4	Objective

The objective is to enhance the system so that a UE camping in EPS to perform NSSAA. The same technical approach as followed for Secondary authentication/authorization by a DN-AAA server in EPS will be followed.

Note: This work will require 1 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | ADD a clause similar to 5.17.2.5 for NSSAA | SA#106 (September 2024)? | Target completion to be determined at SA2#161 or whenever approval takes place |
| 23.502 | ADD an annex similar to Annex H | SA#106 (September 2024)? | Target completion to be determined at SA2#161 or whenever approval takes place |

# 6	Work item Rapporteur(s)

Nokia (alessio.casat@nokia.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Impacts to NGAP is to be handled by RAN3.

Impact to subscription data to be handled by CT4

Impact to NSA to be handled by CT1

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell