SA WG2 Meeting #160 	S2-2313147

Chicago, Illinois, USA, 13-17 November 2023	(revision of)

Source:		Nokia, Nokia Shanghai Bell

Title:		New WID: Per UE S-NSSAI priority awareness in the 5GS

Document for:	Information

Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items  
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Per UE S-NSSAI priority awareness in the 5GS

Acronym: TEI19

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

# 3	Justification

The 5GS currently does not allow end to end signalling of what network slices are more relevant to a UE. Today only per SUPI range configuration is possible but this is not flexible (e.g. if an updated/upgraded subscription package changes the relative importance of slices of a subscriber then the SUPI needs to be changed! i.e. a new USIM needs to be delivered to the customer). This is very cumbersome to operate, especially in roaming scenarios.

Knowing the relative importance of network slices is critical for e.g. the setting of the Target NSSAI, for the handling of NSSRG in the UE, for improvements of the steering of the UE to the relevant band in the NG-RAN when per UE RRM needs to be performed taking into account the Allowed and Partially allowed network slices. It can also be useful information for the setting of the RFSP and other information like the NSAG priority.

For instance: The NG-RAN receives the Allowed NSSAI, but it cannot determine which of the S-NSSAIs in the Allowed NSSAI is more important for the UE. Having this information is beneficial e.g. to allow effective per UE RRM policies and also to take decisions based on the relative importance of a network slice when e.g. the candidate cells for handover may support different sets of S-NSSAIs, or e.g. when at time of congestion the "essential" set of DRBs for the more important slices need to be preserved.

For instance, a UE allowed NSSAI may include S-NSSAI 1 and S-NSSAI 2 (both with DRBs that need to be handed over). For the (R)AN it is difficult to determine which target cell to use if the target support only one of them but not both, all things being equal.

In figure 1 the candidate target cells are TA3 and TA2 supporting S-NSSAI 1 and S-NSSAI2 respectively... which cell to choose?

In short, if the (R)AN node knows that S-NSSAI 1 is more important for the UE, it can have a tool to provide a more predictable and more appropriate service experience. This relative importance among S-NSSAIs in general cannot be the same for all the UEs. Hence the need to receive information from the AMF.

In another use case: If the (R)AN is congested, one possible strategy for handling load could also be to shed the least important S-NSSAIs DRBs for the UEs.

Since the importance of a slice for the UE can derive from the subscription type of the UE (linked to the particular operator commercial offering for the set of network slices in the UE subscription), it is proposed that the priority of the S-NSSAI is defined as part of the subscription data. It should be understood that this importance value is for the UE and not an absolute priority value across UEs.

# 4	Objective

The objective is to include in the information the AMF sends to the (R)AN when it sends the Allowed NSSAI or Partially Allowed NSSAI to the NG-RAN any per UE S-NSSAI priority for the S-NSSAIs. The S-NSSAI priority is derived from a Subscribed S-NSSAI priority the AMF receives, if any. The AMF also sends the same information to the UE.

The AMF receives this information from the UDM for the subscribed S-NSSAIs and in roaming case a suitable mapping is performed to applicable VPLMN  S-NSSAIs mapping to the subscribed S-NSSAIs.

The S-NSSAI priority can also be used by the PCF and AMF derive the RFSP for a set of S-NSSAIs e.g. in Allowed, Partially allowed NSSAIs, Rejected S-NSSAIs

Note: This work will require 1 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | ADD the handling of per UE S-NSSAI priority to (R)AN and UE based on subscription data | SA#106 (September 2024)? | Target completion to be determined at SA2#161 or whenever approval takes place |
| 23.502 | ADD the handling of per UE S-NSSAI priority to (R)AN and UE based on subscription data | SA#106 (September 2024)? | Target completion to be determined at SA2#161 or whenever approval takes place |

# 6	Work item Rapporteur(s)

Nokia (alessio.casat@nokia.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Impacts to NGAP is to be handled by RAN3.

Impact to subscription data to be handled by CT4

Impact to NSA to be handled by CT1

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell