3GPP TSG-SA2 Meeting #160	S2-2313162

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1021 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revised the procedure for requesting and collecting analytics from MDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo (Beijing) Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SA2#159 meeting an LS from SA5 (S2-2310134) was received providing more infroamtion related to the discovery process that enables the NWDAF to select the most suitable MDAF in order to obtain analytics. The LS suggests that the discovery can be perfromed in two steps. In the initial step: “The OAM MnS discovery mechanism allows consumers to discover management capabilities including endpoint address at the MnS discovery service producer for use by MnS consumers to interact with the corresponding MnS producers.” In a follow up step “Once discovered, both MnS consumer and producer can interact and obtain the required information.” In our understanding the second step is optional. This contribution aims to capture this process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution clarifies the case of selecting the most suitable MDAF when there are many potential options after the initial discovery process. It introduces changes in the figure and text by adding steps 4-5 to illustrate the case where the intial discovery returns several MDAF endpoint addresses. In particular, it introduces steps 4-5, to enable NWDAF to obtain further required information from potential MDAFs directly in order to allow the NWDAF to select the most suitable MDAF. It also introduces a step 6 that seperates the decision of selecting the most suitable MDAF from the step that issues the MDARequest towards the selected MDAF. Finally it deletes the editor note and NOTE 2 that were added temporarily until the SA5 LS was received. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NWDAF would not be select the most suitable MDAF when the initial discovery step returns seveal MDAF endpoint addresses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 6.2.14	Analytics Collection from MDAF

#### 6.2.14.1	General

The MDA functional overview and service framework in Figure 5.1-1 as defined in TS 28.104 [45] is used by NWDAF to trigger the MDA MnS to request analytics from the MDA Management Function (MDAF).

Before NWDAF requests analytics from the MDA Management Function, the NWDAF firstly discovers the MDA Management Function via the MnS discovery service producer as defined in clause 5 of TS 28.537 [46].

#### 6.2.14.2	Procedure for analytics collection from MDAF

Figure 6.2.14.2-1: Procedure for analytics collection from MDAF

Precondition: Initially MDAF(s) or MDA MnS producers register their capabilities, i.e., MnS information or MnS profile as described in clause 5 of TS 28.537 [46] to a MnS discovery service producer. The MnS discovery service producer may contain partial information related to the capabilities of MDA MnS producer.

1.	An analytics consumer issues a request or subscription of network analytics towards the selected NWDAF as described in clause 6.1.

2.	NWDAF discovers the MDA Management Function from the MnS discovery service producer by sending a MnS producer discovery service operation.

NOTE 1:	The service operation may include parameters for MDA MnS discovery, e.g., requested MDA Type, Area of Interest, Network Slice information, etc. The detailed parameters are up to SA WG5.

3.	The MnS discovery service producer provides the relevant MnS information of the MDA Management Function.

NOTE 1:	MnS information refer to the data describing a MnS producer and their capabilities, which is used by the consumer to discover the producer of specific Management Services and to derive the addresses of the Management Services as defined in TS 28.537 [46] and TS 28.662 [41].

- Optionally, if more than one MDAF information is received, the NWDAF uses the address received and issues a request to specific MDAF(s) to discover the further capabilities related to, e.g., computing, latency, etc., which rely on the current workload.

- Each MDAF provides in a response the request capabilities.

- If the MnS information of more than one MDAF is received, NWDAF selects the most suitable MDAF and gets the address of the selected MDAF from the MnS information.

7.	Then NWDAF sends analytics request to the MDAF by triggering a MDARequest service operation as defined in clause 9.3.2 of TS 28.104 [45]. The service operation may include the following parameters:

-	requestedMDAOutputs: MDAOutputPerMDAType(type of analytics) and MDAOutputIEFilters (which assist in indicating explicitly the analytics output requested from the set of MDA outputs and the conditions, considering i) threshold cross, for reporting numeric outputs and ii) timeout, when an output is no longer needed).

-	reportingMethod: indicates the selected reporting method for analytics output. It can only be notification-based reporting since file-based, and streaming-based reporting are not supported in NWDAF.

-	reportingTarget: indicates the reporting target of the MDA outputs.

-	analyticsScope: Area of Interest, Network Slice information, etc.

-	startTime: indicates the start time of the analytics requested by the MnS consumer.

-	stopTime: indicates the stop time of the analytics requested by the MnS consumer.

NOTE 2:	Definitions and details of the parameters can be found in TS 28.104 [45].

8.	The MDA Management Function provides the analytics to the NWDAF by triggering an MDAReporting service operation.

8.	NWDAF provides the analytics response back to the analytics consumer after processing the analytics provided by the MDA Management Function together with other data receives from NF sources according to the Analytics ID defined in clause 6.

* * * * End of Change * * * *