3GPP TSG-SA WG2#160			S2-2313164

13 - 17 November, 2023, Chicago, USA

Source:	Samsung

Title:	New KI on supporting standalone IMS data channel

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract: This paper proposes to add the key issue of how to support standalone IMS data channel without accompanying audio/video/messaging media in an IMS session.

1. Introduction

One of the FS_NG_RTC_Ph2 study objectives is to support the following aspect as approved in SP-231196:

-	WT-4: Enhancements to IMS data channel related services and operational aspects.

-	WT-4.1: how to support standalone IMS data channel without accompanying audio/video/messaging media in an IMS session.

In order to support standalone IMS DC service, many constraints must be resolved.

For example, TS 26.114 has the following restrictions:

A data channel SDP media description shall not be placed before the first SDP speech media description.

Therefore, it is necessary to study how IMS procedures need to be enhanced to support standalone IMS data channel service.

2. Proposal

It is proposed to add the following key issue into TS 23.700-77.

# * * * * First change * * * *

5.X	Key Issue #X: Supporting standalone IMS data channel

5.X.1	Description

This key issue studies how IMS DC architecture, procedures need to be enhanced to support standalone IMS data channel service without accompanying audio/video/messaging media in an IMS session, this key issue will study:

-	Study how IMS procedures need to be enhanced to support standalone IMS data channel service without accompanying audio/video/messaging media.

-  Study how to provide existing audio/video/messaging media in standalone IMS data channel service.

Note: Alignment with SA4 is required.

# * * * * End of changes * * * *