3GPP TSG-WG SA2 Meeting #160 	S2-2313176

Chicago, IL, USA, 13-17 November 2023

# Title:	[Draft] Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

# Response to:	LS on 5G ProSe UE-to-UE relay discovery with security aspects (C1-237897/S2-2311951)

# Release:	Release 18

# Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:	SA3

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR0406 to TS 23.304

1. Overall Description:

SA2 would like to thank CT1 for the LS on 5G ProSe UE-to-UE relay discovery with security aspects (C1-237897/S2-2311951).

Regarding the question from CT1:

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

SA2 would like to provide the answer as follows:

In case the User Info IDs of the 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set of the UE-to-UE Relay Discovery Response message, the 5G ProSe UE-to-UE Relay would not be able to associate the message to a previously received 5G ProSe UE-to-UE Relay Discovery Solicitation message. In order to forward the message, the 5G ProSe UE-to-UE Relay should use the “Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and receiving Relay Discovery Solicitation messages” as specified in TS 23.304 clause 5.1.5.1.

SA2 agreed to clarify this aspect with the attached CR0406.

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above into account when developing the stage 3 specifications.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160 Adhoc-e	22-29 January 2024	e-meeting

TSG-SA2 Meeting #161		26 February-3 March 2024	Athens, Greece