3GPP TSG-WG SA2 Meeting #160 	S2-2313183

Chicago, IL, USA, 13-17 November 2023

# Title:	[Draft] Reply LS on L2ID and User Info for L2 based U2U

# Response to:	LS on L2ID and User Info for L2 based U2U (R2-2311566 / S2-2311980)

# Release:	Release 18

# Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank RAN2 for the LS on L2ID and User Info for L2 based U2U (R2-2311566 / S2-2311980).

SA2 has discussed the two options indicated by RAN2:

- the RRCReconfigurationSidelink message carries both the L2 ID and the Local ID of the peer Remote UE and assumes that the association between the User Info ID and the L2 ID of the peer Remote UE is done at the Remote UE ProSe layer.; and

- the RRCReconfigurationSidelink message carries both the User Info ID and the Local ID of the peer Remote UE, and the association between the User Info ID and the Local ID is done at the Remote UE ProSe layer.

SA2 would like to provide the following feedback:

- Option A) above is not supported by the existing procedures of TS 23.304 for UE-to-UE Relay operation, defined in clause 6.7.2 and 6.4.3.7.1, because the two 5G ProSe End UEs do not have knowledge of each other’s Layer-2 ID.

- Option B) can be supported with existing procedures in TS 23.304, and no change is required.

In additional, SA2 would like to inform RAN2 that in the design assumption of TS 23.304, the Layer-2 ID was not considered a stable identifier for the UE at ProSe layer. Additionally, the Layer-2 ID may change from time to time. As explained in clause 5.8.2.4:

The UE maintains a mapping between the Application Layer IDs and the source Layer-2 IDs used for the PC5 unicast links, as the ProSe application layer does not use the Layer-2 IDs. This allows the change of source Layer-2 ID without interrupting the ProSe applications.

Therefore, from SA2 perspective, option B) would be a better choice for Rel-18, as it has less impacts to both SA2 and RAN2 work.

SA2 would like to ask RAN2 to update its working assumptions accordingly and provide feedback to SA2 if technical issues is identified for option B).

2. Actions:

To RAN2:

ACTION: 	SA2 would like to ask RAN2 to take the above into account and update the working assumptions accordingly.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160 Adhoc-e	22-29 January 2024	e-meeting

TSG-SA2 Meeting #161		26 February-3 March 2024	Athens, Greece